| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0117-v1 |
| --- | --- |

| Title: | AhG12: ECM coding performance for HDR/WCG content and suggested common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121 | Tel: Email: | tlu@dolby.com nanh@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Dolby Laboratories Inc, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents HDR/WCG coding results for ECM3.1 with two encoder settings. The first setting follows the VTM HDR/WCG common test conditions. The second setting adds encoder configuration optimizations recently added for SDR coding with ECM and the CTU size of HLG sequences is 256. The results of ECM3.1 HDR coding compared with VTM11.0 with MCTF are as follows:

First setting (VTM HDR/WCG CTC):

For PQ content, the BD rate savings for wPSNRY/U/V/DE100/L100 are:

AI { -6.19% -18.43% -22.07% -12.31% -6.65%	}

RA { -11.19% -25.44% -28.46%  -19.21% -11.73% }

For HLG content, the BD rate savings for PSNRY/U/V are:

AI { -5.88% -12.24% -16.13% }

RA {-9.84% -18.70% -21.28% }

Second setting (VTM HDR/WCG CTC + ECM encoder optimization + CTU 256 for HLG):

For PQ content, the BD rate savings for wPSNRY/U/V/DE100/L100 are:

AI { -6.19% -18.43% -22.07% -12.31% -6.65%	}

RA { -11.59% -27.82% -31.98% -22.98% -12.41% }

For HLG content, the BD rate savings for PSNRY/U/V are:

AI { -6.02% -11.86% -16.05% }

RA { -10.79% -20.84% -24.42% }

It is recommended to establish ECM HDR/WCG common test conditions corresponding to the second setting.

# Introduction

ECM coding performance for SDR video has been studied in recent JVET meetings, but the study on ECM coding performance for HDR/WCG video has not been carried out yet. This contribution presents the latest ECM coding performance for HDR/WCG content and proposes to establish HDR/WCG common test condition for ECM.

The earlier ECM version (e.g. ECM1.x and ECM2.x) did not produce correct HDR/WCG coding results because of several software bugs related to large CTU size and weighted distortion. During the ECM3 software integration phase, these bugs were identified and fixed [1][2]. The released ECM3.1 software[3] is therefore used to encode HDR/WCG content for a meaningful study.

# HDR/WCG testing condition for ECM

ECM provides many new features over VTM. The new features include both new coding tools and encoder optimizations. The HDR coding experiments inherit the coding tool settings being used for SDR content. On top of that, two settings are used to study the performance of ECM HDR/WCG coding.

The first setting reuses the VVC HDR/WCG common test condition defined in [4]. The second setting additionally studies the new encoder optimizations being used in ECM SDR coding and increasing the CTU size to 256 for HLG sequences. The new encoder optimizations are as follows. The large CTU size (256) for video resolution at or beyond 4K is applicable for HDR-H2 (HLG) content. Also, the resolution adaptive partitioning optimization introduced in [5] for RA coding is applicable for both HDR-H1 (PQ) and HDR-H2 (HLG) content. The partitioning parameters used for class B (1920x1080) are applied for class H1. The partitioning parameters used for class A (3840x2160) are applied for class H2, which thus aligns the partitioning settings based on resolution regardless of SDR or HDR. The exact encoder parameter changes are listed in Table 1:

Table 1 New encoder settings

|  | HDR-H1 (PQ) | HDR-H2 (HLG) |
| --- | --- | --- |
| AI | no change | CTUSize: 256 |
| RA | MaxMTTHierarchyDepth: 4 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 256 MaxMTTHierarchyDepth: 4 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 |

# Experiments and results

The anchor used for the ECM HDR/WCG coding study is VTM11.0 with MCTF (aligned with SDR coding efficiency study).

The BD rate savings of the first setting over anchor for AI and RA test conditions are listed in Table 2.

Table 2 Setting 1 test results

|  |  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 MCTF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | wPSNR |  |  |  |  | PSNR |  |  |  |  |  |  |  |  |
|  |  | DE100 |  | PSNR-L100 |  | Y | U |  | V |  | Y | U |  | V |  | EncT |  | DecT |  |
| Class H1 |  | -12.31% |  | -6.65% |  | -6.19% | -18.43% |  | -22.07% |  | -6.08% | -18.18% |  | -21.57% |  | 325% |  | 249% |  |
| Class H2 |  |  |  |  |  |  |  |  |  |  | -5.88% | -12.24% |  | -16.13% |  | 312% |  | 254% |  |
| Overall |  | -12.31% |  | -6.65% |  | -6.19% | -18.43% |  | -22.07% |  | -6.01% | -16.02% |  | -19.60% |  | 320% |  | 250% |  |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11.0 MCTF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | wPSNR |  |  |  |  | PSNR |  |  |  |  |  |  |  |  |  |
|  | DE100 |  | PSNR-L100 |  | Y |  |  | U | V | Y |  |  | U |  | V |  | EncT |  | DecT |
| Class H1 | -19.21% |  | -11.73% |  | -11.19% |  |  | -25.44% | -28.46% | -11.11% |  |  | -25.07% |  | -28.32% |  | 332% |  | 427% |
| Class H2 |  |  |  |  |  |  |  |  |  | -9.84% |  |  | -18.70% |  | -21.28% |  | 344% |  | 458% |
| Overall | -19.21% |  | -11.73% |  | -11.19% |  |  | -25.44% | -28.46% | -10.65% |  |  | -22.76% |  | -25.76% |  | 336% |  | 438% |

The BD rate savings of the second setting over anchor for AI and RA test conditions are listed in Table 3. (The AI results for H1 is identical in setting1 and setting 2 because no new optimization is applied). It is observed that the ECM new encoder optimization settings and larger CTU size can further improve the coding efficiency for HDR/WCG coding as well.

Table 3 Setting 2 test results

|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 MCTF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | wPSNR |  |  |  |  |  | PSNR |  |  |  |  |  |  |
|  | DE100 |  | PSNR-L100 |  | Y | U |  |  | V |  | Y | U |  | V |  | EncT | DecT |
| Class H1 | -12.31% |  | -6.65% |  | -6.19% | -18.43% |  |  | -22.07% |  | -6.08% | -18.18% |  | -21.57% |  | 325% | 249% |
| Class H2 |  |  |  |  |  |  |  |  |  |  | -6.02% | -11.86% |  | -16.05% |  | 322% | 261% |
| Overall | -12.31% |  | -6.65% |  | -6.19% | -18.43% |  |  | -22.07% |  | -6.06% | -15.88% |  | -19.57% |  | 324% | 253% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11.0 MCTF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | wPSNR |  |  |  |  |  | PSNR |  |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 |  | Y |  |  | U | V |  | Y |  |  | U |  | V | EncT | DecT |
| Class H1 | -22.98% | -12.41% |  | -11.59% |  |  | -27.82% | -31.98% |  | -11.49% |  |  | -27.21% |  | -31.36% | 404% | 402% |
| Class H2 |  |  |  |  |  |  |  |  |  | -10.79% |  |  | -20.84% |  | -24.42% | 400% | 469% |
| Overall | -22.98% | -12.41% |  | -11.59% |  |  | -27.82% | -31.98% |  | -11.24% |  |  | -24.89% |  | -28.84% | 402% | 425% |

A revised version of JVET-V2011 [4] on “JVET common test conditions and evaluation procedures for HDR/WCG video” is attached with this contribution to serve as the draft for recommended ECM HDR/WCG common test condition.

# Conclusions

This contribution presented ECM3.1 coding performance for HDR/WCG content with two encoder settings. It is recommended to establish common test condition for ECM HDR/WCG coding corresponding to the second setting.

# Acknowledgement

We would like to thank Interdigital for crosschecking this contribution.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/46

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/18

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-V2011, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Teleconference, May 2021.

- M. Coban, V. Seregin, M. Karczewicz (Qualcomm), K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang (Bytedance), F. Le Léannec, K. Naser, T. Dumas, A. Robert, F. Galpin, E. François (InterDigital), “EE2: Encoder partitioning optimization for ECM and crosscheck of EE2-1.1”, JVET-X0144, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Teleconference, October 2021.

# Patent rights declaration(s)

Dolby Laboratories Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).