| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0134-v1 |
| --- | --- |

| Title: | EE2-3.4, EE2-3.5, EE2-3.6: Experimental results of the MV candidates reordering in candidate types based on template matching costs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Han Huang, Vadim Seregin, Chun-Chi Chen, Marta Karczewicz (Qualcomm Inc.) Ru-Ling Liao, Jie Chen, Yan Ye, Xinwei Li (Alibaba group) | Email: | {yjchang, hanhuang, vseregin} @qti.qualcomm.com {ruling.lrl, yan.ye}@alibaba-inc.com |
|  | Lei Zhao, Kai Zhang, Na Zhang, Li Zhang (Bytedance Inc.) |  | {zhaolei.0324, zhangkai.video, zhangna.cynthia, lizhang.idm} @bytedance.com |
|  | Guillaume Laroche, Patrice Onno, Romain Bellessort (Canon) |  | {guillaume.laroche, Patrice.onno}@crf.canon.fr |
| Source: | Qualcomm Inc., Alibaba group, Bytedance Inc., Canon |  |  |

_____________________________

# Abstract

This contribution presents experimental results of EE2-3.4, EE2-3.5, and EE2-3.6 for the MV candidates reordering in candidate types based on template matching costs. Depending on the experiments, the MV candidates reordering may be applied to single TMVP type, single NA-MVP type, and the hybrid candidate types which include NA-MVP, HMVP and PAMVP types. On top of ECM-3.1, the experimental results are reported as follows:

Test 3.4: MV candidate reordering separately applied to TMVP type and NA-MVP type

RA: -0.52 % (Y), -0.52 % (U), -0.5 % (V), 100% (EncT), 10x% (DecT)

LB: -0.24 % (Y), -0.15 % (U), -0.52 % (V), 100% (EncT), 10x% (DecT)

Test 3.5a: MV candidate reordering in the hybrid candidate types, i.e., NA-MVP, HMVP, PAMVP

RA: -0.34% (Y), -0.29% (U), -0.33 % (V), % (EncT), % (DecT)

LB: -0.41 % (Y), -0.59 % (U), -0.51 % (V), % (EncT), % (DecT)

Test 3.5b: Test 3.5a software combined with Test 3.2

RA: -0.38% (Y), -0.37% (U), -0.35% (V), 100% (EncT), 99% (DecT)

LB: -0.47% (Y), -0.43% (U), -0.47% (V), 101% (EncT), 104% (DecT)

Test 3.6a: Test 3.4 software combined with Test 3.3a

RA: -0.54 % (Y), -0.54 % (U), -0.52 % (V), 100% (EncT), 10x% (DecT)

LB: -0.54 % (Y), -0.34 % (U), -0.66 % (V), 100% (EncT), 10x% (DecT)

Test 3.3a + Test 3.4 + Test 3.5a: Test 3.5a software combined with Test 3.3a and TMVP reordering in Test 3.4

RA: -0. % (Y), -0. % (U), -0. % (V), % (EncT), % (DecT)

LB: -0. % (Y), -0. % (U), -0. % (V), % (EncT), % (DecT)

# Introduction

In ECM [1], the regular merge, TM merge, and BM merge candidate lists are constructed by including the following six types of candidates in order:

- Spatial MVP (SMVP) from spatial neighbour CUs

- Temporal MVP (TMVP) from collocated CUs

- Non-adjacent MVP (NA-MVP) from spatially non-adjacent CUs

- History-based MVP (HMVP) from a FIFO table

- Pairwise average MVP (PAMVP)

- Zero MVs.

In 24th JVET meeting, JVET-X0133 proposed to reorder the MV candidates based on the TM cost values in a single candidate type, which is TMVP type or NA-SMVP type. JVET-X0087 proposed to reorder the MV candidates from the hybrid candidate types (including NA-MVP and HMVP) based on the TM cost values. Both proposals were studied in EE2, i.e., Test 3.4 (JVET-X0133) and Test 3.5a (JVET-X0087). In this contribution, the experimental results of Test 3.4 and Test 3.5 are reported, as well as combination test results of Test 3.2, Test 3.3a, Test 3.4, and Test 3.5a.

# Experimental tests

## Test 3.4: MV candidate type-based ARMC

In this test, MV candidate type-based ARMC in JVET-X0133 is investigated. In the method, the merge candidates in one single candidate type, e.g., TMVP or NA-MVP, are reordered based on the ARMC TM cost values. The reordered candidates are then added into the merge candidate list. The TMVP candidate type adds more TMVP candidates with more temporal positions and different inter prediction directions to perform the reordering and the selection. Moreover, NA-MVP candidate type is further extended with more spatially non-adjacent positions.

## Test 3.5a: Template matching based merge candidate list construction (TM-MCLC)

With TM-MCLC in JVET-X0087, NA-MVP, HMVP, and PAMVP candidates are put into the merge candidate list in an ascending order of template matching costs. Moreover, non-adjacent MVPs is further extended with more spatial and non-adjacent temporal positions.

## Combination tests

The combination test results of Test 3.2, Test 3.3a, Test 3.4 and Test 3.5a are also reported in this contribution. The combination tests are as follows:

Test 3.5b (Combination of Test 3.5a and Test 3.2): On top of 3.5a software, the PAMVP is removed from the merge list and derived during the reordering process of merge candidate list (ARMC). In addition, each merge candidate, in the non-reordered subgroup, is replaced by a pairwise between the first candidate and this candidate.

Test 3.6a (Combination of Test 3.4 and Test 3.3a): On top of 3.4 software, it adds the following method from Test 3.3a:

- The reference picture of the TMVP candidate is selected from any one of reference pictures in the reference picture list according to a pre-defined rule. The selected reference picture is the one whose scaling factor is the closest to 1.

Test 3.3a + Test 3.4 + Test 3.5a: On top of 3.5a software, it adds the following methods from Test 3.4 and Test 3.3a:

- Test 3.4: TMVP candidates reordering based on TM cost values.

- Test 3.3a: The reference picture of the TMVP candidate is selected from any one of reference pictures in the reference picture list according to a pre-defined rule. The selected reference picture is the one whose scaling factor is the closest to 1.

# Experimental results

Five experiments are tested on top of ECM-3.1 under the common test conditions [2], and are listed as follows:

Test 3.4: MV candidate type-based ARMC

Test 3.5a: Template matching based merge candidate list construction

Test 3.5b: Test 3.5a + Test 3.2 (on top of Test 3.5a software)

Test 3.6a: Test 3.4 + Test 3.3a (on top of Test 3.4 software)

Test 3.3a + Test 3.4 + Test 3.5a (on top of Test 3.5a software)

Table 1. Performance of Test 3.4

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.67% | -0.71% |  |  |  |  |  |  |  |
| Class A2 | -0.57% | -0.58% | -0.65% |  |  |  |  |  |  |  |
| Class B | -0.48% | -0.47% | -0.37% |  |  | -0.26% | -0.37% | -0.54% |  |  |
| Class C | -0.51% | -0.42% | -0.39% |  |  | -0.33% | -0.32% | -0.09% |  |  |
| Class E |  |  |  |  |  | -0.08% | 0.43% | -1.07% |  |  |
| Overall | -0.52% | -0.52% | -0.50% |  |  | -0.24% | -0.15% | -0.52% |  |  |
| Class D | -0.42% | -0.43% | -0.29% |  |  | -0.01% | 0.71% | -0.67% |  |  |
| Class F | -0.33% | -0.38% | -0.33% |  |  | -0.32% | 0.03% | -0.54% |  |  |

Table 2. Performance of Test 3.5a

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.29% | -0.38% |  |  |  |  |  |  |  |
| Class A2 | -0.38% | -0.32% | -0.40% |  |  |  |  |  |  |  |
| Class B | -0.37% | -0.33% | -0.30% |  |  | -0.39% | -0.75% | -0.70% |  |  |
| Class C | -0.31% | -0.23% | -0.28% |  |  | -0.55% | -0.80% | -0.50% |  |  |
| Class E |  |  |  |  |  | -0.24% | -0.07% | -0.22% |  |  |
| Overall | -0.34% | -0.29% | -0.33% |  |  | -0.41% | -0.59% | -0.51% |  |  |
| Class D | -0.22% | -0.14% | -0.26% |  |  | -0.26% | -0.36% | -0.62% |  |  |
| Class F | -0.24% | -0.21% | -0.27% |  |  |  |  |  |  |  |

Table 3. Performance of Test 3.5b

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.41% | -0.44% | 99% | 95% |  |  |  |  |  |
| Class A2 | -0.41% | -0.43% | -0.43% | 100% | 97% |  |  |  |  |  |
| Class B | -0.40% | -0.37% | -0.28% | 101% | 101% | -0.46% | -0.47% | -0.48% | 102% | 104% |
| Class C | -0.37% | -0.31% | -0.32% | 99% | 101% | -0.58% | -0.42% | -0.68% | 100% | 116% |
| Class E |  |  |  |  |  | -0.36% | -0.37% | -0.17% | 100% | 89% |
| Overall | -0.38% | -0.37% | -0.35% | 100% | 99% | -0.47% | -0.43% | -0.47% | 101% | 104% |
| Class D | -0.23% | -0.13% | -0.24% | 101% | 110% | -0.18% | 0.06% | -0.01% | 102% | 119% |
| Class F | -0.24% | -0.26% | -0.40% | 102% | 91% | -0.20% | -0.31% | -0.47% | 100% | 86% |

Table 4. Performance of Test 3.6a

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.73% | -0.72% |  |  |  |  |  |  |  |
| Class A2 | -0.58% | -0.57% | -0.65% |  |  |  |  |  |  |  |
| Class B | -0.49% | -0.44% | -0.39% |  |  | -0.46% | -0.51% | -0.75% |  |  |
| Class C | -0.57% | -0.48% | -0.43% |  |  | -0.94% | -0.75% | -0.94% |  |  |
| Class E |  |  |  |  |  | -0.12% | 0.50% | -0.11% |  |  |
| Overall | -0.54% | -0.54% | -0.52% |  |  | -0.54% | -0.34% | -0.66% |  |  |
| Class D | -0.46% | -0.25% | -0.24% |  |  | -0.84% | -0.80% | -1.73% |  |  |
| Class F | -0.34% | -0.41% | -0.33% |  |  | -0.59% | -0.36% | -1.03% |  |  |

Table 5. Performance of Test 3.3a + Test 3.4 + Test 3.5a

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.49% | -0.37% | -0.41% |  |  |  |  |  |  |  |
| Class C | -0.50% | -0.44% | -0.57% |  |  | -1.16% | -1.30% | -1.15% |  |  |
| Class E |  |  |  |  |  | -0.37% | 0.29% | -0.60% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.45% | -0.30% | -0.31% |  |  | -1.12% | -0.88% | -2.03 |  |  |
| Class F | -0.34% | -0.42% | -0.39% |  |  |  |  |  |  |  |

# Conclusion

This contribution presents tests of EE2-3.4 ~ EE2-3.6. The proposed methods contribute promising coding gains with negligible encoding time change and acceptable decoding time. It is recommended to adopt this contribution into ECM.

# References

- M. Coban, F. Le Léannec, M. Sarwer, and J. Ström, “Algorithm description of Enhanced Compression Model 3 (ECM 3),” JVET-X2025, Dec. 2021.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).