| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0139-v1 |
| --- | --- |

| Title: | Non-EE2: On the extended number of active reference pictures and reference picture reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the result of extending the number of active reference pictures in random access configurations. The averaged BD-rate performance is -0.xx%, -0.xx%, -0.xx%. This contribution also proposes a reference picture reordering method. Combined with the extended number of active reference pictures, it reports -0.xx%, -0.xx%, -0.xx% BD-rate in random access configurations. Under the common test conditions, -0.xx%, -0.xx%, -0.xx% BD-rate in random access configurations and  -0.xx%, -0.xx%, -0.xx% BD-rate in low delay B configurations are reported.

# Introduction

In the random access configurations of current common test conditions for enhanced compression tool testing, the maximum number of active reference pictures in each reference list is set to 2. This contribution reports the result of extending the number of active reference pictures in the random access configurations by setting the number of active reference pictures is equal to the number of available reference pictures. The detail of settings for one GOP is provided in the attached configure file.

In this contribution, a reference picture reordering method is also proposed to allow block level adaptation of reference picture index assignment.

# Proposed reference picture reordering method

The proposed reference picture reordering method is based on the template matching cost.  For the uni-prediction AMVP mode, the reference pictures in List 0 and List 1 are interweaved to generate a joint list. For each hypothesis of the reference picture in the joint list, motion information can be derived accordingly and template matching is performed to calculate the cost. The joint list is reordered based on ascending order of the template matching cost. The index of the selected reference picture in the reordered joint list is signaled in the bitstream. For the bi-prediction AMVP mode, a list of pairs of reference pictures from List 0 and List 1 is generated and similarly reordered based on the template matching cost. The index of the selected pair is signaled in the bitstream.

# Test results

Three test results are reported in this contribution. All the tests use the ECM-3.1 under the common test conditions as anchor. In the Test 1, the number of active reference pictures in the random access configurations is set equal to number of available reference pictures. The result is shown in Table 1, -0.xx%, -0.xx%, -0.xx% BD-rate is reported. In the Test 2, the proposed reference picture reorder method is applied on top of ECM-3.1. The number of active reference pictures in the random access configurations is set equal to number of available reference, same as in first test. The result is shown in Table 2, -0.xx%, -0.xx%, -0.xx% BD-rate is reported. In the Test 3, the proposed reference picture reorder method is evaluated under common test conditions of ECM-3.1. The result is shown in Table 3, -0.xx%, -0.xx%, -0.xx% BD-rate is reported.

Table 1 Performance of ECM3.1 with extended number of active reference pictures

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.10% | -0.41% | 102% | 99% |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.30% | -0.31% | 85% | 89% |
| Class C | -0.23% | -0.23% | -0.20% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.05% | 0.04% | 0.10% | 106% | 102% |

Table 2 Performance of proposed reference picture reordering method with extended number of active reference pictures

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | -0.36% | -0.37% | 104% | 103% |
| Class C | -0.45% | -0.41% | -0.43% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.28% | -0.13% | -0.15% | 107% | 106% |

Table 3 Performance of proposed reference picture reordering method under common test conditions

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.13% |  | -0.07% |  | -0.06% |  | 100% |  | 101% |
| Class C | -0.17% |  | -0.07% |  | -0.05% |  | 100% |  | 103% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.19% |  | -0.10% |  | -0.18% |  | 102% |  | 107% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C | -0.30% | -0.56% |  | -0.60% |  | 101% |  | 115% |  |
| Class E | -0.14% | 0.75% |  | -0.46% |  | 100% |  | 106% |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.29% | -0.34% |  | -0.99% |  | 104% |  | 118% |  |

# Conclusion

In this contribution, it reports test results of extending the number of active reference pictures and reference picture reordering method. Overall, -0.xx%, -0.xx%, -0.xx% BD-rate can be achieved in random access configurations.  It’s suggested to study the proposed configuration and reference picture reordering method in EE.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).