| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0180-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0150 "EE2-1: Tests on unsymmetric partitioning methods", tests 1.1a and 1.1b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Le Léannec | Tel: Email: | fabricel@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking of EE2 tests 1.1a and 1.1b on unsymmetric partitioning methods.

It is reported that the submitted EE2 test 1.1 source code matches the description given in JVET-Y0150.

Moreover, partial simulations have been achieved, at the time this cross-check report is uploaded, and the match the results reported in JVET-Y0150, both for BD-rate results and runtimes.

# Introduction

Contribution JVET-Y0150 proposes two asymmetric partitioning method on top of ECM-3.1, which are ABT (Asymmetric Binary Tree) and UQT (Unsymmetric Quad-Tree).

ABT introduces 4 split modes. ABT-H1 and ABT-H2 split a CU with dimension W×H into two child-CUs with dimensions {W×H/4, W×3H/4} and {W×3H/4, W×H/4}, respectively. ABT-V1 and ABT-V2 split a CU with dimension W×H into two child-CUs with dimensions {W/4×H, 3W/4×H} and {3W/4×H, W/4×H}, respectively.

UQT introduces 4 other splits modes. UQT-H1 and UQT-H2 split a CU with dimension W×H into four child-CUs with dimensions {W×H/8, W×H/2, W×H/4, W×H/8} and {W×H/8, W×H/4, W×H/2, W×H/8}, respectively. UQT-V1 and UQT-V2 split a CU with dimension W×H into four child-CUs with dimensions {W/8×H, W/2×H, W/4×H, W/8×H} and {W/8×H, W/4×H, W/2×H, W/8×H}, respectively.

Tests 1.1a and 1.1b, i.e. ECM-3.1 with ABT-only and UQT-only respectively have been run in ECM test conditions [1]. The obtained partial results and runtime match the results reported in contribution JVET-Y0150.

Table 1: partial random access cross-check results of test 1.1a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class C | -0.48% | -0.76% | -0.64% | 170% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class D | -0.36% | -0.34% | -0.50% | 175% | 102% |
| Class F | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class TGM | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |

Table 2: partial random access cross-check results of test 1.1b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class C | -0.26% | -0.24% | -0.29% | 126% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | -0.11% | -0.06% | -0.30% | 122% | 100% |
| Class F | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class TGM | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |

Moreover, the source code of EE2 Test 1 has also been carefully reviewed and conforms to the technical description provided in EE2.

# Cited References

[1] JVET-W2017-v1, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, Marta Karczewicz Yan Ye, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.