| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0214-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0061: EE1-2.1: Super Resolution with existing VVC functionality |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This document verifies reported PSNR and bitrate numbers in JVET-Y0061 (EE1-2.1). It additionally also confirms PSNR and bitrate that proponent reported for the remote viewing test.

# Introduction

In some case it can be beneficial to encode a sequence in lower resolution and then upscale it before display instead of coding it in full resolution. VVC provide RPR which can enable such functionality.

In JVET-Y0061 [1] they have selected to use RPR with 2x down-scaling when it has compression benefits over encoding in original resolution. When RPR is used they have also determined the QP that gives a bitrate within 10% of the bitrate from encoding at original resolution.

# Results

We confirm the reported bitrate and PSNR when RPR is used both for the NNVC CTC [3] and the remote viewing test [2].

Table 1: NNVC CTC

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -4,88% | 3,22% | 2,63% | -10,17% | -0,21% | -2,93% | 82% | 82% | 3% |
| Class A2 | -1,08% | 16,02% | 10,37% | -4,47% | 13,20% | 3,29% | 84% | 88% | 1% |
| Class B | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1,19% | 3,85% | 2,60% | -2,93% | 2,60% | 0,07% | 93% | 94% | 1% |
| Class D | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class F | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -2,10% | 2,49% | 3,94% | -4,45% | 2,07% | 4,51% | 104% | 82% | 2% |
| Class A2 | -1,77% | 5,70% | 4,11% | -6,79% | 4,13% | 0,98% | 95% | 85% | 2% |
| Class B | -0,02% | -0,08% | 0,27% | -0,21% | -0,14% | 0,54% | 100% | 98% | 0% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class E | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Overall | -0,65% | 1,34% | 1,42% | -1,93% | 0,99% | 1,06% | 100% | #DIV/0! | 1% |
| Class D | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class F | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 100% | 100% | 0% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 2: Remote viewing

| Sequence | QP | Bitrate | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- |
| Tango | 35 | 1168,134 | 36,3821 | 45,2788 | 41,8113 |
| Tango | 32 | 1667,923 | 37,3821 | 46,0637 | 42,7605 |
| Campfire | 32 | 2518,678 | 34,7948 | 32,1361 | 37,308 |
| Campfire | 28 | 4009,252 | 35,7458 | 32,6883 | 38,15 |
| CatRobot | 36 | 1110,766 | 34,7773 | 38,61 | 37,5183 |
| CatRobot | 33 | 1610,349 | 35,8285 | 39,2356 | 38,4039 |
| DaylightRoad | 36 | 1028,04 | 33,4158 | 41,601 | 39,1288 |
| DaylightRoad | 33 | 1521,741 | 34,2569 | 42,3688 | 39,7998 |

# References

[1] J. Sauer, E. Alshina, “EE1-2.1: Super Resolution with existing VVC functionality”, JVET-Y0061, on-line meeting, January 2022

[2] E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang , “Exploration Experiments on Neural Network-based Video Coding (EE1)”, JVET-X2023, on-line meeting, October 2022

[3] S. Liu, A. Segall, E. Alshina, R.-L. Liao , “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, on-line meeting, October 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).