| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0244-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0155 and combination with JVET-Y0126 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karsten Suehring | Email: | karsten.suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This crosscheck reports coding results for JVET-Y0155 (AHG10: Fixes and clean up for temporal prefilter) and provides additional results for the combination of JVET-Y0155 and JVET-Y0126 (AHG10: VTM encoder configurations for tests targeting improved coding performance). v2 of this document adds full results for LDP.

# Software

A first version of the software patch for VTM 15.0 was provided by the proponent of JVET-Y0155. The patch was reviewed to match the description. Suggestions for improvements were accepted and added into later versions of the patch. Results were created with the first version because these changes related to usability issues, not to coding performance.

It should be noted that the addition of the FirstValidFrame and LastValidFrame parameters may require adaptations to coding scripts, especially for segment-wise coding, because the parameters control which frames may be used for temporal prefiltering. If not set properly, segments may mismatch.

The patch for JVET-Y0126 was taken from the proposal document (v1) and applied on top of the patch for JVET-Y0155. The patch applied without conflicts and was not reviewed in detail.

# Results

## JVET-Y0155

The coding results match the results reported in the contribution. An Excel sheet with the results is attached to this crosscheck.

A mismatch was found in the anchor for the CatRobot sequence. While the proponent used a 300-frame version of the sequence, the crosschecker used a 600-frame version of the sequence. VTM-15.0 uses frames after the coded 300 frames for temporal prefiltering, when these frames are available. With 300 frames to be coded, additional frames were used for the crosscheck anchor, while these frames were not used for the proponent anchor. With the introduction of the LastValidFrame parameter and setting it to 300, this difference does not appear in the test for the proposal.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,09% | -0,58% | -0,71% | 100% | 97% |
| Class A2 | -0,12% | -0,56% | -0,42% | 99% | 98% |
| Class B | -0,38% | -0,87% | -0,83% | 99% | 96% |
| Class C | -0,03% | -0,54% | -0,28% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0,17% | -0,66% | -0,58% | 99% | 97% |
| Class D | 0,00% | -0,58% | -0,21% | 98% | 98% |
| Class F | 0,00% | 0,00% | 0,00% | 99% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,12% | -0,37% | -0,51% | 101% | 101% |
| Class C | 0,03% | -0,10% | -0,24% | 102% | 99% |
| Class E | -0,07% | -0,10% | -0,45% | 100% | 98% |
| Overall | -0,06% | -0,21% | -0,40% | 101% | 99% |
| Class D | 0,10% | -0,44% | 0,03% | 99% | 97% |
| Class F | 0,00% | 0,00% | 0,00% | 100% | 96% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,23% | -0,43% | -0,80% | 100% | 98% |
| Class C | -0,01% | 0,25% | -0,31% | 104% | 101% |
| Class E | -0,19% | 0,46% | 0,63% | 100% | 94% |
| Overall | -0,15% | 0,02% | -0,28% | 101% | 98% |
| Class D | 0,01% | 0,01% | -0,12% | 99% | 98% |
| Class F | 0,00% | 0,00% | 0,00% | 101% | 97% |

## JVET-Y0155 + JVET-Y0126

In random access configuration, it can be observed that the gains reported for JVET-Y0155 and JVET-Y0126 appear to be additive.

In low delay configuration, the combined results so far appear close to the gain reported in JVET-Y0126, when compared to the VTM 14.0 anchor. This is expected because there is rather small performance impact of JVET-Y0155 for the LD configurations. The largest gain would be expected in class B LDP, which for which the run (of JVET-Y0126) is unfortunately not finished yet.

The full Excel sheet is attached to this contribution.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,66% | -4,15% | -5,11% | 157% | 99% |
| Class A2 | -2,91% | -5,91% | -5,02% | 166% | 99% |
| Class B | -3,42% | -8,32% | -8,56% | 168% | 94% |
| Class C | -3,00% | -7,37% | -7,04% | 177% | 90% |
| Class E |  |  |  |  |  |
| Overall | -2,85% | -6,75% | -6,76% | 168% | 95% |
| Class D | -2,12% | -7,48% | -6,97% | 173% | 93% |
| Class F | -3,29% | -5,06% | -4,90% | 173% | 92% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1,63% | -1,49% | -1,11% | 154% | 91% |
| Class E | -3,27% | -4,04% | -3,51% | 127% | 92% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1,20% | -1,23% | -1,62% | 152% | 92% |
| Class F | -1,51% | -1,41% | -1,26% | 134% | 94% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3,85% | -3,42% | -4,22% | 148% | 98% |
| Class C | -1,75% | -1,41% | -1,23% | 160% | 94% |
| Class E | -3,41% | -3,95% | -4,93% | 135% | 90% |
| Overall | -3,04% | -2,88% | -3,40% | 148% | 95% |
| Class D | -1,46% | -2,06% | -1,80% | 157% | 94% |
| Class F | -1,38% | -0,63% | -0,95% | 135% | 95% |