| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0062-v0 |
| --- | --- |

| Title: | AHG12: RPR luma filter modifications and RPR enabling fixes in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Kenneth Andersson | Email: | ruoyang.yu@ericsson.com kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In ECM-4.0, the luma interpolation filters used in sub-pel motion compensation for both affine and non-affine blocks are 12-tap. These 12-tap luma filters are also used as up-sampling RPR filters for non-affine blocks in the motion compensation process when the reference picture has a smaller spatial resolution than the current picture. However, the up-sampling RPR filters for affine blocks remain to be 6-tap. It is also noted that when the reference picture has a larger spatial resolution than the current picture, the down-sampling luma RPR filters used in the motion compensation process remain to be 8-tap for non-affine blocks and 6-tap for affine blocks.

This contribution proposes to use the existing 12-tap up-sampling RPR filters also for affine blocks and proposes to increase the down-sampling luma RPR filters to 12-tap to align with the filter taps of the up-sampling RPR filters. Two sets of 12-tap down-sampling luma RPR filters are proposed, one set mainly for 1.5x down-sampling and the other set mainly for 2x down-sampling.

Furthermore, it is found that ECM-4.0 does not fully supports RPR enabling, especially when the RPR configuration enables picture resolution switching. The contribution identifies the issues and proposes corresponding fixes.

The proposed up-sampling RPR filter changes for affine blocks as well as the 12-tap down-sampling RPR filters are tested on top of ECM-4.0 with the proposed RPR fixes. The test configuration used are RA 1.5x and RA 2x scaling. Both test configurations enable picture resolution switching every 16 pictures. The BD-rate impact is reported as -0.25% (Y) and -0.11% (Y), respectively.

# Introduction and Proposal

## ECM luma RPR filters

In ECM-4.0 [1], the luma interpolation filters used in sub-pel motion compensation for both affine and non-affine blocks are 12-tap. When the reference picture has a smaller spatial resolution than the current picture, these 12-tap luma filters are also used as up-sampling RPR filters in the motion compensation process for non-affine blocks. However, the up-sampling RPR filters that used in the motion compensation process for affine blocks remain to be 6-tap.

When the reference picture has a larger spatial resolution than the current picture, it is noted that the down-sampling luma RPR filters used in the motion compensation process remains to be 8-tap for non-affine blocks and 6-tap for affine blocks.

To resolve such a design inconsistency, it is proposed to use the existing 12-tap up-sampling RPR filters for affine blocks and increase the down-sampling luma RPR filters also to 12-tap for both affine and non-affine blocks. Two sets of 12-tap down-sampling luma RPR filters are proposed, one set mainly for 1.5x down-sampling and the other mainly for 2x down-sampling.

### Proposed up-sampling luma RPR filter changes for affine blocks

For affine blocks, it is proposed to use the existing 12-tap interpolation filters as the up-sampling luma RPR filters.

### Proposed 1.5x down-sampling luma RPR filters

The proposed 12-tap filter coefficients are shown in the following table.

Table 1. Proposed coefficients for 1.5x down-sampling 12-tap luma RPR filters

| Fractional sample position | Interpolation filter coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0/16 | 0 | 10 | -12 | -19 | 70 | 158 | 70 | -19 | -12 | 10 | 0 | 0 |
| 1/16 | 0 | 10 | -9 | -22 | 62 | 156 | 79 | -17 | -13 | 9 | 1 | 0 |
| 2/16 | -1 | 10 | -8 | -22 | 54 | 155 | 87 | -13 | -16 | 9 | 2 | -1 |
| 3/16 | -2 | 10 | -5 | -23 | 46 | 152 | 95 | -9 | -18 | 9 | 2 | -1 |
| 4/16 | -2 | 9 | -4 | -23 | 39 | 150 | 103 | -6 | -20 | 8 | 4 | -2 |
| 5/16 | -2 | 8 | -1 | -24 | 33 | 144 | 111 | -2 | -20 | 6 | 5 | -2 |
| 6/16 | -3 | 9 | -1 | -24 | 26 | 142 | 117 | 4 | -22 | 5 | 5 | -2 |
| 7/16 | -3 | 8 | 0 | -23 | 20 | 137 | 123 | 10 | -23 | 4 | 5 | -2 |
| 8/16 | -2 | 6 | 3 | -25 | 16 | 130 | 130 | 16 | -25 | 3 | 6 | -2 |
| 9/16 | -2 | 5 | 4 | -23 | 10 | 123 | 137 | 20 | -23 | 0 | 8 | -3 |
| 10/16 | -2 | 5 | 5 | -22 | 4 | 117 | 142 | 26 | -24 | -1 | 9 | -3 |
| 11/16 | -2 | 5 | 6 | -20 | -2 | 111 | 144 | 33 | -24 | -1 | 8 | -2 |
| 12/16 | -2 | 4 | 8 | -20 | -6 | 103 | 150 | 39 | -23 | -4 | 9 | -2 |
| 13/16 | -1 | 2 | 9 | -18 | -9 | 95 | 152 | 46 | -23 | -5 | 10 | -2 |
| 14/16 | -1 | 2 | 9 | -16 | -13 | 87 | 155 | 54 | -22 | -8 | 10 | -1 |
| 15/16 | 0 | 1 | 9 | -13 | -17 | 79 | 156 | 62 | -22 | -9 | 10 | 0 |

The proposed filter was designed based on 12-tap SHM [2] source 1.5x down-sampling filters. Compared to the 12-tap SHM source 1.5x down-sampling filters, the proposed 12-tap filters have more accurate phase-delay characteristics while maintains the magnitude response of the SHM 1.5x filter to a large extent. The following figure shows a comparison of the magnitude response as well as the phase-delay between the proposed filter and the SHM filter at fractional position 6/16.

Figure 1. Magnitude and phase delay of the SHM filter and the proposed filter at 6/16

### Proposed 2.0x down-sampling luma RPR filters

The proposed 12-tap filter coefficients are shown in the following table. The proposed filters are the same as the 12-tap SHM source 2.0x down-sampling filters but with filter gains increased from 128 to 256.

Table 2. Proposed coefficients for 2.0x down-sampling 12-tap luma RPR filters

| Fractional sample position | Interpolation filter coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0/16 | 4 | -6 | -18 | 12 | 78 | 116 | 78 | 12 | -18 | -6 | 4 | 0 |
| 1/16 | 4 | -6 | -18 | 8 | 76 | 116 | 86 | 14 | -18 | -8 | 2 | 0 |
| 2/16 | 4 | -4 | -18 | 4 | 70 | 116 | 88 | 18 | -16 | -8 | 2 | 0 |
| 3/16 | 2 | -4 | -18 | 2 | 68 | 116 | 92 | 22 | -16 | -10 | 2 | 0 |
| 4/16 | 2 | -2 | -16 | -2 | 62 | 114 | 94 | 26 | -14 | -10 | 2 | 0 |
| 5/16 | 2 | -2 | -16 | -4 | 58 | 112 | 98 | 30 | -14 | -12 | 2 | 2 |
| 6/16 | 2 | 0 | -16 | -6 | 52 | 110 | 102 | 34 | -14 | -12 | 2 | 2 |
| 7/16 | 2 | 0 | -14 | -8 | 48 | 108 | 104 | 38 | -12 | -14 | 2 | 2 |
| 8/16 | 2 | 0 | -14 | -10 | 44 | 106 | 106 | 44 | -10 | -14 | 0 | 2 |
| 9/16 | 2 | 2 | -14 | -12 | 38 | 104 | 108 | 48 | -8 | -14 | 0 | 2 |
| 10/16 | 2 | 2 | -12 | -14 | 34 | 102 | 110 | 52 | -6 | -16 | 0 | 2 |
| 11/16 | 2 | 2 | -12 | -14 | 30 | 98 | 112 | 58 | -4 | -16 | -2 | 2 |
| 12/16 | 0 | 2 | -10 | -14 | 26 | 94 | 114 | 62 | -2 | -16 | -2 | 2 |
| 13/16 | 0 | 2 | -10 | -16 | 22 | 92 | 116 | 68 | 2 | -18 | -4 | 2 |
| 14/16 | 0 | 2 | -8 | -16 | 18 | 88 | 116 | 70 | 4 | -18 | -4 | 4 |
| 15/16 | 0 | 2 | -8 | -18 | 14 | 86 | 116 | 76 | 8 | -18 | -6 | 4 |

## ECM RPR enabling issues and proposed fixes

It is found that ECM-4.0 does not fully supports RPR enabling, especially when the RPR configuration enables picture resolution switching. The issues that identified in this contribution are listed in the following table.

| Issue related macros | Issues |
| --- | --- |
| JVET_Y0065_GPM_INTRA | Fetching of motion information and IPM (intra prediction mode) information from reference picture do not consider resolution difference between the reference picture and current picture. |
| JVET_Y0067_ENHANCED_MMVD_MVD_SIGN_PRED | a) The template based MVD sign derivation is enabled regardless of resolution difference between the reference picture and the current picture. b) In MMVD candidate reordering (function sortInterMergeMMVDCandidates() and function sortAffineMergeCandidates()), the prediction sample generation for templates does not consider reference picture resolution difference from the current picture resolution. |
| JVET_Y0134_TMVP_NAMVP_CAND_REORDERING | In merge candidate reordering inside a same candidate type group (function adjustMergeCandidatesInOneCandidateGroup()), the prediction sample generation for templates does not consider reference picture resolution difference from the current picture resolution. |
| ALF_IMPROVEMENT | The SIMD code in function simdFilter9x9Blk() does not support input width being non-dividable by 8. This scenario can happen for boundary chroma CTB for class D when using 1.5x scaling. |

The proposed fixes for the above issues are summarized in the following table.

| Issue related macros | Proposed fixes |
| --- | --- |
| JVET_Y0065_GPM_INTRA | Add coordinates scaling according to the resolution ratio between the reference picture and the current picture when fetching motion information and IPM. This fix follows the same sprit as previous adopted RPR fixes in JVET-X0121 [3]. |
| JVET_Y0067_ENHANCED_MMVD_MVD_SIGN_PRED | a) Add reference picture and current picture resolution checks for the template based MVD sign derivation. When the reference picture is determined to not be scaled, the template based MVD sign derivation is enabled. When the reference picture is determined to be scaled, the MVD sign information is signaled and parsed using the conventional VVC approach. b) In MMVD candidate reordering (function sortInterMergeMMVDCandidates() and function sortAffineMergeCandidates()), when a candidate is associated with a rescaled reference picture, the prediction sample generation for the templates and the corresponding template cost calculation for the candidate is bypassed. The corresponding template cost for the candidate is set to 0. This fix follows the same sprit as previous adopted RPR fixes in JVET-X0121 and JVET-Y0128[4]. |
| JVET_Y0134_TMVP_NAMVP_CAND_REORDERING | In merge candidate reordering inside a same candidate type group (function adjustMergeCandidatesInOneCandidateGroup()), when a candidate is associated with a rescaled reference picture, the prediction sample generation for the templates and the corresponding template cost calculation for the candidate is bypassed. The corresponding template cost for the candidate is set to 0. This fix follows the same sprit as previous adopted RPR fixes in JVET-X0121 and JVET-Y0128. |
| ALF_IMPROVEMENT | In the SIMD function simdFilter9x9Blk(), when the width of the input block is non-dividable by 8, Non-SIMD version of the ALF filtering code is invoked. |

# Results

The proposed up-sampling luma RPR filter changes for affine blocks as well as the proposed 12-tap down-sampling RPR filters are tested on top of ECM-4.0 with the proposed RPR fixes.

## Results for proposed 1.5x down-sampling luma RPR filters (together with the up-sampling RPR filter changes for affine blocks)

In this test, the ECM Random Access CTC configuration [5] is used but with the following RPR configuration parameters enabled.

(Resolution switch after every 16 frames)

ScalingRatioHor : 1.5

ScalingRatioVer : 1.5

FractionNumFrames : 1.0

RPR : 1

UpscaledOutput : 2

SwitchPocPeriod : 16

The BD-rate PSNR results calculation is based on the PSNR2 (the PSNR value that measures after up-sampling of the decoded picture). It should be noted that the encoding and decoding run time are unreliable.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 with proposed RPR fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.01% | -0.01% | 101% | 99% |
| Class A2 | -0.18% | -0.16% | -0.13% | 100% | 99% |
| Class B | -0.30% | -0.25% | -0.20% | 97% | 98% |
| Class C | -0.37% | -0.34% | -0.23% | 108% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.21% | -0.16% | 101% | 99% |
| Class D | -0.40% | -0.02% | 0.04% | 93% | 98% |

## Results for proposed 2.0x down-sampling luma RPR filters (together with the up-sampling RPR filter changes for affine blocks)

In this test, the ECM Random Access CTC configuration is used but with the following RPR configuration parameters enabled.

(Resolution switch after every 16 frames)

ScalingRatioHor : 2.0

ScalingRatioVer : 2.0

FractionNumFrames : 1.0

RPR : 1

UpscaledOutput : 2

SwitchPocPeriod : 16

The BD-rate PSNR results calculation is based on the PSNR2 (the PSNR value that measures after up-sampling of the decoded picture). It should be noted that the encoding and decoding run time are unreliable.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 with proposed RPR fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.07% | -0.05% | 100% | 105% |
| Class A2 | -0.09% | -0.01% | 0.11% | 100% | 105% |
| Class B | -0.11% | -0.06% | 0.00% | 98% | 96% |
| Class C | -0.18% | -0.02% | -0.12% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.01% | -0.02% | 99% | 102% |
| Class D | -0.21% | 0.09% | 0.21% | 86% | 92% |

# Conclusion

This contribution proposes to use the existing 12-tap up-sampling RPR filters also for affine blocks and increase the down-sampling luma RPR filters in ECM to 12-tap to align with the filter taps of the up-sampling RPR filters. It is asserted the proposal improves the design consistency of RPR filters in ECM. Furthermore, since the ECM already supports 12-tap up-sampling RPR filters, the proposal is asserted to not increase the worst-case complexity.

The contribution also proposes fixes for RPR enabling in ECM. The proposed fixes are asserted to be straightforward and follow the spirit of previous adopted RPR fixes. It is confirmed that with the proposed fixes, ECM-4.0 can be run under RPR configurations with picture resolution switching enabled.

It is proposed to include the up-sampling RPR filter changes for affine blocks and the proposed 12-tap down-sampling luma RPR filters as well as the RPR enabling fixes into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

[2] https://vcgit.hhi.fraunhofer.de/jvet/SHM/-/tree/SHM-12.4

[3] P. Bordes, F. Galpin, F. Leleannec, E. Francois, “EE2-related: bug fixes for enabling RPR in ECM”, JVET-X0121, Oct. 2021.

[4] Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz, P. Bordes, “Non-EE2: Fixing issues for RPR enabling and non-CTC configuration in ECM”, JVET-Y0128, Jan. 2022.

[5] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).