| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0063-v3 |
| --- | --- |

| Title: | AHG12: Fix for parsing of MHP information in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In ECM-4.0, the tool multi-hypothesis prediction (MHP) allows signalling of multiple pieces of additional motion information for an inter block. The MHP is noted to be applicable for a current inter block when it is coded using normal merge, affine merge, MMVD or AMVP with BCW enabled. Furthermore, when the current block is coded using affine AMVP with BCW, the MHP is only applicable when the block is coded with affine AMVR off (e.g., with its motion vector difference resolution at quarter-pel). In other words, the presence of MHP motion information conditionally relies on the affine AMVR information. However, it is noted that the parsing of affine AMVR information is done after the MHP motion information is parsed. It is asserted that such a design has a parsing issue since the parsing of MHP motion information depends on undefined affine AMVR information. The contribution proposes a fix to resolve such a parsing issue. The proposed fix has been tested under ECM-4.0 RA and LDB common test configuration. The overall BD-rate impact for luma is reported to be 0.00% and -0.01%, respectively.

# Introduction

The multi-hypothesis prediction (MHP) tool in ECM-4.0 [1] allows signalling of multiple pieces of additional motion information for an inter block. The MHP is noted to be applicable for a current inter block when it is coded using normal merge, affine merge, MMVD or AMVP with BCW enabled. Furthermore, when the current block is coded using affine AMVP with BCW enabled, the MHP is only applicable when the block is coded with affine AMVR off (e.g., with its motion vector difference resolution at quarter-pel). In other words, the presence of MHP motion information conditionally relies on the affine AMVR information. However, it is noted that the parsing of the affine AMVR information is done after the MHP motion information is parsed. It is asserted that such a design has a parsing issue since the parsing of MHP motion information depends on undefined AMVR information.

The following figure shows an overview of the current parsing flow in ECM-4.0. It should be noted that the figure does not show the syntax structure in a typical specification format but contains some pseudocode for potentially easier explanation.

As can be seen from the figure above, the MHP motion information is parsed first (in mh_pred_data) which depends on the affine AMVR information (stored in variable pu.cu->imv, which is undefined at this parsing stage). After the parsing of MHP motion information, MVD information is parsed (in mvd_coding). The parsing of the MVD sign information depends on PresenceOfMhpData by checking the number of additional pieces of MHP motion information. When there is presence of additional piece of MHP motion information (PresenceOfMhpData equals to 1), the MVD sign information is parsed directly after the MVD magnitude information as in the conventional approach. Otherwise, the MVD sign information will be parsed at a later stage (in msvd_data) using the recent adopted JVET-Y0067 [2] sign index signalling approach. After the MVD information is parsed, the parsing of affine AMVR information is done (in affine_amvr_mode) and the affine AMVR information (pu.cu->imv) is finally assigned with a defined value.

It can also be noticed that it is not feasible to simply move the parsing of affine AMVR information (in affine_amvr_mode) before the parsing of MHP motion information (in mh_pred_data) since the presence of affine AMVR information depends on the presence of a non-zero MVD.

The asserted parsing issue is not triggered in ECM-4.0 CTC configurations. However, when using the CTC configuration but with “BcwFast = 0”, the ECM-4.0 encoder will generate a bitstream that is not decodable by the ECM-4.0 decoder.

# Proposal

The proposed fix adjusts the parsing order of related syntax elements as explained below.

It is proposed to first parse the MVD magnitude information, then parse the AMVR information (The presence of non-zero MVD can be determined based on the MVD magnitude information). After the AMVR information is parsed (where pu.cu->imv can be assigned with a defined value), the MHP motion information is parsed (with the defined pu.cu->imv value). After the MHP motion information is parsed, the MVD sign information is further parsed depending on the presence of MHP motion information (i.e., when MHP motion information is present, the MVD sign information is parsed using the conventional signalling. Otherwise, the MVD sign information is parsed based on the JVET-Y0067 template-based sign index signalling).

The following figure shows the proposed parsing flow.

# Results

The proposed fix was tested and compared against ECM-4.0 anchor under the ECM RA common test configurations [3]. The encoding run time and decoding run time are asserted to be unreliable.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.03% | 104% | 104% |
| Class A2 | 0.01% | -0.04% | -0.04% | 101% | 103% |
| Class B | 0.00% | 0.00% | 0.04% | 102% | 98% |
| Class C | 0.00% | 0.07% | -0.06% | 107% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 104% | 101% |
| Class D | 0.00% | 0.01% | -0.02% | 112% | 104% |

It is confirmed that together the proposed fix, the ECM-4.0 generated bitstream using CTC with “BcwFast = 0” can be decodable.

In v2 of the document, the proposed fix was reported to be further tested and compared against ECM-4.0 under ECM LDB CTC [3]. The BD-rate result is shown in the table below. The encoding run time and decoding run time are asserted to be unreliable.

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.24% | -0.11% | 104% | 101% |
| Class C | -0.04% | 0.25% | -0.08% | 100% | 106% |
| Class E | -0.06% | 0.17% | -0.46% | 100% | 105% |
| Overall | -0.01% | 0.22% | -0.19% | 102% | 104% |
| Class D | -0.02% | 0.25% | 0.40% | 96% | 98% |

# Conclusion

This contribution proposes a fix for the parsing issue of MHP motion information. It is asserted that the proposed fix is straightforward and does not require any additional encoder side changes. It is therefore proposed to include the proposed fix to the next version of ECM.

# Reference

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

[2] M. Salehifar, Y. He, K. Zhang, N. Zhang, L. Zhang, Y. Zhang, B. Ray, H. Huang, V. Seregin, M. Karczewicz, “EE2-3.9 and EE2-3.10: TM based reordering for MMVD and affine MMVD and MVD sign prediction”, JVET-Y0067, Jan. 2022.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).