| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0068-v1 |
| --- | --- |

| Title: | EE2-2.6 related: Longer chroma interpolation filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson .com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The chroma interpolation filters in ECM-4.0 are 4-tap which is much shorter than the 12-tap luma interpolation filters. In JVET-Y0172, it was shown that healthy chroma gains can be obtained by increasing the chroma interpolation filter length also to 12-tap. In EE2-2.6 three sets of chroma interpolation filter lengths are tested, 12, 8 and 6-tap. In this contribution three alternative sets of chroma interpolation filters are proposed. It is asserted that the proposed sets have more accurate phase characteristics at lower frequencies.

The impact on coding efficiency over ECM-4.0 anchor is reported for Luma/Cb/Cr as follows:

12-tap: RA: -0,06%/-1,70%/-1,68%, LDB: -0,10%/-2,11%/-1,88%

8-tap:   RA: -0,06%/-1,44%/-1,47%, LDB: -0,10%/-2,04%/-1,81%

6-tap:   RA: -0,05%/-1,25%/	-1,17%, LDB: -0,10%/-1,36%/-1,48%

Encoding and decoding time is asserted to be similar as the anchor.

Since most of the gains can be kept with 6-tap or 8-tap it is thus suggested to include one of the proposed 6-tap or 8-tap interpolation filter sets in the next version of ECM.

# Introduction

Longer chroma interpolation filters have shown to improve coding efficiency for ECM [1]. To investigate the trade-off of different filter lengths 12-tap, 8-tap and 6-tap are tested in EE2-2.6 [2].

By inspection of the phase and magnitude responses we noted that the phases of the filters tested in EE2-2.6 were misaligned at lower frequencies. The aim of this contribution is to alleviate such phase misalignment issues.

Below the phase and magnitude responses of the filters suggested in EE2-2.6 are shown. The green straight lines in the top part (a) of the figures show the ideal phase delay responses, and the red curves show the phase delay responses for the EE2-2.6 filters. Likewise, the green line in the bottom part (b) of the figures shows the ideal magnitude responses and the red curves show the magnitude responses of the EE2-2.6 filters. It can be shown that the EE2-2.6 filters have suboptimal phase characteristics at lower frequencies. Especially noticeable for 12-tap and 8-tap but also for 6-tap filters.

Figure 1: Plots of phase delay responses (a) and magnitude of frequency responses (b) of 12-tap chroma interpolation filters in EE2-2.6.

Figure 2: Plots of phase delay responses (a) and magnitude of frequency responses (b) of 8-tap chroma interpolation filters in EE2-2.6.

Figure 3: Plots of phase delay responses (a) and magnitude of frequency responses (b) of 6-tap chroma interpolation filters in EE2-2.6.

# Proposal

It is suggested to select one set of the enhanced chroma interpolation filters from below.

12-tap:

{{ 0, 0, 0, 0, 0, 256, 0, 0, 0, 0, 0, 0, },

{  0, 1, -2, 3, -7, 255, 8, -3, 2, -1, 0, 0, },

{ -1, 3, -4, 6, -14, 254, 17, -8, 5, -3, 1, 0, },

{ -1, 3, -5, 9, -21, 253, 25, -10, 5, -3, 1, 0, },

{ -1, 4, -8, 13, -26, 249, 34, -14, 8, -4, 1, 0, },

{ -2, 6, -10, 16, -32, 246, 44, -18, 9, -4, 1, 0, },

{ -2, 5, -9, 17, -36, 241, 54, -22, 12, -6, 3, -1, },

{ -1, 4, -9, 18, -39, 235, 65, -26, 13, -6, 3, -1, },

{ -2, 6, -12, 22, -44, 230, 75, -28, 14, -7, 3, -1, },

{ -2, 6, -12, 23, -47, 224, 85, -31, 15, -7, 3, -1, },

{ -2, 7, -13, 23, -48, 216, 97, -36, 18, -9, 4, -1, },

{ -2, 6, -12, 23, -49, 209, 106, -37, 19, -10, 4, -1, },

{ -3, 8, -14, 26, -52, 200, 119, -42, 22, -12, 6, -2, },

{ -2, 6, -12, 24, -51, 191, 129, -43, 22, -12, 6, -2, },

{ -2, 7, -14, 25, -50, 180, 141, -46, 23, -12, 6, -2, },

{ -2, 6, -12, 24, -50, 171, 151, -47, 23, -12, 6, -2, },

{ -2, 6, -13, 25, -50, 162, 162, -50, 25, -13, 6, -2, },

{ -2, 6, -12, 23, -47, 151, 171, -50, 24, -12, 6, -2, },

{ -2, 6, -12, 23, -46, 141, 180, -50, 25, -14, 7, -2, },

{ -2, 6, -12, 22, -43, 129, 191, -51, 24, -12, 6, -2, },

{ -2, 6, -12, 22, -42, 119, 200, -52, 26, -14, 8, -3, },

{ -1, 4, -10, 19, -37, 106, 209, -49, 23, -12, 6, -2, },

{ -1, 4, -9, 18, -36, 97, 216, -48, 23, -13, 7, -2, },

{ -1, 3, -7, 15, -31, 85, 224, -47, 23, -12, 6, -2, },

{ -1, 3, -7, 14, -28, 75, 230, -44, 22, -12, 6, -2, },

{ -1, 3, -6, 13, -26, 65, 235, -39, 18, -9, 4, -1, },

,{ -1, 3, -6, 12, -22, 54, 241, -36, 17, -9, 5, -2, },

{ 0, 1, -4, 9, -18, 44, 246, -32, 16, -10, 6, -2, },

{ 0, 1, -4, 8, -14, 34, 249, -26, 13, -8, 4, -1, },

{ 0, 1, -3, 5, -10, 25, 253, -21, 9, -5, 3, -1, },

{ 0, 1, -3, 5, -8, 17, 254, -14, 6, -4, 3, -1, },

{ 0, 0, -1, 2, -3, 8, 255, -7, 3, -2, 1, 0, }}

8-tap:

{{ 0, 0, 0, 256, 0, 0, 0, 0, },

{ -1, 3, -7, 254, 11, -6, 3, -1, },

{ -1, 4, -12, 253, 16, -6, 3, -1, },

{ -2, 7, -18, 249, 28, -12, 6, -2, },

{ -2, 8, -23, 248, 33, -11, 4, -1, },

{ -3, 11, -28, 242, 47, -19, 8, -2, },

{ -3, 12, -33, 241, 51, -16, 5, -1, },

{ -4, 15, -38, 236, 62, -21, 8, -2, },

{ -5, 18, -43, 231, 72, -23, 8, -2, },

{ -6, 19, -44, 223, 84, -28, 12, -4, },

{ -4, 15, -40, 210, 99, -35, 15, -4, },

{ -3, 12, -36, 195, 117, -42, 18, -5, },

{ -4, 15, -40, 190, 124, -42, 18, -5, },

{ -4, 15, -41, 183, 133, -42, 16, -4, },

{ -4, 15, -42, 176, 142, -44, 18, -5, },

{ -5, 17, -42, 165, 153, -45, 18, -5, },

{ -4, 16, -44, 160, 160, -44, 16, -4, },

{ -5, 18, -45, 153, 165, -42, 17, -5, },

{ -5, 18, -44, 142, 176, -42, 15, -4, },

{ -4, 16, -42, 133, 183, -41, 15, -4, },

{ -5, 18, -42, 124, 190, -40, 15, -4, },

{ -5, 18, -42, 117, 195, -36, 12, -3, },

{ -4, 15, -35, 99, 210, -40, 15, -4, },

{ -4, 12, -28, 84, 223, -44, 19, -6, },

{ -2, 8, -23, 72, 231, -43, 18, -5, },

{ -2, 8, -21, 62, 236, -38, 15, -4, },

{ -1, 5, -16, 51, 241, -33, 12, -3, },

{ -2, 8, -19, 47, 242, -28, 11, -3, },

{ -1, 4, -11, 33, 248, -23, 8, -2, },

{ -2, 6, -12, 28, 249, -18, 7, -2, },

{ -1, 3, -6, 16, 253, -12, 4, -1, },

{ -1, 3, -6, 11, 254, -7, 3, -1, }}

6-tap:

{{0, 0, 256, 0, 0, 0, },

{ 1, -6, 256, 6, -1, 0, },

{ 2, -11, 254, 14, -4, 1, },

{ 4, -18, 252, 23, -6, 1, },

{ 6, -24, 249, 32, -9, 2, },

{ 6, -26, 244, 41, -12, 3, },

{ 7, -30, 239, 53, -18, 5, },

{ 8, -34, 235, 61, -19, 5, },

{ 10, -38, 228, 72, -22, 6, },

{ 10, -39, 220, 84, -26, 7, },

{ 10, -40, 213, 94, -29, 8, },

{ 11, -42, 205, 105, -32, 9, },

{ 11, -42, 196, 116, -35, 10, },

{ 11, -42, 186, 128, -37, 10, },

{ 11, -42, 177, 138, -38, 10, },

{ 11, -41, 167, 148, -40, 11, },

{ 11, -41, 158, 158, -41, 11, },

{ 11, -40, 148, 167, -41, 11, },

{ 10, -38, 138, 177, -42, 11, },

{ 10, -37, 128, 186, -42, 11, },

{ 10, -35, 116, 196, -42, 11, },

{ 9, -32, 105, 205, -42, 11, },

{ 8, -29, 94, 213, -40, 10, },

{ 7, -26, 84, 220, -39, 10, },

{ 6, -22, 72, 228, -38, 10, },

{ 5, -19, 61, 235, -34, 8, },

{ 5, -18, 53, 239, -30, 7, },

{ 3, -12, 41, 244, -26, 6, },

{ 2, -9, 32, 249, -24, 6, },

{ 1, -6, 23, 252, -18, 4, },

{ 1, -4, 14, 254, -11, 2, },

{ 0, -1, 6, 256, -6, 1, }}

The phase and magnitude responses for the proposed filters are shown in below figures 4 to 6. The proposed filters are closer to the ideal phase and magnitude especially at lower frequencies.

Figure 4: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 12-tap chroma interpolation filters.

Figure 5: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 8-tap chroma interpolation filters.

Figure 6: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 6-tap chroma interpolation filters.

# Results

The objective performance is compared against ECM-4.0 [3] for CTC in [4]. We also looked at some decoded sequences based on the different filter lengths in comparison to the ECM 4-tap filter and did not find any issue.

## Proposed 12-tap chroma IF for CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,11% | -0,17% | 99% | 114% |
| Class A2 | -0,05% | -1,16% | -1,20% | 100% | 112% |
| Class B | -0,10% | -1,78% | -1,88% | 97% | 105% |
| Class C | -0,07% | -3,20% | -2,94% | 103% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0,06% | -1,70% | -1,68% | 100% | 110% |
| Class D | -0,05% | -4,85% | -5,26% | 105% | 106% |
| Class F | -0,03% | -1,32% | -1,39% | 94% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,14% | -2,52% | -2,64% | 97% | 102% |
| Class C | -0,09% | -3,05% | -3,05% | 99% | 102% |
| Class E | -0,05% | -0,16% | 0,95% | 98% | 102% |
| Overall | -0,10% | -2,11% | -1,88% | 98% | 102% |
| Class D | -0,24% | -4,01% | -5,81% | 97% | 107% |
| Class F | -0,06% | -0,52% | -0,76% | 93% | 95% |

## Proposed 8-tap chroma IF for CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,19% | -0,20% | 103% | 109% |
| Class A2 | -0,06% | -0,95% | -0,96% | 103% | 117% |
| Class B | -0,09% | -1,44% | -1,73% | 98% | 107% |
| Class C | -0,07% | -2,73% | -2,48% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0,06% | -1,44% | -1,47% | 100% | 109% |
| Class D | -0,06% | -3,99% | -4,62% | 102% | 98% |
| Class F | -0,04% | -1,18% | -1,29% | 97% | 96% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,12% | -2,37% | -2,58% | 102% | 112% |
| Class C | -0,11% | -2,72% | -2,87% | 95% | 98% |
| Class E | -0,08% | -0,60% | 0,88% | 99% | 100% |
| Overall | -0,10% | -2,04% | -1,81% | 99% | 104% |
| Class D | -0,14% | -3,45% | -4,85% | 100% | 105% |
| Class F | -0,13% | -0,60% | -1,17% | 93% | 96% |

## Proposed 6-tap chroma IF for CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,18% | -0,08% | 103% | 108% |
| Class A2 | -0,05% | -0,92% | -0,84% | 103% | 118% |
| Class B | -0,08% | -1,24% | -1,32% | 99% | 104% |
| Class C | -0,04% | -2,30% | -2,04% | 105% | 112% |
| Class E |  |  |  |  |  |
| Overall | -0,05% | -1,25% | -1,17% | 102% | 110% |
| Class D | -0,04% | -3,47% | -3,78% | 107% | 104% |
| Class F | -0,03% | -0,99% | -1,02% | 98% | 94% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,13% | -1,84% | -2,02% | 102% | 103% |
| Class C | -0,10% | -2,04% | -2,61% | 102% | 105% |
| Class E | -0,05% | 0,35% | 0,94% | 105% | 107% |
| Overall | -0,10% | -1,36% | -1,48% | 103% | 105% |
| Class D | -0,16% | -3,08% | -4,03% | 109% | 113% |
| Class F | -0,08% | -0,08% | -1,18% | 100% | 102% |

# Conclusions

It has been shown that increasing the chroma interpolation filter length to 8-tap can maintain most of the gains of 12-tap but that 6-tap performs a bit worse than other increased filter lengths. It is suggested to include one of the proposed 6-tap and 8-tap chroma interpolation filter sets into the next version of ECM.

# References

[1] X. Xie, K. Zhang, L. Zhang, J. Li, M. Wang, S. Wang, “Non-EE2: Long tap interpolation filtering on chroma components”, JVET-Y0172, Meeting by Teleconference, January 2022

[2] V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Meeting by Teleconference, January 2022

[3] ECM-4.0 reference software,  https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Meeting by Teleconference, January 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).