| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0072-v1 |
| --- | --- |

| Title: | AHG10/AHG12: Enhanced reference picture structures for ECM and VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Sjöberg, R. Yu, L. Litwic | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution suggests a modified reference picture lists setting for ECM/VTM random access to store more pictures in the allowed Decoded Picture Buffer (DPB). The proposal also includes a change to the QP value for two pictures in the GOP. This is claimed to be almost identical to the settings proposed for VTM in JVET-Y0126 with the addition to order active entries in L0 and L1 such that pictures closer to the current picture in terms of POC are earlier in the lists.

This contribution also suggests a modified reference picture lists setting for ECM/VTM low-delay B configuration. Comparing to the current ECM/VTM LDB common test configuration, the proposal stores more pictures in the Decoded Picture Buffer (DPB) but maintains the same number of active reference pictures in L0 and L1. The proposal also suggests an adjustment of the QP offset settings for every second picture in the GOP.

The objective benefit in terms of BDR is reported as follows:

RA:   VTM: -0.52%	Y/-0.56%U/-0.55%V     ECM: -0.43%Y/-0.68%U/-0.64%V

LDB: VTM:  -1.63%Y/-3.23%U/-2.87%V    ECM: -1.62%Y/-2.76%U/-2.94%V

We propose to update the CTC with the proposed changes.

# Introduction

For levels other than level 15.5, the VVC specification supports usage of 7 pictures in the DPB plus the current picture (since maxDpbPicBuf is equal to 8). By increasing the number of active pictures, we found it possible to get some BDR improvements, but it heavily impacted the encoding time. We also did not see any substantial BD-rate gains by removing duplicates in the reference picture lists as was suggested in JVET-L0113 [5]. However, by further experimentation we identified some additional picture references that provides gains with limited complexity increase, and that removing duplicates can be used to reduce the complexity without significant BD-rate losses.

For low-delay B, by avoiding duplication of reference pictures in L1 that already exist in L0 and instead make use of some additional reference pictures, we found some good gains. To reduce the encoding time, which can be a burden especially for lower QP values, we also propose an increase of the QP values by 1 for every second picture.

# Proposed settings for random access

It is suggested to modify the random access configuration [1][2] as follows, the modifications are high-lighted in yellow for VTM, same applies to ECM. The change is almost identical to the changes proposed in JVET-Y0126 [6] besides a cleanup of POC ordering of active reference pictures from close to more distant:

#        Type POC QPoffset QPOffsetModelOff QPOffsetModelScale CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 CbTcOffsetDiv2 CbBetaOffsetDiv2 CrTcOffsetDiv2 CrBetaOffsetDiv2                                                       #                                                                                                    #                                                                                    temporal_id  active tot         L0                               active        tot          L1

Frame1  : B   32  -1   0.0     0.0     0  0  1.0  1  0  1  0  1  0  0            2  5       32 64 48 40 36            1  2        32 48

Frame2  : B   16   0  -4.9309  0.2265  0  0  1.0  2  0  2  0  2  0  1      3  5       16 32 48 24 20            1  1       -16

Frame3  : B    8   1  -4.5000  0.1900  0  0  1.0  3  0  3  0  3  0  2       4  5        8 24 16 40 12             2  2        -8 -24

Frame4  : B    4   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3       3  3        4  8 20                        3  3        -4 -12 -28

Frame5  : B    2   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4       3  3        2  6 18                        4  4        -2 -6 -14 -30

Frame6  : B    1   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5       1  1        1                                 2  5        -1 -3 -7 -15 -31

Frame7  : B    3   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5       2  2        1  3                             2  4        -1 -5 -13 -29

Frame8  : B    6   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4       3  3        2  4  6                         3  3        -2 -10 -26

Frame9  : B    5   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5       2  2        1  5                             2  4        -1 -3 -11 -27

Frame10 : B    7   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5      2  3        1  3  7                         2  3        -1 -9 -25

Frame11 : B   12   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3     3  4        4  8 12  6                    2  2        -4 -20

Frame12 : B   10   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4     4  4        2  4  6 10                    3  3        -2 -6 -22

Frame13 : B    9   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5     2  3         1  5  9                         2  4        -1 -3 -7 -23

Frame14 : B   11   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3         1  3 11                        2  3        -1 -5 -21

Frame15 : B   14   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    4  4         2  4  6 14                    2  2        -2 -18

Frame16 : B   13   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3         1  5 13                        2  3        -1 -3 -19

Frame17 : B   15   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4         1  3  7 15                    2  2        -1 -17

Frame18 : B   24   1  -4.5000  0.1900  0  0  1.0  3  0  3  0  3  0  2    3  3         8 16 24                       1  1        -8

Frame19 : B   20   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3    3  3        4 12 20                        2  2        -4 -12

Frame20 : B   18   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    3  3        2 10 18                        3  3        -2 -6 -14

Frame21 : B   17   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3        1  9 17                         2  4        -1 -3 -7 -15

Frame22 : B   19   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3        1  3 19                         2  3        -1 -5 -13

Frame23 : B   22   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    3  3        2  6 22                         3  3        -2 -10 4

Frame24 : B   21   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3        1  5 21                         2  3        -1 -3 -11

Frame25 : B   23   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  3  7 23                     2  2        -1 -9

Frame26 : B   28   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3    4  4        4  8 12 28                    1  1        -4

Frame27 : B   26   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    4  4        2  6 10 26                    2  2        -2 -6

Frame28 : B   25   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  5  9 25                     2  3        -1 -3 -7

Frame29 : B   27   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  3 11 27                    2  2        -1 -5

Frame30 : B   30   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    4  4        2  6 14 30                    1  1        -2

Frame31 : B   29   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  5 13 29                    2  2        -1 -3

Frame32 : B   31   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  5        1  3  7 15 31                1  1        -1

The changes can be summarized as adding some additional references for several pictures, removing duplicates for some pictures and thereby removing some references and also putting some references closer to the current picture. In addition to that we also propose to increase the QP offset for POC 8 and POC 24 by 1.

# Proposed settings for low-delay B

It is suggested to modify the ECM/VTM low-delay B configuration [1][2] as follows, the modifications are highlighted in yellow:

#        Type POC QPoffset QPOffsetModelOff QPOffsetModelScale CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 CbTcOffsetDiv2 CbBetaOffsetDiv2 CrTcOffsetDiv2 CrBetaOffsetDiv2                                                       #                                                                                                    #                                                                                    temporal_id  active tot         L0                               active        tot          L1

Frame1:  B    1   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 9 17 25            4                   4      1 3 5 33

Frame2:  B    2   4       -6.5  0.2590         0  0  1.0  0  0  0  0   0  0  0   4                4         1 2 10 18            4                   4      1 2 4 26

Frame3:  B    3   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 3 11 19            4                   4      1 3 5 27

Frame4:  B    4   4       -6.5  0.2590         0  0  1.0  0  0  0  0   0  0  0   4                4         1 4 12 20            4                   4      1 2 4 28

Frame5:  B    5   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 5 13 21            4                   4      1 3 5 29

Frame6:  B    6   4       -6.5  0.2590         0  0  1.0  0  0  0  0   0  0  0   4                4         1 6 14 22            4                   4      1 2 6 30

Frame7:  B    7   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 7 15 23            4                   4      1 3 7 31

Frame8:  B    8   1        0.0   0.0               0  0  1.0  0  0  0  0   0  0  0   4                4         1 8 16 24           4                   4      1 2 4 32

The changes can be summarized as adding two additional reference pictures instead of two of the duplicates from L0 in L1. The number of active reference pictures is unchanged. We also propose to increase the QP offset by 1 and to adjust the QP offset scale parameter for the pictures with odd POC values.

# Results

We provide results for ECM CTC [3] and VTM CTC [4]. A significant improvement to the objective performance can be noted. The encoding time numbers provided are not accurate.

We have also verified that the subjective quality is maintained.

## ECM

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.38% | -0.41% | 105% | 99% |
| Class A2 | -0.19% | -0.50% | -0.50% | 103% | 102% |
| Class B | -0.66% | -0.90% | -0.77% | 108% | 102% |
| Class C | -0.54% | -0.77% | -0.74% | 105% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | -0.68% | -0.64% | 105% | 102% |
| Class D | -0.20% | -0.35% | -0.41% | 107% | 101% |
| Class F | -0.40% | -0.48% | -0.46% | 105% | 99% |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.62% | -1.99% | -2.02% | 99% | 97% |
| Class C | -2.77% | -3.86% | -3.79% | 97% | 96% |
| Class E | -1.76% | -2.57% | -3.36% | 99% | 92% |
| Overall | -1.62% | -2.76% | -2.94% | 98% | 95% |
| Class D | -1.27% | -2.19% | -2.00% | 99% | 96% |
| Class F | -2.47% | -2.69% | -2.68% | 95% | 94% |

## VTM

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.17% | -0.26% | 106% | 101% |
| Class A2 | -0.34% | -0.49% | -0.50% |  |  |
| Class B | -0.64% | -0.73% | -0.68% | 105% | 100% |
| Class C | -0.68% | -0.68% | -0.63% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.56% | -0.55% |  |  |
| Class D | -0.43% | -0.34% | -0.31% | 103% | 98% |
| Class F | -0.37% | -0.34% | -0.31% | 103% | 101% |

|  | Low-delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.73% | -2.46% | -2.05% |  |  |
| Class C | -2.51% | -3.90% | -3.79% | 100% | 97% |
| Class E | -1.96% | -3.62% | -3.01% | 96% | 98% |
| Overall | -1.63% | -3.23% | -2.87% |  |  |
| Class D | -1.98% | -3.84% | -3.78% | 97% | 96% |
| Class F | -2.74% | -3.44% | -3.36% |  |  |

# Conclusions

We have shown that there is some benefit to use more references in the random access and low-delay B configuration and it seems to be justified in terms of additional complexity.

We therefore suggest updating the CTC for VTM and ECM with these changes.

# References

[1] ECM-4.0 reference software,  https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[2] VTM-16.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Meeting by Teleconference, January 2022

[4] F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-Y2010, Meeting by Teleconference, January 2022

[5] F. Hendry, Y-K Wang, J. Chen, “On final reference picture lists in the CTC random access simulation”, JVET-L0113, Macao, China, Oct 2018

[6] D.Rusanovskyy, M.Karczewicz (Qualcomm), K.Andersson, R.Sjöberg, L.Litwic (Ericsson), P.Nikitin, G.Martin-Cocher (InterDigital), A.Wieckowski, J.Brandenburg, B.Bross (HHI), “AHG10: VTM encoder configurations for tests targeting improved coding performance”, JVET-Y0126, Meeting by Teleconference, January 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).