| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20 – 29 April. 2022 | Document: JVET-Z0073-v3 |
| --- | --- |

| Title: | EE1-1.5: Test on NN-based filter as proposed in JVET-Y0052 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Dan Zou Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn zoudan@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents the EE results of JVET-Y0052 [1]. It mainly includes the test results of the CNN-based post-processing filter proposed in JVET-Y0052 [1] and some ablation studies. The ablation studies include comparing the contributions of standard convolution and depthwise separable convolution to the proposed filter as well as evaluating the performance of the proposed filter as an in-loop filter in VTM. The ablation experiments reveal that depthwise separable convolution can save nearly four times the number of parameters with comparable performance under the same configuration compared to standard convolution.

Introduction

JVET-Y0052 [1] proposes a CNN post-processing filter, called weakly connected dense attention neural network (WCDANN). In this contribution, several ablation studies are conducted to verify the performance of WCDANN. Firstly, the depthwise separable convolution in WCDANN is replaced with the standard convolution to verify the performance of both. Then, this contribution evaluates the performance of the WCDANN as a loop filter under various embedding configurations to confirm the optimal VTM embedding. Finally, we update the CTC test results of Class A1, A2, B, E based on JVET-Y0052 [1].

Proposed method

In JVET-Y0052 [1], we report a CNN post-processing filter based on depthwise separable convolution and attention mechanism, named WCDANN. As shown in Fig. 1, WCDANN is composed of two basic modules WCDAB and RAB to extract the important residual features from the input image. WCDANN includes residual connections in each module to promote the circulation of residual information, while using two attention modules CAB and CSAB to enhance the important residual features in the output of RAB and WCDAB, respectively. Moreover, WCDAB and RAB are shown in Figs. 2 and 3. In RAB, depthwise separable convolution is used as the basic convolution unit to greatly reduce the amount of model parameters and generate a lightweight model.

Fig. 1 Architecture of WCDANN. WCDANN consists of M WCDABs. “3ⅹ3 Conv” denotes 3ⅹ3 common convolution layer. ReLU denotes Rectified Linear Unit for activation. Each Conv has 64 channels. WCDABM denotes the M-th weakly connect dense attention block.

Fig. 2 Architecture of WCDAB. WCDAB consists of M WCDABs. “1ⅹ1 Conv” denotes 1ⅹ1 common convolution layer. RABi denotes i-th residual attention block (i=1, …,4). CSAB denotes channel-spatial joint attention block.

Fig. 3 Architecture of RAB.

Experimental Setting

Post-Processing filter (Test 1.5.1)

Based on JVET-Y0052, we keep the model fixed and test classes A1, A2, B, E using WCDANN as a post-processing filter using the same test environment, and the results are reported in Section 4.1.

Standard Convolution and Depthwise Separable Convolution (Test 1.5.2)

In this ablation experiment, all depthwise separable convolutions in WCDANN are replaced with standard convolutions and trained under the same setting. Specifically, we replace all depthwise separable convolutions in RAB and the other structures remain unchanged. The modified RAB are shown in Fig. 4.

Fig. 4 Architecture of modified RAB.

Embedding in VTM (Test 1.5.3)

Next, we explore the performance of WCDANN embedded in VTM. In the process, we conduct experiments by various embedding methods and compare the corresponding performance of each embedding method. Specifically, five embedding methods are used, and the test results are discussed in Section 4.3. After comparison, we draw a conclusion that the best embedding method is "LDSCA", i.e. LMCS-DBF-SAO-CNNLF-ALF, as shown in Fig. 5. It should be noted that the parameters of WCDANN remain the same, i.e. the proposed filter is not retrained.

Fig. 5 Pipeline of LDSCA. L: LMCS. D: DBF. S: SAO. C: CNNLF. A: ALF.

In all embedding methods, the slice level flag and the CTU flag are enabled, which means the proposed filter can be turned on/off at the CTU level and slice level. We found that turning off DBF and SAO for I slice can improve the performance, while for P and B slice, DBF and SAO need to be enabled. Therefore, apply this rule to the four embedding methods in Fig. 5.

Experiment Results

In this section, firstly, the test results of WCDANN on the CTC test sequence are supplemented, then we compare the performance of the standard convolution and the depthwise separable convolution as the basic convolution unit of WCDANN, and finally, we explore the performance of WCDANN as a loop filter under various embedding schemes and obtained the optimal scheme.

All ablation experiments are performed based on VTM-11.0-NNVC [2] according to the common test conditions (CTCs) defined in [4]. We perform the evaluation in the Random Access (RA), All Intra (AI) and Low Delay P (LDP) configurations.

Results of EE1-1.5.1

Based on JVET-Y0052, we supplement the test results on Class A1, A2, B, E when WCDANN is used as a post-processing filter as follows. Compared with VTM-11.0-NNVC, WCDANN achieves average 4.12%, 2.81% and 3.81% BD-rate reductions for Y channel on each test classes, under AI, RA and LDP configurations, respectively.

Table 1 BD rate of WCDANN-DSConv over VTM-11.0 NNVC in RA configuration.

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -2.23% | 0,00 % | 0,00 % | -1.78% | 0,00 % | 0,00 % | 0% |
| Class A2 | -2.70% | 0,00 % | 0,00 % | -2.09% | 0,00 % | 0,00 % | 0% |
| Class B | -2.73% | 0,00 % | 0,00 % | -2.68% | 0,00 % | 0,00 % | 0% |
| Class C | -3.43% | 0,00 % | 0,00 % | -2.64% | 0,00 % | 0,00 % | 0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -2.81% | 0,00 % | 0,00 % | -2.37% | 0,00 % | 0,00 % | 0% |
| Class D | -4.76% | 0,00 % | 0,00 % | -2.68% | 0,00 % | 0,00 % | 0% |

Table 2 BD rate of WCDANN-DSConv over VTM-11.0 NNVC in AI configuration.

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -2.98% | 0,00 % | 0,00 % | -2.85% | 0,00 % | 0,00 % | 0% |
| Class A2 | -3.19% | 0,00 % | 0,00 % | -3.71% | 0,00 % | 0,00 % | 0% |
| Class B | -3.48% | 0,00 % | 0,00 % | -3.68% | 0,00 % | 0,00 % | 0% |
| Class C | -4.58% | 0,00 % | 0,00 % | -3.99% | 0,00 % | 0,00 % | 0% |
| Class E | -6.66% | 0,00 % | 0,00 % | -7.85% | 0,00 % | 0,00 % | 0% |
| Overall | -4.12% | 0,00 % | 0,00 % | -4.31% | 0,00 % | 0,00 % | 0% |
| Class D | -4.97% | 0,00 % | 0,00 % | -3.70% | 0,00 % | 0,00 % | 0% |

Table 3 BD rate of WCDANN-DSConv over VTM-11.0 NNVC in LDP configuration.

|  | Low delay P Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -2.53% | 0,00 % | 0,00 % | -2.75% | 0,00 % | 0,00 % | 0% |
| Class A2 | -2.92% | 0,00 % | 0,00 % | -2.94% | 0,00 % | 0,00 % | 0% |
| Class B | -3.49% | 0,00 % | 0,00 % | -3.94% | 0,00 % | 0,00 % | 0% |
| Class C | -3.95% | 0,00 % | 0,00 % | -3.12% | 0,00 % | 0,00 % | 0% |
| Class E | -6.33% | 0,00 % | 0,00 % | -7.30% | 0,00 % | 0,00 % | 0% |
| Overall | -3.81% | 0,00 % | 0,00 % | -3.95% | 0,00 % | 0,00 % | 0% |
| Class D | -4.72% | 0,00 % | 0,00 % | -3.17% | 0,00 % | 0,00 % | 0% |

Results of EE1-1.5.2

We replace all depthwise separable convolutions in WCDANN with standard convolutions, and retrain the proposed network under the same setting, and at the same time, test the modified network under the same test conditions, and the results are shown below. For the convenience of distinction, we name the original WCDANN as WCDANN-DSConv, and the modified WCDANN as WCDANN-SConv. Compared with WCDANN-DSConv, WCDANN-SConv achieves average 0.42%, -0.20% and 0.12% BD-rate reductions for Y channel on each test classes, under AI, RA and LDP configurations, respectively. The results show that the depthwise separable convolution achieves similar performance to the standard convolution.

Table 4 BD rate of WCDANN-SConv over VTM-11.0 NNVC in RA configuration.

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -1.93% | 0,00 % | 0,00 % | -1.76% | 0,00 % | 0,00 % | 0% |
| Class A2 | -2.38% | 0,00 % | 0,00 % | -1.74% | 0,00 % | 0,00 % | 0% |
| Class B | -2.45% | 0,00 % | 0,00 % | -2.32% | 0,00 % | 0,00 % | 0% |
| Class C | -3.47% | 0,00 % | 0,00 % | -2.35% | 0,00 % | 0,00 % | 0% |
| Class E |  | 0,00 % | 0,00 % |  | 0,00 % | 0,00 % |  |
| Overall | -2.61% | 0,00 % | 0,00 % | -2.10% | 0,00 % | 0,00 % | 0% |
| Class D | -4.75% | 0,00 % | 0,00 % | -2.56% | 0,00 % | 0,00 % | 0% |

Table 5 BD rate of WCDANN- SConv over VTM-11.0 NNVC in AI configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -2.76% | 0,00 % | 0,00 % | -2.95% | 0,00 % | 0,00 % | 0% |
| Class A2 | -3.50% | 0,00 % | 0,00 % | -4.10% | 0,00 % | 0,00 % | 0% |
| Class B | -3.81% | 0,00 % | 0,00 % | -4.06% | 0,00 % | 0,00 % | 0% |
| Class C | -5.77% | 0,00 % | 0,00 % | -4.66% | 0,00 % | 0,00 % | 0% |
| Class E | -6.95% | 0,00 % | 0,00 % | -7.81% | 0,00 % | 0,00 % | 0% |
| Overall | -4.54% | 0,00 % | 0,00 % | -4.64% | 0,00 % | 0,00 % | 0% |
| Class D | -5.43% | 0,00 % | 0,00 % | -4.18% | 0,00 % | 0,00 % | 0% |

Table 6 BD rate of WCDANN- SConv over VTM-11.0 NNVC in LDP configuration.

|  | Low delay P Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -2.29% | 0,00 % | 0,00 % | -2.75% | 0,00 % | 0,00 % | 0% |
| Class A2 | -3.05% | 0,00 % | 0,00 % | -3.02% | 0,00 % | 0,00 % | 0% |
| Class B | -3.74% | 0,00 % | 0,00 % | -3.64% | 0,00 % | 0,00 % | 0% |
| Class C | -4.45% | 0,00 % | 0,00 % | -3.54% | 0,00 % | 0,00 % | 0% |
| Class E | -6.07% | 0,00 % | 0,00 % | -6.79% | 0,00 % | 0,00 % | 0% |
| Overall | -3.93% | 0,00 % | 0,00 % | -3.89% | 0,00 % | 0,00 % | 0% |
| Class D | -4.92% | 0,00 % | 0,00 % | -3.54% | 0,00 % | 0,00 % | 0% |

After comparing the overall performance, we tested the parameters, Flops, and running time (average running time per frame for video sequences in Class C) of the two networks. The results are shown in Table 7.

Table 7 Comparison of model complexity of WCDANN-DSConv and WCDANN-SConv

| Model | Model size |  | FLOPs（128*128） |  | Runtime (GPU) | Runtime (CPU) |
| --- | --- | --- | --- | --- | --- | --- |
| WCDANN-DSConv | WCDANNsmall | 0.43M | WCDANNsmall | 6.55G | 0.0036s | 4.67s |
|  | WCDANNlarge | 0.79M | WCDANNlarge | 11.88G | 0.0064s | 9.12s |
| WCDANN-SConv | WCDANNsmall | 1.59M | WCDANNsmall | 25.50G | 0.0035s | 4.37s |
|  | WCDANNlarge | 3.10M | WCDANNlarge | 49.78G | 0.0062s | 8.58s |

It can be seen from the comparison that WCDANN-DSConv and WCDANN-SConv have almost the same running time, however, WCDANN-DSConv saves almost three times the number of parameters.

Results of EE1-1.5.3

We try five different embedding methods, which are named according to the position in the loop filter: LCDSA, LDCSA, LDSCA, LDSAC, and LCA. We verify the performance of the above five embedding methods on C classes in RA configurations, and the results are shown in Table 8.

Table 8 Performance comparison of different embedding methods

| Embedding methods | Y-PSNR | Y-MSIM |
| --- | --- | --- |
| LCDSA | -2.54% | -2.31% |
| LDCSA | -2.81% | -2.62% |
| LDSCA | -2.78% | -2.61% |
| LDSAC | -2.67% | -2.78% |
| LCA | -2.24% | -2.00% |

As can be seen from Table 8, compared with LCDSA, LDCSA has higher performance, which shows that DBF behind WCDANN degrades performance. The performances of LDCSA and LDSCA are similar, indicating that SAO is hardly activated during the process. Compared with LDSCA, the performance of LDSAC is degraded, which shows that ALF has a great effect on the performance improvement. Finally, we try to use WCDANN to replace DBF and SAO, i.e. LCA. The LCA result does not perform well, which shows that WCDANN is not suitable for replacing DBF and SAO, and confirms that WCDANN is not suitable for processing the signal before DBF.

The optimal schemes are LDCSA and LDSCA. Since the post-processing filters are all enabled when we compress the training set, we think that the proposed filter may be better at processing the signal after SAO, thus we choose LDSCA as the final embedding method and test it on C and D classes in AI, RA, and LDP configurations. The test results are shown in Table 9, Table10 and Table 11. For the convenience of distinction, we refer to WCDANN working in the loop filter as WCDANN-Loop. Compared with VTM-11.0-NNVC, WCDANN-Loop achieves average 4.66%, 3.04% and 2.93% BD-rate reductions for Y channel on C and D classes, under AI, RA and LDP configurations, respectively.

Table 9 BD rate of CNNLF over VTM-11.0 NNVC in RA configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class C | -2.78% | 0,00 % | 0,00 % | -2,61 % | 0,00 % | 0,00 % | 0 % |
| Class D | -3.30% | 0,00 % | 0,00 % | -1.97 % | 0,00 % | 0,00 % | 0 % |

Table 10 BD rate of CNNLF over VTM-11.0 NNVC in AI configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class C | -4,62 % | 0,00 % | 0,00 % | -4.56 % | 0,00 % | 0,00 % | 0 % |
| Class D | -4.69% | 0,00 % | 0,00 % | -4.03 % | 0,00 % | 0,00 % | 0 % |

Table 11 BD rate of CNNLF over VTM-11.0 NNVC in LDP configuration.

|  | Low Delay P Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class C | -2.81 % | 0,00 % | 0,00 % | -2.68 % | 0,00 % | 0,00 % | 0 % |
| Class D | -3.05 % | 0,00 % | 0,00 % | -1.98 % | 0,00 % | 0,00 % | 1 % |

It can be seen from the results that in AI configuration, WCDANN achieves similar performance to the post-processing filter in the loop filter, which shows that WCDANN performs better at I-frames. However, in RA and LDP configurations, compared to the post-processing filter, the performance of WCDANN is degraded when working in the loop filter. It seems that in RA and LDP configurations, B-frames or P-frames predominate, while these frames are very different from I-frames and usually of low quality and have serious artifacts, thus when processing these frames, more auxiliary information, such as partition and prediction, needs to be used to help the filter. Therefore, in future work, we intend to combine auxiliary information to train WCDANN while expanding the input channels of the network to train in-loop filters for YUV three-channel video frame enhancement.

- Conclusions

This contribution provides some complementary experimental and ablation studies on JVET-Y0052. First, we supplement the test results of JVET-Y0052, and the new results demonstrate that WCDANN achieves average 4.12%, 2.81% and 3.81% BD-rate reductions over VTM-11.0-NNVC for Y channel on A1, A2, B, C, D and E classes in AI, RA and LDP configurations, respectively. Second, we compare the performance of depthwise separable convolution with standard convolution, confirming the effectiveness of depthwise separable convolution. Finally, we compared different embedding schemes and determined the optimal embedding method (LDSCA).

- References

- H. Zhang, C. Jung, D. Zou and M. Li, “AHG11: CNN post-processing filter based on depthwise separable convolution and attention mechanism,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-Y0052, January 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- M. Santamaria, Y.-H. Lam, J. Lainema, F. Cricri, R. Ghaznavi-Youvalari, A. Zare, H. Zhang, H. R. Tavakoli, and M. Hannuksela, “AHG11: Content-adaptive neural network post-processing filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-W0057, April 2021.

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).