| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0074-v1 |
| --- | --- |

| Title: | AHG11: Neural Network Based Motion Compensation Enhancement for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma Ru-Ling Liao Yan Ye | Tel: Email: | {changyue.mcy, ruling.lrl, yan.ye } @alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes to enhance the motion compensated prediction of coding units with neural network. The proposed method is performed on all inter-coded coding units with a flag in CU level to control the proposed method on or off. Experimental results demonstrate that, compared with VTM-11.0-nnvc-1.0, the proposed method achieves 1.29%, 6.02%, and 5.53% BD-rate reductions for Y, U, and V components under RA configuration, respectively.

Introduction

In this contribution, a neural network based motion compensation enhancement method is proposed. The proposed method is performed on an inter-coded coding unit after it finishes motion compensation process.

The proposed method is initially presented in the proposal JVET-Y0090 [1] in JVET-Y meeting. Compared with JVET-Y0090, there are several modifications in this contribution:

- The proposed method is applied to both square and non-square inter coding units with a flag in CU level to control the proposed method on or off.

- The encoding complexity of the proposed method is largely decreased by conducting model inference process on fewer inter coding unit candidates during RDO process.

- The proposed method is applied to both luma and chroma components of the coding unit.

Proposed Method

Overview

This contribution proposes a neural network based motion compensation enhancement method. The proposed method is applied to both square and non-square inter coding units with a flag in CU level to control the proposed method on or off. There are 6 contexts when encoding the flag into bitstream, 3 for AMVP mode and 3 for merge mode. For the 3 contexts in AMVP or merge mode, one is corresponding to the condition when the flags of left and up neighboring coding units of current coding unit are both false, one is corresponding to the condition when there is one flag of left and up neighboring coding units of current coding unit is true, one is corresponding to the condition when the flags of left and up neighboring coding units of current coding unit are both true. When applying the proposed method to non-square inter coding units, the coding block with shape w×h shares the same neural network model with the coding block with shape h×w through transposing. Considering the bit overheads of small coding units are large, only partial coding unit shapes are enabled in the proposed method, which is depending on the test sequence resolution. Specifically, for Class A1, A2, only the coding units with both height and width are greater than 16 are enabled; for Class B, C, only the coding units with both height and width are greater than 8 are enabled; for Class D, only the coding units with both height and width are greater than 4 are enabled.

When implementing the proposed method into VTM-11.0-nnvc-1.0 platform, to decrease the encoding complexity without sacrificing too much coding performance, the neural network inference process of the proposed method is conducted on the final one inter coding unit candidate after comparing all the AMVP and merge coding unit candidates in CU level during RDO process.

The proposed method is applied to both luma and chroma components of the coding unit, where the luma and chroma components both have their own neural network models. When the proposed method is applied to luma, the luma component of the motion compensated prediction is used as neural network input. When the proposed method is applied to chroma, both the luma and chroma components of the motion compensated prediction are used as neural network input. The luma and chroma components of the coding unit are controlled together with a shared flag in CU level.

Neural Network Architecture

Fig. 1 presents the structure of the proposed motion compensation enhancement neural network, where Fig. 1(a) and Fig. 1(b) present the overall network structure of the luma and chroma components, Fig. 1(c) presents the network structure of the residual unit. The proposed network of luma component takes the luma component of the motion compensated prediction as input. The proposed network of chroma components takes both the luma and chroma components of the motion compensated prediction as input, where the luma component is sub-sampled to 4 parts to have the same resolution with chroma components. The prediction is first fed into a convolutional layer to extract the feature maps. After that, the feature maps pass through 10 residual units [2] and a convolutional layer. Finally, the neural network output is obtained by adding the input prediction and the learned residual information.

For all the convolutional layers in Fig. 1, kernel size of 3x3 is used. The number of the feature maps in each convolutional layer except the last layer is set to 64. The last convolutional layer produces 1 feature map for the neural network of luma component, 2 feature maps for the neural network of chroma components. For activation function, ReLU is used.

(a) Overall network structure of luma component              (b) Overall network structure of chroma components

(c) Network structure of residual unit

Figure 1. The structure of the proposed motion compensation enhancement neural network.

Neural Network Inference

In inference stage, PyTorch 1.7.1 is used for performing the proposed neural network based motion compensation enhancement in VTM-11.0-nnvc-1.0. The inference information is summarized in Table 1 as suggested by [3].

Table 1. Inference information.

| Inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | N/A |
|  | CPU Type | Intel(R) Xeon(R) CPU E5-2682 v4 @ 2.50GHz |
|  | Framework: | PyTorch 1.7.1 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 776.71k/model (luma), 780.16k/model (chroma) |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 3.03M/model (luma), 3.04M/model (chroma) |
|  | MACs | 783.1k/pixel (luma), 786.7k/pixel (chroma) |
| Optional | Total Conv. Layers | 22 |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 block at a time |
|  | Patch size | Depending on the block |

Neural Network Training

In training stage, PyTorch 1.7.0 is used for training the proposed neural network. For different coding unit sizes (128x128, 64x64, 32x32, 16x16, 8x8, 64x128/128x64, 32x64/64x32, 16x64/64x16, 16x32/32x16, 8x64/64x8, 8x32/32x8, 8x16/16x8), QPs (QP22, QP27, QP32, QP37, QP42) and components (Y, UV), the proposed network is trained independently. For luma component, there are in total 60 neural network models. For chroma components, as only square block is enabled currently, there are in total 25 neural network models. In total, there are 85 models. The BVI-DVC [4] dataset is used to train the neural network. The sequences are compressed by VTM-11.0-nnvc-1.0 with RA configurations. The training information is summarized in Table 2 as suggested by [3].

Table 2. Training information.

| Training information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | NVIDIA Tesla V100-SXM2-32GB |
|  | CPU Type | Intel(R) Xeon(R) Platinum 8163 CPU @ 2.50GHz |
|  | Framework: | PyTorch 1.7.0 |
|  | Number of GPUs per Task: | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Total Parameter Number: | 776.71k/model (luma), 780.16k/model (chroma) |
|  | Parameter Precision (Bits): | 32 (F) |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
| Optional | Patch size | Depending on the block |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | L2 |
|  | Preprocessing: | Normalize to 0~1 |

Experimental Results

The common test conditions defined in [3] are used to evaluate the proposed method. The experimental results on VTM-11.0-nnvc-1.0 under RA configuration are shown in Table 3. It is observed that 1.29%, 6.02%, and 5.53% BD-rate reductions are achieved for Y, U and V components under RA configuration, respectively.

Table 3. Experimental results on VTM-11.0-nnvc-1.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-nnv-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -3.97% | -3.47% | 277% | 18515% |
| Class A2 | -1.70% | -6.96% | -6.50% | 290% | 20645% |
| Class B | -1.32% | -6.68% | -6.53% | 454% | 21720% |
| Class C | -1.23% | -6.04% | -5.11% | 388% | 21564% |
| Overall | -1.29% | -6.02% | -5.53% | 361% | 20785% |
| Class D | -3.24% | -5.57% | -6.49% | 560% | 41393% |

Conclusions

This contribution presents a neural network based motion compensation enhancement method. Experimental results demonstrate that the proposed method achieves coding performance improvement compared with VTM-11.0-nnvc-1.0.

References

- C. Ma, R.-L. Liao, Y. Ye. AHG11: Neural Network Based Motion Compensation Enhancement for Video Coding. JVET-Y0090, 25th Meeting, by teleconference, 12–21 January 2022.

- Kaiming He, Xiangyu Zhang, Shaoqing Ren, and Jian Sun, “Deep residual learning for image recognition," in Proceedings of the IEEE conference on computer vision and pattern recognition, 2016, pp. 770-778.

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, 24th Meeting: by teleconference, 6 – 15 October 2021.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).