| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0084-v1 |
| --- | --- |

| Title: | EE2-3.4: IBC with Template Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert K. Naser T. Poirier Y. Chen F. Galpin | Email: | antoine.robert@interdigital.com karam.naser@interdigital.com tangi.poirier@interdigital.com ya.chen@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 test 3.4 that proposes to enable Template Matching with IBC merge and AMVP modes. For both modes, a distance is imposed between the input candidates that are refined with Template Matching. During the refinement process, performed at integer-pel precision so it accesses only to samples without interpolation, the IBC reference region constraint is checked at every step.

The reported simulation results of the proposed method on top of the ECM-4.0 in SCC test conditions show gains of {-0.60%, -0.38%, -0.43%}, {-0.46%, -0.52%, -0.45%} and {-0.36%, -0.20%, -0.43%} for YUV in AI, RA and LDB respectively with a limited increase in encoder runtime of 3% to 5% and an unchanged complexity of the decoder.

# Introduction

Template Matching has been introduced in the ECM [1] first for the regular merge and AMVP modes, then, more recently, in the GPM and CIIP merge modes and for adaptive reordering of predictor lists.

JVET-Y0088 [2] proposed to extend Template Matching to IBC.

# Proposal

It is proposed to use Template Matching with IBC for both IBC merge mode and IBC AMVP mode as described in the EE2 [3].

In this proposal, the IBC-TM merge list has been modified compared to the one used by regular IBC merge mode such that the candidates are selected according to a pruning method with a motion distance between the candidates as in the regular TM merge mode. The ending zero motion fulfillment (which is a nonsense regarding Intra coding) has been replaced by motion vectors to the left (-W, 0), top (0, -H) and top-left (-W, -H), where W is the width and H the height of the current CU.

In the IBC-TM merge mode, the selected candidates are refined with the Template Matching method prior to the RDO or decoding process. The IBC-TM merge mode has been put in competition with the regular IBC merge mode and a TM-merge flag is signaled.

In the IBC-TM AMVP mode, up to 3 candidates are selected from the IBC-TM merge list. Each of those 3 selected candidates are refined using the Template Matching method and sorted according to their resulting Template Matching cost. Only the 2 first ones are then considered in the motion estimation process as usual.

The Template Matching refinement for both IBC-TM merge and AMVP modes is quite simple since IBC motion vectors are constrained (i) to be integer and (ii) within a reference region as shown in Figure 1. So, in IBC-TM merge mode, all refinements are performed at integer precision, and in IBC-TM AMVP mode, they are performed either at integer or 4-pel precision depending on the AMVR value. Such a refinement accesses only to samples without interpolation. In both cases, the refined motion vectors and the used template in each refinement step must respect the constraint of the reference region.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Figure 1: IBC reference region depending on current CU position

# Simulation results

The proposed method has been tested on top of the ECM-4.0 software [4] under the CTC conditions [5] and in SCC configuration. The simulation results are depicted in Table 1.

Table 1: Proposed IBC-TM on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.65% | -0.25% | -0.37% | 105% | 102% |
| Class TGM | -0.56% | -0.52% | -0.48% | 105% | 103% |
| Overall | -0.60% | -0.38% | -0.43% | 105% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.43% | -0.67% | -0.51% | 104% | 100% |
| Class TGM | -0.49% | -0.37% | -0.38% | 104% | 102% |
| Overall | -0.46% | -0.52% | -0.45% | 104% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.32% | -0.08% | -0.57% | 103% | 100% |
| Class TGM | -0.39% | -0.32% | -0.28% | 102% | 101% |
| Overall | -0.36% | -0.20% | -0.43% | 103% | 101% |

# Conclusion

In this contribution, a simple Template Matching mode for IBC is proposed for both merge and AMVP modes. A coding efficiency of {-0.60%, -0.46%, -0.36%} for Y in AI, RA and LDB has been achieved on top of the ECM-4.0 with a limited increase in encoder runtime of 3% to 5%, but without any additional decoding time increase. We therefore suggest adopting this proposal in the next ECM release.

# References

- M. Coban, F. Le Léannec, K. Naser, J. Ström, “Algorithm description of Enhanced Compression Model 4 (ECM 4)”, JVET-Y2025, January 2022.

- A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin, “EE2-related: IBC with Template Matching”, JVET-Y0088, January. 2022.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, January 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).