| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0105-v2 |
| --- | --- |

| Title: | AHG12: Virtual boundary processing for the new In-Loop filter tools in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Nan Hu Vadim Seregin Marta Karczewicz | Tel: Email: | akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc., |  |  |

_____________________________

# Abstract

This contribution enables the virtual boundary (VB) processing for the new In-loop filters in ECM-4.0, which are Cross-component sample adaptive offset (CCSAO), Bilateral filter (BIF) and the improved ALF and CCALF.

The proposed method is evaluated on two software bases. The first software base being JVET-Y1063 [2], which proposed a new GDR implementation which internally uses the VB processing. However, JVET-Y0163 software disabled the new In-loop filter tools of ECM-3.0 as the virtual boundary processing is missing. This contribution implements the VB processing and reports the BD-Rate numbers on top of JVET-Y0163 GDR implementation. For the LDP configuration, the BD-Rate impact is as follows: x.xx%(Y), x.xx%(U), x.xx% (V).

Secondly, on top of ECM-4.0 common test condition, VB is tested by explicitly adding one horizontal virtual boundary and one vertical virtual boundary and the results in RA are as follows:
0.25%(Y), 0.16%(U), 0.17% (V).

The software for both the bases is included along with the contribution package.

# Introduction

VVC standard [1], supports disabling of in-loop filters across vertical and horizontal virtual boundaries to mainly remove undesired filtering artifacts when filtering is applied across discontinuous edges (e.g., 360° video can have several cases where the edges are discontinuous).

JVET-Y0163 [1] proposes a new GDR implementation for the ECM platform, This GDR implementation relies on the VB processing feature. However, this contribution turns off the new in-loop filters tools of ECM compared to VVC. Following are the tools which were turned off:

- ALF_IMPROVEMENT

- JVET_V0094_BILATERAL_FILTER

- JVET_X0071_CHROMA_BILATERAL_FILTER

- JVET_W0066_CCSAO

- JVET_X0071_LONGER_CCALF

- JVET_X0071_ALF_BAND_CLASSIFIER

The current contribution enables the VB processing for all the new in-loop filter tools.

# Proposal

For CCSAO, the VB processing is similar to SAO, wherein the filtering of a given sample is disabled if the sample falls on a virtual boundary or if the neighbor sample is outside the virtual boundary. It is to be noted that unlike the Band offset (BO) of SAO which purely relies on the current sample information to derive the band information, the BO of CCSAO can use any of the 8 spatial neighbor samples to derive the band information (as shown in Figure 1, below). Therefore, all the 8 neighboring samples and the current sample are checked for availability before deriving the band information.

Horizontal boundary

Vertical boundary

Figure1. Virtual Boundaries processing for CCSAO BO.

For the Luma bilateral filter and the Chroma bilateral filter, the existing padding mechanism (repetitive padding) which is used at the picture boundaries is also applied for the samples near virtual boundaries.

The ALF filter design in ECM uses a maximum filter footprint of 13 x 13 diamond shape filter. Therefore, the minimum padding size is increased to 6 samples. For improved CCALF, padding size of 4 samples is used. The padding mechanism for the virtual boundaries is kept the same as in VVC (which is repetitive padding)

# Simulation results

The proposed method is evaluated on two software bases: The first software base being JVET-Y1063 [2] which proposed a new GDR implementation which internally uses the VB processing.  The second software base used is ECM-4.0

## BD-Rate results with JVET-Y1063 GDR implementation (with new In-loop filters turned off) as Anchor

VB processing for the new in-loop filters of ECM is enabled on top of JVET-Y0163 [2] and the simulation is performed under common test conditions as described in JVET-X2017 [6] (It is to be noted that JVET-Y0163 uses ECM-3.0 as software base).

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-Y0163 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1.93% | -10.00% | -8.65% | 119% | 129% |
| Class E | -3.76% | -10.07% | -12.94% | 123% | 142% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.62% | -8.88% | -6.81% | 117% | 122% |
| Class F | -2.04% | -10.32% | -9.29% | 135% | 131% |
| Class TGM | -3.54% | -10.03% | -9.22% | 155% | 137% |

Reportedly, most of the BD-Rate gain of the new in-loop filter tools is retained by enabling the VB processing over JVET-Y0163 Anchor. The EncT and DecT are not reliable as the Anchor and Test were run in different points in time.

## BD-Rate results with ECM-4.0 Anchor

VB processing for the new in-loop filters of ECM is enabled on top of ECM-4.0 [4] and the simulation is performed under common test conditions as described in JVET-Y2017 [7].

The implementation is tested by explicitly adding one vertical virtual boundary (pos: 64) and one horizontal virtual boundary (pos: 64) for all the classes.

The BD-Rate impact is as follows:

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.19% |  | 0.00% |  | 0.28% |  | 104% |  | 104% |  |
| Class A2 | 0.26% |  | 0.14% |  | 0.07% |  | 103% |  | 102% |  |
| Class B | 0.19% |  | 0.24% |  | 0.19% |  | 102% |  | 103% |  |
| Class C | 0.37% |  | 0.20% |  | 0.15% |  | 101% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.25% |  | 0.16% |  | 0.17% |  | 102% |  | 102% |  |
| Class D | 0.41% |  | 0.24% |  | 0.29% |  | 99% |  | 99% |  |
| Class F | 0.23% |  | 0.00% |  | 0.09% |  | 101% |  | 101% |  |
| Class TGM | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.27% | 0.48% | 0.18% | 98% | 99% |
| Class E | 0.11% | -0.24% | 0.69% | 100% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.39% | 0.66% | 1.17% | 101% | 100% |
| Class F | 0.23% | 1.37% | -0.35% | 100% | 100% |
| Class TGM | 0.12% | 0.26% | 0.23% | 101% | 100% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | 1.02% | 0.60% | 101% | 102% |
| Class C | 0.41% | 0.59% | 0.48% | 99% | 98% |
| Class E | 0.10% | 0.98% | 0.84% | 104% | 107% |
| Overall | 0.23% | 0.87% | 0.62% | 101% | 102% |
| Class D | 0.45% | 0.87% | 1.06% | 97% | 97% |
| Class F | 0.33% | 0.22% | 0.65% | 103% | 104% |
| Class TGM | 0.06% | 0.21% | 0.09% | 106% | 104% |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.18% |  | 0.22% |  | 0.16% |  | 102% |  | 104% |  |
| Class A2 | 0.22% |  | 0.12% |  | 0.14% |  | 101% |  | 101% |  |
| Class B | 0.19% |  | 0.29% |  | 0.25% |  | 101% |  | 105% |  |
| Class C | 0.57% |  | 0.28% |  | 0.32% |  | 101% |  | 101% |  |
| Class E | 0.52% |  | 0.40% |  | 0.28% |  | 100% |  | 102% |  |
| Overall | 0.33% |  | 0.26% |  | 0.24% |  | 101% |  | 103% |  |
| Class D | 0.78% |  | 0.33% |  | 0.40% |  | 100% |  | 98% |  |
| Class F | 0.36% |  | 0.27% |  | 0.30% |  | 103% |  | 104% |  |
| Class TGM | 0.09% |  | 0.00% |  | 0.03% |  | 102% |  | 105% |  |

# Conclusion

Virtual boundary processing for the new in-loop filter tools of ECM is enabled. It is reported that with the software base as JVET-Y0163, the proposed VB processing captures most of the in-loop filter gain. The BD-Rate impact on top of ECM-4.0 as Anchor is also reported. The test is conducted by explicitly adding virtual boundaries at certain pre-defined positions.

It is suggested to adopt the proposed VB processing method for the new in-loop filter tools of ECM into next version of ECM.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, Jun. 2020, Teleconference

- L. Wang et al. AHG7: GDR without encoder constraints for ECM, JVET-Y0163, January 2022, Teleconference

- ECM-3.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.0

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0

- C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “EE2-5.1: Cross-component Sample Adaptive offset”, JVET-W0066, July 2021, Teleconference

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, November 2021, Teleconference.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January 2022, Teleconference.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).