| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0115-v1 |
| --- | --- |

| Title: | AHG13: proposed film grain synthesis reference model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban E. François | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document describes a film grain synthesis model that supports both modes of FGS SEI (frequency-filtering and autoregressive), as well as other parameter formats, with attention paid to hardware complexity. It is proposed to study this model in ad-hoc group on film grain technologies, as a candidate for the reference model of a potential “soft conformance” definition.

# Introduction

The possibility of (soft-)conformance of film grain synthesis with a reference model was discussed within ad-hoc group 13 on film grain technologies. It is desirable that this model supports all variants of film grain parameters from FGS SEI (both frequency-filtering model and autoregressive model) [1], including support of multiple film grain patterns within a picture, while taking great care of hardware implementation complexity.

Support of film grain parameters in the form of ITU-T T.35 user metadata registered by AOM [2] (and used in the AV1 specification [3]) is of additional interest, as well as comparative implementation complexity: since some chip vendors may have implemented film grain in accordance with the specifications of AOM, this document investigates if it is possible to leverage those implementations to synthesize film grain based on FGS SEI, or what would be the incremental complexity.

Details on those film grain parameters, existing implementations, and parameter estimation methods are given in a technical report being drafted under the umbrella of AHG13 [4].

This document proposes a reference model that supports all film grain parameter variants cited above, while controlling hardware complexity by restricting the number of grain patterns, avoiding modulo operation and any kind of line buffer.

# FGS SEI features, implementation architecture, complexity issues

FGS SEI defines film grain characteristics as either bandwidth-limited noise (hereafter referred to as “frequency-filtering mode”, or FF mode) or noise filtered by autoregressive coefficients (hereafter referred to as “AR mode”). Also, to adapt film grain to target picture, FGS SEI defines a set of intensity intervals (per color component), with potentially different grain amplitude scaling factors and/or different characteristics.

## Local adaptation of grain characteristics

A major functional difference with AOM registered metadata is that the latter supports only uniform film grain characteristics within a picture: it adapts grain amplitude, but not grain “shape”. To determine if an AOM-based implementation would be suitable, the question is then if film grain shape truly varies within a picture, and whether it is an essential feature.

Figure 1 illustrates an analysis of one luma frame from the CrowdRun sequence, which was shot on 65mm Kodak Vision 250D color film and scanned with 1 pixel covering about 17µm. For each image patch in this figure, original is on the left, followed by a denoised version, then the difference (x4 to be more visible) which is the film grain, and finally an FFT cloud illustrating the spatial frequency distribution of the grain. It appears that both amplitude and sharpness (or size) of film grain clearly depend on exposure: stronger and sharper/finer on light areas, lighter and smoother/coarser on dark areas. Thus, it can be concluded that, if the film look is to be recreated (artistic intent), then ability to vary the film grain characteristics within a picture is an essential feature.

Figure 1: film grain dependency on exposure [original / denoised / diff / FFT]

## Implementation architecture overview

A detailed bit-exact implementation of the FF mode has been described in SMPTE RDD-5 [5], with some restrictions on FGS SEI syntax element values. The general architecture of this implementation is common to all implementations considered here, and is described below.

Figure 2: general film grain synthesis architecture

First, a set of film grain patterns (64x64 patches) is retrieved or generated. This only happens upon configuration change, and at most once per picture.

Then, the picture is split into blocks (16x16 for RDD5), and for each block, a random location is picked from the 64x64 patches. This allows making a full picture of film grain from a small patch, while still looking random. This step is called “randomization” in the following.

Then local adaptation is performed: grain pattern and scale are selected based on the local intensity of the pixels.

Finally, deblocking is applied to smooth transitions (mostly randomization blocks), and the result is blended to the target picture.

Table 1 lists implementation differences between FGS SEI (RDD5) and AOM-based implementations (referred to as just “AOM” in the following). Some of these differences are discussed in the following sections, that detail the steps described above and the decisions taken for the proposed model.

Table 1 – FGS SEI vs AOM differences

| Feature | SEI / RDD5 | AOM |
| --- | --- | --- |
| Master pattern generation (init, firmware) | Frequency-based (gaussian, inverse DCT), or AR (6 symmetric coefficients, half-diamond shape) | AR (up to 25 coefficients, rectangle-shaped) |
| Master patterns (on-chip memory) | 64x64, up to 10 + 10 | 64x64 luma + 2x 32x32 chroma (4:2:0 mode) or 64x64 (4:4:4 mode) |
| Pattern selection | Intensity-based (8x8 block average). Per-color. | Fixed: per color plane |
| Pseudo-noise generator | 31-bit (32-bit register) | 16-bit |
| Randomization | Pseudo-random x/y offset within pattern, every 16x16 block + flip sign | Pseudo-random x/y offset within pattern, every 32x32 block (or 16x16 for chroma) |
| Scaling | Intensity-based (8x8 block average), stepwise function. Per-color. | Luma or color-mix based, (pixel-based), piecewise-linear function. |
| Deblocking | 8x8, mix of pre-computed attenuation (horizontal edges) and live filtering (vertical edges, ¼ ½ ¼) | 2-pixel overlap (right & bottom) = live filtering, per 32x32 block. Non-trivial coefficients (17/27 or 22/23) |
| Clipping | Fixed (full range) | Flexible (full/legal) |

## Pattern generation

As grain patterns are generated only once per picture, complexity is not critical since this can be performed in firmware. Supporting several types of film grain parameters is not considered to be an issue. A degree of flexibility is desirable at this stage, potentially considering other types of grain pattern generation (e.g. actual scans), mapping between parameter variants (e.g. translating frequency-filtering to autoregressive), etc.

A user application could have a small number of presets, matching some known film stocks, that could be described in different parameter formats (FGS SEI, etc) depending on the context.

In an FGS SEI, it is recommended that model parameters are transmitted by order of importance, so that the firmware can adapt the number of patterns to what the hardware supports by just retaining the first ones. The patterns are then sorted in order of increasing intensity, to be indexed by the pattern LUTs and to enable interpolation (see below).

In the proposed model, the patterns are correlated, so that pixel-wise local adaptation is possible (see section 2.5).

- For frequency-filtering model, a common random pattern is zeroed-out above cut-off frequencies before iDCT.

- For autoregressive models, a common random pattern is filtered by the various sets of coefficients

The output of the pattern generation (or configuration decoding) step is:

- A set of 8 64x64 patterns for luma, and a set of 8 patterns for chroma (32x32 in 4:2:0 mode).

- Scaling LUTs (one for each channel)

- Pattern selection LUTs (one for each channel)

Those are potentially stored in high-speed on-chip memory, so their size has a hardware cost implication. More specifically, the number of patterns has the greatest impact. RDD5 specified a maximum of 10+10 patterns, but 8 is considered sufficient for the proposed reference model. Reducing that number to 4 luma + 4 chroma could be considered if interpolation is implemented in the local adaptation step (see below).

The next steps are potentially performed in hardware.

## Randomization

As said above, the randomization step consists in taking small blocks at random locations in the 64x64 patterns to extend them to the full picture. The size of block and the random generator differ between RDD5 and AOM specification. As shown in Figure 3, AOM randomization appears too limited, with too much apparent repetitions (that looks worse in color, and not better in video mode).

The limitations of AOM randomization come from the 32x32 block size, which is too close to the 64x64 pattern size: the probability of overlapping random locations is too high. Also the 16-bit random generator of AOM may not shuffle locations as efficiently as the 32-bit RDD5 generator does, and RDD5 has a random sign flip in addition to random offsets, which increases diversity but is not present in AOM.

same pattern

RDD5

AOM

Figure 3: RDD5 vs AOM randomization (640x480 grain from a 64x64 pattern)

The proposed model uses:

- 16x16 blocks (for luma; 8x8 for chroma in 4:2:0 mode)

- the RDD5 random generator (a single XOR + 32-bit register)

- random offset derivation that avoids modulo (which is too costly), by performing multiplication (by simple factors: 3 or 13) + mask instead

  - offsets are restricted to multiples of 2 or 4 to help lowering the cost of pattern memory

- random sign flip as in RDD5

## Local adaptation and multiple patterns

A first problem for local adaptation is the usage of a local average: RDD5 selects a scaling factor and a pattern based on an 8x8 average of current color component intensity. In hardware, that means a 7-line buffer, which can be too costly. On the other hand, AOM scaling is sample-based, which does not require line buffers; a 2x1 luma average is performed to support chroma scaling based on a luma/chroma intensity mix (also called “cross-correlation”), but since this is horizontal only, it does not require line buffer either.

A second problem is the support of pattern adaptation, which is absent from AOM, but required by FGS SEI. This is supported in RDD5, but again based on an 8x8 intensity average, which is undesirable.

The proposed model:

- Supports sample-based scaling, which causes no issue compared to block-based: is it actually better

- Supports chroma scaling based on a luma/chroma mix, as in AOM

- Supports sample-based pattern selection, which requires correlated patterns to avoid visible transitions, as described in section 2.3 and illustrated in Figure 4.

- Optionally supports pattern interpolation: the pattern LUT has an integer and a fractional part.

  - The integer part is the current pattern

  - The fractional part is an alpha value used to blend current and next pattern

target image

pattern selection

uncorrelated patterns

correlated patterns

Figure 4: sample-based pattern change between uncorrelated or correlated patterns

## Deblocking

When local adaptation is based on sub-block average, deblocking is needed for both randomization blocks and adaptation sub-blocks. Also, RDD5 uses a trick to avoid extra line buffers for vertical deblocking: it forces vertical random offset to be a multiple of 8, and pre-deblocks by attenuating two lines every 8 in the patterns themselves (at pattern generation stage); this method can create quite visible stripes. On the other hand, in AOM, deblocking is optional and performed by generating 34x34 grain blocks instead of 32x32 and overlapping 2 columns and/or lines between adjacent blocks; this does not require line buffers, but requires running two random generators and dual-read pattern memory (to merge two different random offsets), or even quad-read for the 2x2 corner where 4 blocks overlap, depending on implementation.

In the proposed model, since adaptation is sample-based, deblocking is needed for randomization blocks only (16x16).

- Vertical deblocking is performed by overlapping, as in AOM

- Horizontal deblocking is performed by [¼ ½ ¼] filtering, as in RDD5

# Conclusion

A film grain synthesis model is proposed, that support FGS SEI (both frequency and autoregressive modes) and AOM parameters, with controlled hardware complexity thanks to e.g. a limited number of patterns and no line buffers. It is proposed to study this model in AHG13 as a candidate for the reference model of a potential “soft conformance” definition.

# References

- Versatile supplemental enhancement information messages for coded video bitstreams, Recommendation ITU-T H.274 (08/2020) | International Standard ISO/IEC 23002-7

- AOM film grain synthesis specification working draft: https://aomediacodec.github.io/av1-FilmGrain

- AV1 Bitstream & Decoding Process Specification, The Alliance for Open Media, Version 1.0.0 with Errata 1, January 18, 2019.

- JVET-Y2020, "Film grain synthesis technology for video applications", Jan. 2022

- Film grain technology specifications for H.264 | MPEG-4 AVC bitstreams, SMPTE RDD 5-2006

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).