| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0120-v1 |
| --- | --- |

| Title: | AHG9: Shutter interval information SEI message for VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Fangjun Pu, Walt Husak, Peng Yin, Taoran Lu, Arjun Arora, Tong Shao Dolby Laboratories, Inc. Jeeva Raj Arumugam, Mallikarjun Kamble, Saurabh Agrawal, Kaustubh Patankar Ittiam Systems | Email: | {sean.mccarthy, pyin}@dolby.com {jeeva.raj, kaustubh.patankar} @ittiam.com |
| Source: | Dolby Laboratories, Inc. and Ittiam Systems |  |  |

_____________________________

# Abstract

This contribution proposes that the shutter interval information (SII) SEI message already specified in HEVC be adopted in the next version of VSEI. A functionally equivalent SII SEI message is also already specified in AVC. The SII SEI message indicates the shutter interval for the associated video source pictures prior to encoding, e.g., for camera-captured content, the shutter interval is the amount of time that an image sensor is exposed to produce each source picture. Shutter interval information can be useful for several use cases, for example: enabling backwards compatibility of high frame rate (HFR) bit streams with receivers that support only standard frame rate (SFR) bit streams such as specified in ETSI TS 101 154 and ATSC A/341; prediction of subjective quality of motion; frame rate conversion; and detection of non-camera-captured content such as screen captures and computer-generated content. Additional descriptions, methods, and showcase VTM software is provided to illustrate the use of the SII SEI message with backwards compatible HFR bit streams.

# Introduction

Shutter interval information (SII) SEI messages are specified in HEVC and AVC ([1][2]) but not yet in VVC and VSEI.  An SII SEI message previously proposed [3] for VSEI was deferred for consideration until after adoption of the HEVC version and finalization of VVC and VSEI version 1. This contribution proposes that the SII SEI message now be adopted into the next version of VSEI and corresponding text on use of the SII SEI message be added to the next version of VVC.

This contribution also provides showcase VTM software to illustrate the use of the SII SEI message with backwards-compatible HFR bit streams such as specified in the Digital Video Broadcasting (DVB) specification for the use of video and audio in broadcast and broadband applications (ETSI TS 101 145) [4].

## Overview of shutter interval

Shutter interval is the amount of time an image sensor or photographic film is exposed to light. Shutter interval is a key parameter as important as frame rate, camera aperture, and sample resolution that describes how content was created.

The shutter interval has a direct impact on the amount of motion blur in an image. The longer the shutter interval, the more motion blur. The shutter interval also has a direct impact on the amount of noise in a captured image. The shorter the shutter interval, the lower the signal to noise ratio will be. For computer-generated content, a shutter interval value equal to 0 can be used to indicate the complete absence of motion blur and photon noise, something impossible for physical cameras.

Shutter interval is an important indicator of the visibility of motion artifacts in video. Watson [5] provides a thorough quantitative analysis of the impact of shutter interval on the perceptibility of motion artifacts in the context of the end-to-end capture and display process. Others [6][7][8] have studied subjectively the effect of shutter interval on motion blur, judder, strobing and other temporal aliasing artefacts.

Shutter interval can be used to guide frame rate conversion, which is a use case that is gaining additional importance with the production of high frame rate content in the entertainment industry and use of high-speed cameras for technical and scientific studies [9]. Mackin, et. al, [10], for example, describes motion compensated frame interpolation using the shutter interval of the source video as input. Other references provide guidance on the use of shutter interval to condition use of frame rate conversion. For example, it is generally understood that frame rate conversion that results in a shutter interval less than ½ the time between pictures (i.e., ½ the reciprocal of frame rate) is unacceptable for frame rates below 50 fps [6]. When frame blending is used for frame rate conversion, ATSC A/341 Annex D [11] indicates that simple averaging of frames produces good results when the shutter interval of the HFR video is equal to the reciprocal of frame rate, but unequal weights favoring the current frame should be used for shorter shutter intervals. Similarly, DVB TM-AVC 1146 [12] recommends setting a blending ratio based on the value of shutter interval.

# Syntax and semantics

The proposed syntax and semantics for the SII SEI message are identical to the syntax and semantics specified in HEVC except that the value of sps_max_sub_layers_minus1 is passed to the SEI message by means of a variable specified in VVC. It is proposed that the SII SEI message payloadType be equal to 205 to be consistent with HEVC and AVC.

## Proposed addition to the VVC specification

It is proposed to add the following to the VVC specification:

D.9.x	Use of the shutter interval information SEI message syntax

For purposes of interpretation of the shutter interval information SEI message, the variable SpsMaxSubLayersMinus1 is set equal to sps_max_sublayers_minus1.

## Proposed addition to the VSEI specification

It is proposed to add the following to the VSEI specification:

8.xx	Shutter interval information SEI message syntax

| shutter_interval_info( payloadSize ) { | Descriptor |
| --- | --- |
| sii_time_scale | u(32) |
| fixed_shutter_interval_within_clvs_flag | u(1) |
| if( fixed_shutter_interval_within_clvs_flag ) |  |
| sii_num_units_in_shutter_interval | u(32) |
| else { |  |
| sii_max_sub_layers_minus1 | u(3) |
| for( i = 0; i <= sii_max_sub_layers_minus1; i++ ) |  |
| sub_layer_num_units_in_shutter_interval[ i ] | u(32) |
| } |  |
| } |  |

    - Shutter interval information SEI message semantics

The shutter interval information SEI message indicates the shutter interval for the associated video source pictures prior to encoding, e.g., for camera-captured content, the shutter interval is amount of time that an image sensor is exposed to produce each source picture.

When a shutter interval information SEI message is present for any picture of a CLVS of a particular layer, a shutter interval information SEI message shall be present for the first picture of the CLVS. The shutter interval information SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All shutter interval information SEI messages that apply to the same CLVS shall have the same content.

sii_time_scale specifies the number of time units that pass in one second. The value of sii_time_scale shall be greater than 0. For example, a time coordinate system that measures time using a 27 MHz clock has an sii_time_scale of 27 000 000.

fixed_shutter_interval_within_clvs_flag equal to 1 specifies that the indicated shutter interval is the same for all temporal sub-layers in the CLVS. fixed_shutter_interval_within_clvs_flag equal to 0 specifies that the indicated shutter interval may not be the same for all temporal sub-layers in the CLVS. When the value of SpsMaxSubLayersMinus1 is equal to 0, the value of fixed_shutter_interval_within_clvs_flag shall be equal to 1.

sii_num_units_in_shutter_interval, when fixed_shutter_interval_within_clvs_flag is equal to 1, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the indicated shutter interval of each picture in the CLVS. The value 0 may be used to indicate that the associated video content contains screen capture content, computer generated content, or other non-camera-captured content.

The indicated shutter interval, denoted by the variable shutterInterval, in units of seconds, is equal to the quotient of sii_num_units_in_shutter_interval divided by sii_time_scale. For example, to represent a shutter interval equal to 0.04 seconds, sii_time_scale may be equal to 27 000 000 and sii_num_units_in_shutter_interval may be equal to 1 080 000.

sii_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CLVS referring to the SPS. The value of sii_max_sub_layers_minus1 shall be equal to the value of SpsMaxSubLayersMinus1.

NOTE – For example, the information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of ATSC A/341:2019 Annex D when sii_max_sub_layers_minus1 is equal to 1 and fixed_shutter_interval_within_clvs_flag is equal to 0.

sub_layer_num_units_in_shutter_interval[ i ], when present, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the shutter interval of each picture in the sub-layer representation with TemporalId equal to i in the CLVS. The sub-layer shutter interval for the sub-layer representation with TemporalId equal to i, denoted by the variable subLayerShutterInterval[ i ], in units of seconds, is equal to the quotient of sub_layer_num_units_in_shutter_interval[ i ] divided by sii_time_scale.

The variable subLayerShutterInterval[ i ], corresponding to the indicated shutter interval of each picture in the sub-layer representation with TemporalId equal to i in the CLVS, is thus derived as follows:

if( fixed_shutter_interval_within_clvs_flag )
	subLayerShutterInterval[ i ] = sii_num_units_in_shutter_interval ÷ sii_time_scale	(X)
else
	subLayerShutterInterval[ i ] = sub_layer_num_units_in_shutter_interval[ i ] ÷ sii_time_scale

# Example use case – backwards-compatible HFR bit stream

## Backwards-compatible HFR video

JVET-M0579 [13] considered design requirements for supporting multiple frame rates within VVC and proposed that temporal scalability can be used to provide backwards compatibility for non-HFR capable receivers. JVET-M0579 also described methods being considered by ATSC and DVB at the time to extract high quality sequences at different bit rates. An update on the ATSC and DVB work is provided below.

A backwards-compatible HFR bit stream is one for which a non-HFR-compatible receiver produces an SFR decoded output.

ETSI TS 101 145 (DVB) specifies two types of HFR bit streams: HFR bit streams using dual PID and temporal scalability (dual PID HFR) and HFR bit streams using a single PID (single PID HFR). For the dual PID case, the HFR bit streams are backwards compatible and SFR receivers are expected to display video with fair quality motion at half the frame rate (up to 60 fps), though the motion quality need not be as good as an HFR receiver at full frame rate (up to 120 fps). For the single PID case, the HFR bit streams are not backwards compatible and SFR receivers are not expected to display video.

Backwards-compatibility can be achieved using temporal scalability to decode only every other HFR picture, for example. However, the resulting SFR output often exhibits temporal judder, strobing, and unnatural motion.

Alternatively, backwards compatibility can be achieved using a bit stream that contains both SFR and HFR pictures assigned to different temporal sublayers and for which the shutter interval for the SFR content is greater than the shutter interval of the HFR content, as illustrated in Figure 1. As noted in ATSC A/341, such bit streams can be created by the camera capture system, in the creation of computer-generated content, or by pre-processing HFR content using methods such as the Multiple Frame Rate Filtering (MFRF) method described in ATSC A/341, which is similar to the picture blending process described in DVB TM-AVC 1146. Another method used in cinematic productions to create synthetic shutter intervals is described by Davis [14].

Figure 1. A backwards-compatible HFR bit stream containing SFR, P’SFR(t), and HFR, P’HFR(t), pictures with different shutter intervals, shutterInterval[ i ]

When a backwards-compatible HFR bit stream as illustrated in is received by a non-HFR-capable decoder, only the backwards-compatible temporal sublayer of the SFR pictures is decoded, as illustrated in Figure 2A. When the bit stream is received by an HFR-capable decoder, all temporal sublayers are decoded and MFRF, or similar process, can be used to reconstruct the HFR video, as follows:

PHFRt=PSFRt-w1PHFRt-1w2							(1)

where PHFRt-1 and PSFRt are adjacent decoded pictures in display order, PHFRt is the current reconstructed HFR picture. The reconstruction process is invoked for every other decoded picture in display order, as illustrated in Figure 2B.

Figure 2. Output process for backwards-compatible HFR bit stream: A, only the SFR video is decoded by a non-HFR-capable receiver; B, post-processing to reconstruct HFR video from the output of an HFR-capable decoder

## Selecting values for weighting parameters w1 and w2

Weighting parameters, w1 and w2, should be selected such that w1+w2=1 so that average luma and chroma values are preserved. In addition, w1 and w2 should be greater than or equal to 0.

When using frame blending to create a backwards-compatible HFR bit stream, ATSC A/341 indicates that setting the values of the weighting parameters to w1=w2=0.5 produces good results when the shutter interval of the HFR video is equal to the reciprocal of HFR frame rate. In this case, the image sensor would be continuously exposed and the shutter interval of the SFR video would consequently be twice the reciprocal of the HFR frame rate. When the HFR shutter interval is significantly less than the reciprocal of the frame rate, ATSC A/341 indicates that it can be beneficial to set the values of the weighting parameters such that w1<w2.

DVB TM-AVC 1146 proposes that the weighting parameters, w1 and w2, be determined from a blending ratio, M:N, as follows:

w1=NM+N 	and 	w2=MM+N 							(2)

Based on subjective testing, DVB TM-AVC 1146 recommends setting a blending ratio of 4:1 (i.e., w1=15 and w2=45) when the HFR video has a shutter interval equal to the reciprocal of HFR frame rate, and either 4:1 or 2:1 (i.e., w1=13 and w2=23) when the HFR video has a shutter interval ½ the reciprocal of the HFR frame rate. DVB TM-AVC 1146 also asserts that a blending ratio of 4:1 is adequate for a wide range of HFR content, but that the optimal blending ratio could be determined based on content.

# Description of the SII SEI message showcase VTM software

## Algorithm description

The HFR reconstruction process (Eqn. 1) is invoked when all the following apply:

- fixed_shutter_interval_within_clvs_flag is equal to 0

- sii_max_sub_layers_minus1 is greater than 0

- subLayerShutterInterval[ sii_max_sub_layers_minus1 ] is less than subLayerShutterInterval [ sii_max_sub_layers_minus1 - 1 ]

- subLayerShutterInterval[ i ] is equal to subLayerShutterInterval [ i – 1 ] for i = 0 … sii_max_sub_layers_minus1 – 1

The values of M and N in Eqn. 2 are derived as follows in the showcase VTM software:

M=shutterIntervali-1shutterIntervali 	and		 N=1					(3)

The showcase software can be modified to experiment with other values for w1 and w2.

## Description of VTM software package

The software provided is based on the latest version of VVC reference software VTM (SHA: 53490ea3dc266cf53073b17cd3ff611e465ac3be) after the shutter interval information SEI messages is merged. Provided software also illustrates the example backwards-compatible HFR use case.

The SII SEI message part is enabled by Macro:

- SHUTTER_INTERVAL_SEI_MESSAGE = 1

The SII-Process part is enabled by Macro:

- SHUTTER_INTERVAL_SEI_PROCESSING = 1.

Two encoder configurations are provided:

- shutter_interval_info_fix.cfg	// common setting with single shutter interval

- shutter_interval_info_mul.cfg	// shutter interval SEI processing with multiple shutter intervals

The encoding command line is:

EncoderAppStatic -c seq.cfg -c encoder_randomaccess_vtm.cfg -c shutter_interval_info_mul.cfg  -sii preprocessed_seq.yuv

The decoding command line is:

DecoderAppStatic -b seq.bit  -o dec.yuv -sii postprocessd_seq.yuv

Optionally, to decode only the SFR video, use:

DecoderAppStatic -b seq.bit  -o dec.yuv -t 4.

# Summary and recommendations

It is recommended that the shutter interval information SEI message already specified in HEVC be adopted in the next version of VSEI and that corresponding text for the use of the SII SEI message be added to the next version of VVC. It is also recommended that the showcase software be adopted in VTM to provide a means of experimenting with shutter interval information.

# References

- S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen, “Illustration of the shutter interval info SEI message in HEVC Draft”, JCTVC-AM0024, teleconference, April 2020

- S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen, “Shutter interval info SEI message in AVC”, JCTVC-AN0023, teleconference, July 2020

- S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen, “AHG17: Shutter interval information SEI message”, JVET-P0338, Geneva, CH, October 2019

- ETSI 101 154: Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications.

- A. B. Watson, "High Frame Rates and Human Vision: A View through the Window of Visibility," in SMPTE Motion Imaging Journal, vol. 122, no. 2, pp. 18-32, March 2013

- C. Carbonara, J. DeFilippis and M. Korpi, "High Frame Rate Capture and Production," SMPTE 2015 Annual Technical Conference and Exhibition, 2015, pp. 1-19

- R. Selfridge, K. C. Noland, and M. Hansard, “Visibility of Motion Blur and Strobing Artefacts in Video at 100 Frames per Second,” BBC R&D White Paper, WHP 328, January 2017

- I. Beitzel, A. Kuder and J. Fröhlich, "The Effect of Synthetic Shutter on Judder Perception—an HFR & HDR Data Set and User Study," in SMPTE Motion Imaging Journal, vol. 129, no. 1, pp. 42-50, Jan.-Feb. 2020

- B. Pueo, “High speed cameras for motion analysis in sports science,” Journal of Human Sport and Exercise, v. 11, n. 1, p. 53-73, dec. 2016

- A. Mackin, F. Zhang and D. Bull, "A Frame Rate Conversion Method Based on a Virtual Shutter Angle," 2019 IEEE International Conference on Image Processing (ICIP), 2019, pp. 4215-4219

- “ATSC Standard: Video – HEVC”, Advanced Television Systems Committee, A/341:2019, 14 Feb. 2019

- “DVB TM-AVC 1146 HFR Guidelines Annex draft.” https://member.dvb.org/wg/TM-AVC/document/32653 (accessed Mar. 24, 2022).

- A. Segall, S. Deshpande, and M. Hannuksela, “On Frame Rate Support and Extraction in VVC” JVET-M0579, Marrakech, MA January 2019

- T. Davis, "Rethinking Frame Rate and Temporal Fidelity in a Cinema Workflow," in SMPTE Motion Imaging Journal, vol. 126, no. 8, pp. 62-71, Oct. 2017

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).