| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0123-v2 |
| --- | --- |

| Title: | EE2-related: On chroma interpolation filters for motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Gan Yue Yu Haoping Yu | Tel: Email: | +612 401 455 210 v-jonathan.gan@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes to replace the chroma interpolation filters in ECM-4.0 with 6 tap interpolation filters derived according to a cosine windowed sinc design. It is asserted that the design method is consistent with the luma interpolation filters in ECM-4.0. The gain on top of ECM-4.0 is reported to be {-0.06%, -1.14%, -1.2%} for RA configuration and {-0.11%, -1.35%, -1.36%} for LDB configuration.

# Introduction

In the ongoing activity on enhanced compression beyond VVC capability, 12 tap interpolation filters for luma motion compensated prediction have been adopted into the ECM-4.0 software [1]. These interpolation filters were adopted from JVET-U0100 [2], and are asserted in that contribution to be derived from a windowed sinc design, using a cosine window function.

Currently, the interpolation filters for chroma motion compensated prediction are unchanged from VVC. However, at the 25th JVET meeting, 12 tap chroma interpolation filters were proposed by JVET-Y0172 [3]. Half of the proposed chroma interpolation filters are identical to the luma interpolation filters in ECM4.0 which interpolate the same fractional position. The other half of the chroma interpolation filters are new. The design principle for these new chroma interpolation filters is not disclosed.

Following discussion of JVET-Y0172, it was agreed ([4]) that the proponent would conduct exploratory experiments on different lengths of chroma interpolation filters: 6 tap, 8 tap, and 12 tap.

This contribution proposes 6 tap chroma interpolation filters derived according to a windowed sinc design with cosine window function. It is asserted that the design method used for the chroma interpolation filters proposed in this contribution is consistent with that used for the luma interpolation filters from JVET-U0100.

# Windowed sinc design

In the context of bandlimited interpolation, the ideal interpolation function is the normalised sinc function:

sincs=sinπsπs           s≠0      1                  s=0

However, the ideal interpolator is not implementable because it has infinite support. To design implementable interpolation filters with finite support, the sinc function may be windowed. For interpolation filters with support length N, in this contribution a cosine window is used with the following analytic form:

ws=    0                      s<-N2cosπsN       -N2≤s≤N20                      s>N2

Floating point filters for each fractional position p (eg p∈032,132,232, …3132) are obtained by sampling the windowed sinc function at positions (s-p) for s∈{-N2+1, -N2+2, …0, 1,…N2}. That is, the floating point interpolation filter for fractional position p is

hp=sinc-N2+1-pw-N2+1-p,sinc-N2+2-pw-N2+2-p,…  sincN2-pwN2-p

To obtain integerised filter coefficients for a fixed bit depth B, the floating point filter coefficients are multiplied by 2B and rounded off to integers, where the rounding is controlled to take care that the DC gain of the filter is 2B.

The above design procedure is followed for N=6, B=8, and fractional positions with increments of 1/32. For informational purposes, the filter coefficient values pre-integerisation (rounded to 2 decimal places) are shown in Table 1:

| Fractional position | Filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0/32 | 0 | 0 | 256 | 0 | 0 | 0 |
| 1/32 | 1.91 | -6.64 | 255.55 | 7.21 | -2.09 | 0.04 |
| 2/32 | 3.63 | -12.71 | 254.22 | 14.95 | -4.33 | 0.18 |
| 3/32 | 5.16 | -18.18 | 252.01 | 23.22 | -6.72 | 0.4 |
| 4/32 | 6.49 | -23.05 | 248.94 | 31.96 | -9.24 | 0.71 |
| 5/32 | 7.62 | -27.32 | 245.02 | 41.16 | -11.86 | 1.1 |
| 6/32 | 8.54 | -30.99 | 240.29 | 50.75 | -14.55 | 1.58 |
| 7/32 | 9.27 | -34.07 | 234.77 | 60.71 | -17.29 | 2.12 |
| 8/32 | 9.8 | -36.57 | 228.51 | 70.98 | -20.04 | 2.73 |
| 9/32 | 10.15 | -38.51 | 221.54 | 81.51 | -22.78 | 3.4 |
| 10/32 | 10.32 | -39.9 | 213.92 | 92.24 | -25.47 | 4.11 |
| 11/32 | 10.33 | -40.78 | 205.69 | 103.11 | -28.07 | 4.84 |
| 12/32 | 10.19 | -41.16 | 196.9 | 114.06 | -30.55 | 5.6 |
| 13/32 | 9.91 | -41.09 | 187.62 | 125.04 | -32.86 | 6.35 |
| 14/32 | 9.52 | -40.58 | 177.91 | 135.96 | -34.97 | 7.08 |
| 15/32 | 9.02 | -39.68 | 167.82 | 146.78 | -36.83 | 7.78 |
| 16/32 | 8.44 | -38.41 | 157.42 | 157.42 | -38.41 | 8.44 |
| 17/32 | 7.78 | -36.83 | 146.78 | 167.82 | -39.68 | 9.02 |
| 18/32 | 7.08 | -34.97 | 135.96 | 177.91 | -40.58 | 9.52 |
| 19/32 | 6.35 | -32.86 | 125.04 | 187.62 | -41.09 | 9.91 |
| 20/32 | 5.6 | -30.55 | 114.06 | 196.9 | -41.16 | 10.19 |
| 21/32 | 4.84 | -28.07 | 103.11 | 205.69 | -40.78 | 10.33 |
| 22/32 | 4.11 | -25.47 | 92.24 | 213.92 | -39.9 | 10.32 |
| 23/32 | 3.4 | -22.78 | 81.51 | 221.54 | -38.51 | 10.15 |
| 24/32 | 2.73 | -20.04 | 70.98 | 228.51 | -36.57 | 9.8 |
| 25/32 | 2.12 | -17.29 | 60.71 | 234.77 | -34.07 | 9.27 |
| 26/32 | 1.58 | -14.55 | 50.75 | 240.29 | -30.99 | 8.54 |
| 27/32 | 1.1 | -11.86 | 41.16 | 245.02 | -27.32 | 7.62 |
| 28/32 | 0.71 | -9.24 | 31.96 | 248.94 | -23.05 | 6.49 |
| 29/32 | 0.4 | -6.72 | 23.22 | 252.01 | -18.18 | 5.16 |
| 30/32 | 0.18 | -4.33 | 14.95 | 254.22 | -12.71 | 3.63 |
| 31/32 | 0.04 | -2.09 | 7.21 | 255.55 | -6.64 | 1.91 |

Table 1: Pre-integerised coefficients for 6 tap windowed sinc interpolation filters

Integerised coefficient values are obtained by either rounding up or down while ensuring that the coefficient values sum to 256. Within this constraint, the coefficient values are optimised to minimise the error from the ideal interpolator:

argminhp ωϵω2

ϵω=hpω2B-e-jωp

# Proposed method

This contribution proposes to replace the 4 tap chroma interpolation filters from VVC with the 6 tap chroma interpolation filters as shown in Table 2.

| Fractional position | Filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0/32 | 0 | 0 | 256 | 0 | 0 | 0 |
| 1/32 | 1 | -6 | 255 | 8 | -2 | 0 |
| 2/32 | 3 | -12 | 254 | 14 | -4 | 1 |
| 3/32 | 5 | -18 | 252 | 23 | -7 | 1 |
| 4/32 | 7 | -23 | 248 | 32 | -9 | 1 |
| 5/32 | 7 | -27 | 245 | 41 | -11 | 1 |
| 6/32 | 8 | -30 | 240 | 50 | -14 | 2 |
| 7/32 | 10 | -34 | 234 | 60 | -17 | 3 |
| 8/32 | 10 | -36 | 228 | 71 | -20 | 3 |
| 9/32 | 10 | -38 | 221 | 81 | -22 | 4 |
| 10/32 | 10 | -39 | 213 | 92 | -25 | 5 |
| 11/32 | 11 | -40 | 205 | 103 | -28 | 5 |
| 12/32 | 11 | -41 | 196 | 114 | -30 | 6 |
| 13/32 | 10 | -41 | 187 | 125 | -32 | 7 |
| 14/32 | 10 | -40 | 177 | 135 | -34 | 8 |
| 15/32 | 10 | -39 | 167 | 146 | -36 | 8 |
| 16/32 | 9 | -38 | 157 | 157 | -38 | 9 |
| 17/32 | 8 | -36 | 146 | 167 | -39 | 10 |
| 18/32 | 8 | -34 | 135 | 177 | -40 | 10 |
| 19/32 | 7 | -32 | 125 | 187 | -41 | 10 |
| 20/32 | 6 | -30 | 114 | 196 | -41 | 11 |
| 21/32 | 5 | -28 | 103 | 205 | -40 | 11 |
| 22/32 | 5 | -25 | 92 | 213 | -39 | 10 |
| 23/32 | 4 | -22 | 81 | 221 | -38 | 10 |
| 24/32 | 3 | -20 | 71 | 228 | -36 | 10 |
| 25/32 | 3 | -17 | 60 | 234 | -34 | 10 |
| 26/32 | 2 | -14 | 50 | 240 | -30 | 8 |
| 27/32 | 1 | -11 | 41 | 245 | -27 | 7 |
| 28/32 | 1 | -9 | 32 | 248 | -23 | 7 |
| 29/32 | 1 | -7 | 23 | 252 | -18 | 5 |
| 30/32 | 1 | -4 | 14 | 254 | -12 | 3 |
| 31/32 | 0 | -2 | 8 | 255 | -6 | 1 |

Table 2: Proposed 6 tap chroma interpolation filters

# Experimental results

The proposed method is tested on top of ECM-4.0 under the CTC conditions [5]. The results are shown in Table 3. Runtimes are not reliable.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.20% | -0.10% | 99% | 100% |
| Class A2 | -0.08% | -0.83% | -0.81% | 99% | 103% |
| Class B | -0.07% | -1.06% | -1.33% | 102% | 101% |
| Class C | -0.05% | -2.17% | -2.14% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -1.14% | -1.20% | 102% | 103% |
| Class D | -0.01% | -3.40% | -3.95% | 105% | 108% |
| Class F | -0.08% | -1.17% | -1.23% | 103% | 108% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -1.60% | -1.78% | 99% | 99% |
| Class C | -0.11% | -1.99% | -2.38% | 102% | 106% |
| Class E | -0.08% | -0.08% | 0.70% | 96% | 101% |
| Overall | -0.11% | -1.35% | -1.36% | 100% | 102% |
| Class D | -0.19% | -2.09% | -3.70% | 101% | 106% |
| Class F | -0.32% | -0.38% | -1.08% | 95% | 100% |

Table 3: Experiment results for proposed 6-tap chroma filters

# Additional information

After upload of this contribution, it was found that the 8-tap and 12-tap tests planned in [4] were withdrawn. For informative purposes, the windowed sinc method described in section 2 is also used to design 8-tap chroma interpolation filters (N=8, B=8). The filter coefficients are as below:

{  0,   0,   0, 256,   0,   0,   0,   0},

{ -1,   2,  -7, 255,   8,  -2,   1,   0},

{ -1,   5, -13, 254,  15,  -5,   2,  -1},

{ -2,   7, -19, 252,  24,  -9,   4,  -1},

{ -3,   9, -25, 249,  33, -12,   5,   0},

{ -3,  11, -29, 245,  43, -15,   5,  -1},

{ -4,  13, -34, 240,  52, -18,   7,   0},

{ -4,  15, -37, 235,  63, -22,   8,  -2},

{ -5,  16, -40, 229,  73, -25,   9,  -1},

{ -5,  17, -43, 222,  84, -28,  11,  -2},

{ -5,  18, -44, 215,  94, -31,  12,  -3},

{ -5,  18, -46, 207, 105, -34,  13,  -2},

{ -5,  18, -46, 198, 116, -37,  15,  -3},

{ -5,  18, -47, 189, 127, -39,  16,  -3},

{ -5,  18, -46, 179, 138, -41,  16,  -3},

{ -5,  18, -46, 170, 149, -43,  17,  -4},

{ -4,  18, -45, 159, 159, -45,  18,  -4},

{ -4,  17, -43, 149, 170, -46,  18,  -5},

{ -3,  16, -41, 138, 179, -46,  18,  -5},

{ -3,  16, -39, 127, 189, -47,  18,  -5},

{ -3,  15, -37, 116, 198, -46,  18,  -5},

{ -2,  13, -34, 105, 207, -46,  18,  -5},

{ -3,  12, -31,  94, 215, -44,  18,  -5},

{ -2,  11, -28,  84, 222, -43,  17,  -5},

{ -1,   9, -25,  73, 229, -40,  16,  -5},

{ -2,   8, -22,  63, 235, -37,  15,  -4},

{  0,   7, -18,  52, 240, -34,  13,  -4},

{ -1,   5, -15,  43, 245, -29,  11,  -3},

{  0,   5, -12,  33, 249, -25,   9,  -3},

{ -1,   4,  -9,  24, 252, -19,   7,  -2},

{ -1,   2,  -5,  15, 254, -13,   5,  -1},

{  0,   1,  -2,   8, 255,  -7,   2,  -1},

Partial results for 8-tap chroma filters designed by windowed sinc are shown in Table 4.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | -2.70% | -2.49% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | -4.08% | -4.57% | 98% | 96% |
| Class F | -0.05% | -1.14% | -1.21% | 97% | 97% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | 0.01% | -0.20% | 1.18% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.18% | -3.29% | -4.94% | 99% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Table 4: Experiment results for 8-tap chroma filters

# Conclusion

This contribution suggests adopting the proposed 6-tap filters to the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM

[2] Y. J. Chang, C. C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. P. Van, H. Wang, Y. Zhang, Z. Zhang, “Compression efficiency methods beyond VVC”, JVET-U0100, Meeting by teleconference, 6–15 January 2021.

[3] X. Xie, K. Zhang, L. Zhang, J. Li, M. Wang, S. Wang, “Non-EE2: Long tap interpolation filtering on chroma components”, JVET-Y0172, Meeting by teleconference, 12-21 January 2022.

[4] V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Meeting by teleconference, 12-21 January 2022.

[5] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing", JVET-Y2017, Meeting by teleconference, 12-21 January 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).