| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0126-v1 |
| --- | --- |

| Title: | Non-EE2: Improvement on Local Illumination Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes two new LIC modes to improve the coding efficiency of ECM. One is LIC_T that only uses the top template samples to calculate the linear model and the other LIC_L only uses the left template samples to calculate the linear model. Compared with ECM-4.0 anchor, the simulation results are shown as below:

RA:

LDB:

# Introduction

In the EE2 common software, ECM, one of the inter prediction tools is local illumination compensation (LIC)[1]. LIC aims to model local illumination variation between the current block and its prediction block as a function of that between a current block template and a reference block template.

The linear model contains two parameters which can be denoted by a slope α and an offset β. These two parameters form a linear equation to compensate illumination changes, and the prediction pred'x at a location x in the current picture is calculated as follows:

pred'x= α∙predx+ β

where predx is a reference sample in a reference picture pointed to by MV at a location x. α and β are derived based on a current block template and a reference block template, so no signalling overhead is required to represent them in the bitstream.

Fig.1 LIC processing

The current design of LIC in ECM is shown in Fig1. LIC uses both the top and left templates to calculate the parameters, when both top and left templates are available. If only one of the templates is available, then LIC uses the only available one to calculate parameters.

A LIC flag is signalled for the AMVP mode to indicate the use of LIC. This LIC flag is included as part of the motion information in addition to the motion vectors and reference indices. When the merge candidate list is constructed, the LIC flag is inherited from the neighbor blocks for merge candidates, except for the temporal motion vector predictor (TMVP).

# Proposed method

This contribution proposes to add two more LIC modes, namely LIC_T mode and LIC_L mode, to improve the coding efficiency of the local illumination compensation. For ease of understanding, the current LIC mode in ECM can be denoted by LIC_TL, which means using both top and left templates to form a linear model.

When the top template is available, LIC_T may be applied to the current block, which only uses the top templates of the current block and reference block to calculate the linear model, shown in Fig.2.

Fig.2 LIC_T mode

Similarly, when the left template is available, LIC_L may be applied to the current block, which only uses the left templates of the current block and reference block to calculate the linear model, shown as in Fig.3.

Fig.3 LIC_L mode

The binarization representation of the proposed LIC mode is shown in Table 1. The first two bins are context coded and the last bin is bypass coded.

Table 1. The binarization of the proposed LIC modes

| LIC usage | bin string |
| --- | --- |
| LIC off | 0 |
| LIC_TL | 10 |
| LIC_T | 110 |
| LIC_L | 111 |

LIC_T and LIC_L will not be inherited with the existing LIC flag currently in the merge mode. When constructing the merge candidate list, the LIC flag will be inherited from the neighbor blocks for the merge candidates and the LIC mode will be always set as LIC_TL.

# Simulation results

The proposed method is implemented on top of ECM-4.0. The simulations were performed following the ECM common test condition (CTC)[2].

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.05% | -0.09% | 109% | 100% |
| Class C | -0.07% | -0.14% | 0.04% | 108% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.05% | 0.01% | 108% | 100% |
| Class F | -0.17% | -0.30% | -0.18% | 105% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusion

This contribution proposes an improvement over the current LIC scheme. The proposed method is implemented on top of ECM-4.0. Compared with the ECM-4.0 anchor, the proposed method can provide good coding performance without increasing decoder-side complexity. It is suggested to further study this proposal in EE2.

# Reference

[1] Yao-Jen Chang, etc. “Compression efficiency methods beyond VVC”, JVET-U0100, Jan. 2021.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).