| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0127-v1 |
| --- | --- |

| Title: | Non-EE2: On the maximum number of MHP merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In ECM-4.0, GPM merge candidate list construction is also used for the merge mode of additional hypothesis in MHP including reusing the maximum number of GPM candidates, so when GPM mode is disabled the maximum number of merge candidates for MHP is undefined. This contribution introduces independent syntax element for the maximum number of MHP merge candidates that MHP merge mode can be enabled independently of GPM.

# Problem statement

In ECM-4.0, GPM merge candidate list construction is also used for the merge mode of additional hypothesis in MHP including reusing the maximum number of GPM candidates, so when GPM mode is disabled the maximum number of merge candidates for MHP is undefined, i.e. MHP cannot be used independently of GPM.

# Proposed solutions

Two solutions can be applied to solve the above-mentioned problem.

Solution 1

Disable the merge mode for the additional hypothesis in MHP if GPM mode is disabled.

Solution 2

Create separate syntax element for the maximum number of MHP merge candidates signaled as a difference max_num_merge_cand_minus_max_num_mhp_cand, similar to that of GPM.  The merge flag for the additional hypothesis is inferred to be false if the maximum number of MHP merge candidates is equal to 0.

An exemplary syntax table for the maximum number of MHP merge candidates is as follows:

| if( inter_multi_hyp_enable_flag && MaxNumMergeCand >= 1 ) { |  |
| --- | --- |
| sps_max_num_merge_cand_minus_max_num_mhp_cand | ue(v) |
| } |  |

The patches to ECM-4.0 of both solutions are attached in contribution package. In the proponent’s view, solution 2 is preferred.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).