| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0135-v1 |
| --- | --- |

| Title: | EE2-Test4.3: Combined tests of EE2-4.1 and EE2-4.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Hossein Golestani, Nan Hu, Patrick Garus, Marta Karczewicz Xiaoyu Xiu, Wei Chen, C.-W. Kuo, Hong-Jheng Jhu, Ning Yan, Xianglin Wang | Email: | vseregin@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | Qualcomm Incorporated, Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Exploration Experiment 2 Test-4.3 for CABAC improvements, which is the combination of Test-4.1 and Test-4.2. Two combined tests are conducted in EE2. Test-4.3a is the combination of the probability estimation with adaptive weight in Test-4.1a and the adaptive window size adjustment in Test-4.2b. Test-4.3b is temporal CABAC inheritance in Test-4.1b and Test-4.2a/c enabled on top of Test-4.3a. Simulation results are summarized as follows:

- Test-4.3a: Probability estimation with adaptive weight and adaptive window size adjustment

- AI:  -0.13 % (Y), 0.07% (U), 0.05% (V), xxx% (EncT), xxx% (DecT)

- RA: -0.22% (Y),  0.15% (U), 0.20% (V), xxx% (EncT), xxx% (DecT)

- LB: -0.36% (Y), -0.22% (U), -0.36%(V), xxx% (EncT), xxx% (DecT)

- Test-4.3a*: Probability estimation with adaptive weight, adaptive window size adjustment and initial probability and rates from Test-4.2

- AI:  -0.14 % (Y), 0.12% (U), 0.11% (V), xxx% (EncT), xxx% (DecT)

- RA: -0.25% (Y),  0.30% (U), 0.50% (V), xxx% (EncT), xxx% (DecT)

- LB: -0.34% (Y),  0.71% (U),  0.73%(V), xxx% (EncT), xxx% (DecT)

- Test-4.3b: Probability estimation with adaptive weight, adaptive window size adjustment, and temporal CABAC inheritance

- AI:  -0.13 % (Y), 0.07% (U), 0.05% (V), xxx% (EncT), xxx% (DecT)

- RA: -0.51% (Y), 0.08% (U), 0.05% (V), xxx% (EncT), xxx% (DecT)

- LB: -0.85% (Y), 0.64% (U), 0.75% (V), xxx% (EncT), xxx% (DecT)

# Introduction

At the JVET-Y meeting, JVET-Y0157 and JVET-Y0181 were proposed to improve the probability estimation accuracy in ECM and the two contributions were included in Exploration Experiment 2 (EE2) as Test-4.1 and Test-4.2 [3] for further evaluation. JVET-Y0157 proposes to calculate the probability of each context based on the weighted combination of its two probability estimates instead of using the simple average in ECM4.0. JVET-Y0181 proposes to dynamically adjust the window size when coding each bin of one context according to its preceding bin. Additionally, in both JVET-Y0157 and JVET-Y0181, it is proposed to initialize the context states of an inter slice by inheriting the probabilities of the inter slice of the previously coded picture that has the same slice type, QP value, and temporal ID. This contribution reports the results of the combined tests of Test-4.1 and Test-4.2.

# Technical aspects

## Test-4.3a

Test-4.3a is the direct combination of Test-4.1a (probability estimation with adaptive weight) and Test-4.2b (adaptive window size adjustment). Specifically, compared to the CABAC design in ECM4.0, Test-4.3a includes the following two modifications:

- Five different weights (which are selected from a pre-defined set {10, 12, 16, 20, 22}/32) are introduced to derive the probability used for the binary arithmetic coding of each context. Three different weights are pre-determined for each context at I-, B- and P-slice types. The weights of I-slice type are only allowed for intra slices while the weights of B- and P-slice types can be switched for inter slices based on the existing syntax sh_cabac_init_flag at slice level.

- Window size for each of the two CABAC states is adjusted by an offset parameter (4 bits) stored in a look-up table. The offset is retrieved from the look-up table by the previously coded bin of each context used as an index, the offset is added to the window size specified for each context in the existed initialization tables.

## Test-4.3b

Test-4.3b is on top of Test-4.3a and additionally applies the temporal CABAC inheritance in Test-4.1b and Test-4.2a/c. Specifically, compared to the CABAC design in ECM4.0, Test-4.3b includes the following three improvements:

- Five different weights (which are selected from a pre-defined set {10, 12, 16, 20, 22}/32) are introduced to derive the probability used for the binary arithmetic coding of each context. Three different weights are pre-determined for each context at I-, B- and P-slice types. The weights of I-slice type are only allowed for intra slices while the weights of B- and P-slice types can be switched for inter slices based on the existing syntax sh_cabac_init_flag at slice level.

- Window size for each of the two CABAC states is adjusted by an offset parameter (4 bits) stored in a look-up table. The offset is retrieved from the look-up table by the previously coded bin of each context used as an index, the offset is added to the window size specified for each context in the existed initialization tables.

- Temporal inheritance-based context initialization is applied to inter slices. The probabilities and the weights of the context models of the inter slice (i.e., B- or P-slice types) that contains the last CTU in the previously coded picture is stored. The stored states will be used to initialize the context models in the next inter slice that has the same slice type, the QP value, and temporal layer ID.

# Simulation results

The proposed scheme was implemented on top of the ECM4.0 [4] and tested using the common test conditions as defined in [5]. Table 1 and Table 3 show the results of Test-4.3a and Test-4.3b. Table 2 shows the complementary results of Test-4.3a (which is named as Test-4.3a*) where the weights used for the probability estimation are derived based on the initial probabilities and rates used in Test-4.2 [5]

Table 1 Results of Test-4.3a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.13% | 0.02% |  |  |
| Class A2 | -0.10% | 0.07% | 0.06% |  |  |
| Class B | -0.11% | 0.05% | 0.11% |  |  |
| Class C | -0.12% | -0.04% | 0.06% |  |  |
| Class E | -0.15% | 0.23% | -0.03% |  |  |
| Overall | -0.13% | 0.07% | 0.05% |  |  |
| Class D | -0.09% | -0.05% | -0.15% |  |  |
| Class F | -0.24% | 0.03% | -0.03% |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.26% | 0.22% |  |  |
| Class A2 | -0.21% | 0.23% | 0.10% |  |  |
| Class B | -0.26% | 0.19% | 0.29% |  |  |
| Class C | -0.19% | -0.04% | 0.16% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.22% | 0.15% | 0.20% |  |  |
| Class D | -0.12% | -0.33% | -0.48% |  |  |
| Class F | -0.18% | -0.28% | -0.12% |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | 0.49% | 0.86% |  |  |
| Class C | -0.27% | 0.36% | -0.23% |  |  |
| Class E | -0.64% | 1.37% | -0.31% |  |  |
| Overall | -0.36% | 0.67% | 0.20% |  |  |
| Class D | -0.37% | 0.23% | 0.10% |  |  |
| Class F | -0.44% | 0.84% | 0.44% |  |  |

Table 2 Results of Test-4.3a*

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.16% | 0.08% |  |  |
| Class A2 | -0.11% | 0.16% | 0.01% |  |  |
| Class B | -0.12% | 0.08% | 0.23% |  |  |
| Class C | -0.13% | 0.08% | 0.00% |  |  |
| Class E | -0.17% | 0.19% | 0.17% |  |  |
| Overall | -0.14% | 0.12% | 0.11% |  |  |
| Class D | -0.12% | -0.02% | -0.03% |  |  |
| Class F | -0.19% | 0.12% | 0.15% |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | 0.50% | 0.67% |  |  |
| Class A2 | -0.24% | 0.27% | 0.29% |  |  |
| Class B | -0.28% | 0.51% | 0.62% |  |  |
| Class C | -0.23% | -0.11% | 0.39% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.25% | 0.30% | 0.50% |  |  |
| Class D | -0.20% | 0.01% | 0.03% |  |  |
| Class F | -0.19% | -0.13% | -0.39% |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | 1.21% | 1.28% |  |  |
| Class C | -0.30% | 0.22% | 0.58% |  |  |
| Class E | -0.54% | 0.54% | 0.01% |  |  |
| Overall | -0.34% | 0.71% | 0.73% |  |  |
| Class D | -0.34% | 0.47% | 0.92% |  |  |
| Class F | -0.36% | 0.37% | 0.11% |  |  |

Table 3 Results of Test-4.3b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.13% | 0.02% |  |  |
| Class A2 | -0.10% | 0.07% | 0.06% |  |  |
| Class B | -0.11% | 0.05% | 0.11% |  |  |
| Class C | -0.12% | -0.04% | 0.06% |  |  |
| Class E | -0.15% | 0.23% | -0.03% |  |  |
| Overall | -0.13% | 0.07% | 0.05% |  |  |
| Class D | -0.09% | -0.05% | -0.15% |  |  |
| Class F | -0.24% | 0.03% | -0.03% |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | 0.12% | -0.03% |  |  |
| Class A2 | -0.34% | 0.07% | -0.15% |  |  |
| Class B | -0.60% | 0.25% | 0.04% |  |  |
| Class C | -0.52% | -0.14% | 0.27% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.51% | 0.08% | 0.05% |  |  |
| Class D | -0.62% | -0.19% | -0.37% |  |  |
| Class F | -0.44% | -0.51% | -0.36% |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | 0.99% | 1.71% |  |  |
| Class C | -0.74% | 0.25% | 0.46% |  |  |
| Class E | -1.23% | 0.59% | -0.45% |  |  |
| Overall | -0.85% | 0.64% | 0.75% |  |  |
| Class D | -1.28% | 0.53% | 1.61% |  |  |
| Class F | -0.82% | -0.44% | -0.65% |  |  |

# Conclusion

The contribution presents the results of EE2-Test-4.3 which is the combination of EE2-Test-4.1 and EE2-Test-4.2. Two combined tests, i.e., Test-4.3a and Test-4.3b are conducted. Simulation results show that both combinations provide promising coding performance improvements and it is recommended to adopt one solution into ECM.

# References

- X. Xiu, W. Chen, C.-W. Kuo, H.-J. Jhu, X. Wang, “AHG12: Improved probability estimation for CABAC”, JVET-Y0157, Feb. 2022.

- V.Seregin, J. Dong, N. Hu, M. Karczewicz, “AHG12: CABAC initialization from previous inter slice”, JVET-Y0181, Feb. 2022.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Apr. 2022.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

- V. Seregin, H. Golestani, N. Hu, P. Garus, M. Karczewicz, “AHG12: Improved probability estimation for CABAC”, JVET-Z0133, April 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).