| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0137-v1 |
| --- | --- |

| Title: | Non-EE2: Adaptive Blending for GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Tel: Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, one CU-level adaptive blending method is proposed to improve the prediction efficiency of geometric partition mode (GPM). For the natural content, the proposed method shows {Y, U, V} BD-rate impacts of {-0.14%, -0.25%, -0.12%} for RA and {-0.17%, -0.04%, 0.02%} for LDB, compared to ECM4.0 anchors. For the TGM content, the BD-rate performance of the proposed method is {-1.90%, -1.30%, -1.26%} for RA and {-2.61%, -2.11%, -2.12%} for LDB. The corresponding impacts on encoding and decoding run-time are 101% and 100% for RA and 102% and 100% for LDB.

# Introduction

During the VVC development, the Geometric Partitioning Mode (GPM) was adopted by [1]. During the ECM development, GPM was further enhanced by, e.g., GPM+TM [2], GPM+MMVD [2], and Inter+Intra GPM [3]. However, the blending strength of GPM has not been improved since the original GPM design. That is, as shown in Fig. 1, the blending strength or blending area width θ is fixed for all different contents.

Fig. 1 GPM blending in the current ECM 4.0

The weighing values in the blending mask can be given by a ramp function

With a fixed θ=2 pel in the current ECM (VVC) design, this ramp function can be quantized as

It is asserted that such design may not always optimal because the fixed blending area width cannot always provide the best blending quality for various types of video contents. For example, screen video contents usually contain strong textures and sharp edges, which refers a narrow blending area to reserve the edge information. For camera-captured content, blending is generally required; but the blending area width is dependent on a number of factors, e.g., the actual boundaries of the moving objects and the motion distinctiveness of two partitions.

# Proposed Method

To resolve the abovementioned issue, in this contribution, one adaptive blending scheme is proposed for the GPM, which dynamically adjusts the width of the blending area surround the GPM partition boundary. Specifically, in the proposed scheme, the width of the blending area (i.e., θ) is allowed to be selected from a set of pre-defined values {0, 1, 2, 4, 8 }. The optimal blending area width is determined for each GPM CU at encoder and signaled to decoder based on one syntax element merge_gpm_blending_width_idx. In the proposed method, all predefined blending strength value is shiftable while all the clipping and shifting operations in the existing GPM blending process can be kept without any changes

In addition, the range of the weights is increased from [0, 8] to [0, 32] to accommodate the increased width of the GPM blending area. Specifically, the weights are calculated as

# Simulation Results

Table 1 shows the simulation results of the proposed method on top of ECM4.0. For national content it shows average luma BD-rate savings of 0.14% and 0.17% for RA and LDB, respectively. For TGM sequences, the corresponding BD-rate savings are 1.90% and 2.61% for RA and LDB.

Table 1 Simulation results for the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.58% | -0.14% | 102% | 99% |
| Class A2 | 0.02% | -0.03% | 0.02% | 98% | 100% |
| Class B | -0.13% | -0.23% | -0.16% | 103% | 100% |
| Class C | -0.28% | -0.18% | -0.14% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.25% | -0.12% | 101% | 100% |
| Class D | -0.18% | -0.28% | 0.05% | 100% | 102% |
| Class F | -0.62% | -0.68% | -0.57% | 105% | 98% |
| Class TGM | -1.90% | -1.30% | -1.26% | 101% | 98% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | 0.04% | 0.09% | 103% | 101% |
| Class C | -0.31% | -0.05% | -0.54% | 105% | 102% |
| Class E | -0.08% | -0.18% | 0.66% | 99% | 98% |
| Overall | -0.17% | -0.04% | 0.02% | 102% | 100% |
| Class D | -0.30% | 0.05% | -0.12% | 102% | 102% |
| Class F | -0.69% | 0.09% | -0.88% | 102% | 99% |
| Class TGM | -2.61% | -2.11% | -2.12% | 102% | 98% |

Additionally, Table 2 shows the simulation results of Class F and TGM sequences, where one high-level control flag is introduced in PPS to completely disable the blending process (which is equivalent to set θ to 0) for the GPM CUs.

Table 2 Simulation results of Class F and TGM sequences when one PPS flag is signaled to disable the blending for the GPM

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.31% | -0.18% | 0.06% | 100% | 99% |
| Class TGM | -2.47% | -1.80% | -1.69% | 99% | 99% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.18% | 0.56% | 0.02% | 100% | 100% |
| Class TGM | -3.52% | -2.67% | -2.49% | 100% | 100% |

# References

[1] H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, R.-L. Liao, J. Chen, Y. Ye, J. Luo, K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang, J. Li, C. S. Lim, Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, L.-F. Chen, X. Li, G. Li, S. Liu, L. P. Van, G. V. d. Auwera, A. K. Ramasubramonian, H. Huang, W.-J. Chien, M. Karczewicz, M. Bläser, J. Sauer, H. Chen, and H. Yang, “Integrated Text for GEO,” JVET, Brussels, BE, 17th meeting, document JVET-Q0806, Jan. 2020

[2] X. Xiu, C.-W. Kuo, X. Wang, R.-L. Liao, Y. Ye, X. Li, J. Chen, Z. Deng, K. Zhang, L. Zhang, N. Zhang, Y. Wang, Y.-J. Chang, H. Huang, V. Seregin, C.-C. Chen, M. Karczewicz, “EE2-related: Combination of EE2-3.3, EE2-3.4 and EE2-3.5,” JVET Teleconference, 23rd meeting, document JVET-W0097, Jul. 2021

[3] Y. Kidani, H. Kato, K. Kawamura, H. Jang, S. Kim, J. Lim, Z. Deng, K. Zhang, L. Zhang, “EE2-3.1: GPM with inter and intra prediction (JVET-X0166)” JVET Teleconference, 25th meeting, document JVET-Y0065, Jan. 2022

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).