| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0148-v1 |
| --- | --- |

| Title: | EE2-2.5 related: AMVP with MVP refinement and ARMC with refined motion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou, Zhuoyi Lv, Jinrong Zhang Yang Wang, Kai Zhang, Na Zhang, Zhipin Deng, Li Zhang | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com jinrong.zhang@vivo.com wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | vivo Mobile Communication Co., Ltd., Bytedance Inc. |  |  |

_____________________________

- Abstract

In this contribution, MVP refinement for regular AMVP mode is introduced on top of the changes in JVET-Z0145 [1], which was firstly reported in JVET-Y0091 [2]. Before using template-based searching to refine the MVP, two changes have been made. First, two out of five candidates are chosen from the candidate list by comparing the template matching costs. Second for each of the two selected candidates, template-based searching process is operated with a search pattern indicated by MMVD to derive an initial refined MVP with minimum template matching cost. Then the template-based searching process in ECM is conducted to the initial refined MVP.

On top of the ECM4.0, simulation results of the proposed method together with the modifications in EE2-2.5b are reported as below:

RA:

On top of the ECM4.0, simulation results of the proposed method together with the modifications in EE2-2.5c are reported as below:

RA:

Introduction

Adaptive reordering of merge candidates with refined motion was established as Test-2.5 in JVET-Y EE2 and test results are reported in JVET-Z0058. In ARMC with refined motion, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR first and the refined motion is used in ARMC to reorder the merge candidate list. ARMC with refined motion is applied to template matching merge (TM merge) mode, adaptive DMVR mode, and template matching for advanced motion vector prediction (TM-AMVP) mode. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering. ARMC with refined motion is tested in Test #2.5a. while a simplified version is tested in Test #2.5b. In Test #2.5b, when template matching is used to derive the refined motion, the template size is set equal to 1. And only the first eight merge candidates are reordered with the refined motion in TM merge mode.

Based on Test #2.5b, more simplified changes are proposed in JVET-Z0145 (i.e., Test #2.5c). First, only the above or left template is used during the motion refinement of TM when the block is flat or narrow. Second, only the first four merge candidates are reordered in TM merge mode instead of the first eight merge candidates in Test #2.5b. Third, when constructing the AMVP list, an MVP candidate with a TM cost larger than a threshold is skipped. Fourth, TM is extended to perform 1/16-pel MVD precision in TM merge mode.

- Proposed methods

After the AMVP list construction, an MVP is derived as the following steps:

Step 1. Select one or two candidates with one or two smallest TM costs from AMVP list.

Step 2. Extend the two candidates by a search pattern where the search positions are indicated by MMVD and derive TM costs for these extended MVPs. The search pattern is illustrated in Fig. 1, where 8 directions and 8 positions along each direction are defined in Table 1.

Table 1 - The offset distance

| distIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance (1/4-pel) | 1 | 2 | 4 | 8 | 12 | 16 | 24 | 32 |

Figure 1. Search pattern

Step 3. If the minimum TM cost for the searched points is less than either of the TM costs of candMv0 or candMv1, operate TM for the MVP with the minimum TM cost in the same manner in ECM.

To reduce the complexity, the simplification is proposed as follows and the performance is shown in Test 2: Only if the motion vector resolution of current CU is 1/4-pel or 1-pel, the size of CU is less than or equal to 128, and NoBackwardPredFlag equals to 1, the proposed method is applied.

In step 2, set each of the two selected candidates (candMv0 and candMv1) as starting point respectively, and operate template-based searching at the search positions indicated by distIdx = {0, 2, 4, 6} along 8 directions to derive the corresponding TM costs for each search position. If thus far the minimum TM cost for the searched points is larger than or equal to either of the TM costs of candMv0 or candMv1, the template-based searching process is terminated. Otherwise, if the searched point with the minimum TM cost is starting with candMv0 (or candMv1) as the center, then only the remaining distIdx = {idx-1, idx+1} (The index of idx with the minimum TM cost from last iteration is used as the center index) along 8 directions around candMv0 (or candMv1) are searched.

- Simulation results

The proposed method together with the modifications in EE2-2.5c and EE2-2.5b were implemented on top of the ECM-4.0 [3] and tested using the common test conditions as defined in JVET-Y2017 [4]. Table 1 shows the overall BD-rate performance.

Table 1. Combination of EE2-2.5b and proposed method on top of ECM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.15% | -0.21% | -0.16% | 105% | 106% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.11% | -0.06% | -0.17% | 106% | 106% |
| Class F |  |  |  |  |  |

Table 2. Combination of EE2-2.5c and proposed method on top of ECM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.15% | -0.17% | -0.14% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.13% | -0.30% | -0.36% |  |  |
| Class F |  |  |  |  |  |

- Conclusion

This contribution reports the results of EE2-2.5-related method, AMVP with MVP refinement and ARMC with refined motion. With the proposed methods, 0.xx% gain and 0.xx% gain can be achieved in RA and LDB configurations, respectively. It is recommended to adopted this related method into the next version of ECM software.

- References

[1] Y. Wang, K. Zhang, N. Zhang, Z. Deng, L. Zhang, “EE2-2.5 related: More test results for ARMC with refined motion”, JVET-Z0145, Feb. 2022.

[2]	C. Zhou, Z. Lv, J. Zhang, “Non-EE2: MVP refinement for regular AMVP mode,” JVET-Y0091, January 2022.

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, January 2022.

Patent rights declaration(s)

Bytedance Inc. and vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).