| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0154-v1 |
| --- | --- |

| Title: | EE1-1.6-related: Improved RDO Considering Deep In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li1, Yue Li2, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | lijunru@bytedance.com yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

# Abstract

This contribution presents an encoder optimization technique on top of the deep learning-based in-loop filtering method in EE1-1.6. The proposed method is asserted to improve the compression efficiency at encoder by introducing CNN-based filtering into the rate-distortion optimization (RDO) process. Compared with VTM11.0_nnvc, the proposed method reportedly shows on average {8.85%, 21.61%, and 22.73%}, {11.55%, 23.15%, and 24.62%}, and {x%, x%, x%} BD-rate reductions for {Y, Cb, Cr} components, under AI, RA, and LDB configurations, respectively.

# Introduction

A deep learning-based in-loop filtering method is presented in JVET-Z0112 [1]. However, the filtering procedure is not considered in the RDO process. In this proposal, CNN-based filtering is integrated into the RDO process for a better estimation of RD cost when deep learning-based in-loop filtering is used.

# Proposed method

It is proposed that CNN-based filtering is involved during the partitioning mode selection. In particular, the samples obtained after CNN-based filtering are compared with original samples to calculate the distortion. The optimal partitioning mode is then selected based on the refined rate-distortion (RD) cost.

To reduce the complexity incurred from applying CNN-based filtering during RDO stage, several fast algorithms are proposed. First, a simplified version of CNN models is additionally trained and used in the RD stage where the simplified models are same with that in JVET-X0084 [2]. The CNN architecture used in the RDO process is presented in Fig. 1. Second, only one filter is included in the RDO process without considering filter selection. Finally, the proposed technique is only applied to the coding units with height and width no larger than 64.

Fig. 1. (a) Architecture of the CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a)

As shown in the subsection 2.1 and 2.2, the inference and training processes of models are same as those in JVET-X0084.

## Inference

PyTorch is used for performing the inference of the proposed CNN filters in VTM. The network information in the inference stage is provided in Table 1 as suggested by [3].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 5.53M/model, 24 models |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~22.1MB/model，24 models in total |
|  | MAC (Giga) | 1.43M/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 69+64 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 32×32, 64×64, 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Training

PyTorch is used as the training platform. The DIV2K [4] and BVI-DVC [5] datasets are adopted to train the CNN filters of I slices and B slices, respectively. Different CNN models are trained to adapt to different QP points. The network information in the training stage is provided in Table 2 as suggested by [3].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed method is tested according to the common test conditions defined in [3]. The test results on top of VTM-11.0_nnvc [6] are shown in Table 3 ~ Table 5. Under AI configurations, the proposed method can bring 8.85%, 21.61%, and 22.73% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 11.55%, 23.15%, and 24.62% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the proposed method on top of VTM-11.0_nnvc (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.71% | -17.84% | -21.23% | 243% | 71154% |
| Class A2 | -12.34% | -23.08% | -25.48% | 240% | 67696% |
| Class B | -11.17% | -25.71% | -25.54% | 239% | 66446% |
| Class C | -12.05% | -23.96% | -25.35% | 219% | 64809% |
| Class E |  |  |  |  |  |
| Overall | -11.55% | -23.15% | -24.62% | 235% | 67165% |
| Class D | -13.44% | -25.56% | -25.91% | 238% | 61006% |
| Class F | -6.19% | -13.53% | -14.14% | 313% | 35448% |

Table 4. Performance of the proposed method on top of VTM-11.0_nnvc (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -10.44% | -17.03% | -17.02% | 216% | 62314% |
| Class E | -8.21% | -17.97% | -17.42% | 436% | 53085% |
| Overall |  |  |  |  |  |
| Class D | -11.61% | -21.17% | -19.35% | 221% | 52207% |
| Class F |  |  |  |  |  |

Table 5. Performance of the proposed method on top of VTM-11.0_nnvc (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -7.77% | -19.83% | -22.72% | 262% | 50005% |
| Class A2 | -8.00% | -21.93% | -21.48% | 210% | 41299% |
| Class B | -8.06% | -22.59% | -23.23% | 208% | 38360% |
| Class C | -9.18% | -20.35% | -23.37% | 189% | 26683% |
| Class E | -11.69% | -23.14% | -22.29% | 206% | 44978% |
| Overall | -8.85% | -21.61% | -22.73% | 211% | 38450% |
| Class D | -8.96% | -20.70% | -22.49% | 174% | 22562% |
| Class F | -5.41% | -13.75% | -13.52% | 154% | 27955% |

Compared with the EE1-1.6 [1], experimental results are shown in Table 6 ~ Table 8. Under AI configurations, the proposed method can bring 0.54%, 0.23%, and 0.29% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 0.62%, 0.28%, and 0.25% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 6. Performance of the proposed method on top of EE1-1.6(RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.20% | -0.11% | 103% | 100% |
| Class A2 | -0.67% | -0.39% | -0.28% | 106% | 99% |
| Class B | -0.67% | -0.33% | -0.38% | 102% | 100% |
| Class C | -0.66% | -0.21% | -0.17% | 108% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.62% | -0.28% | -0.25% | 105% | 100% |
| Class D | -0.61% | 0.07% | 0.22% | 127% | 108% |
| Class F | -0.49% | -0.10% | -0.21% | 94% | 103% |

Table 7. Performance of the proposed method on top of EE1-1.6 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.55% | -0.32% | 0.01% | 115% | 100% |
| Class E | -0.44% | 0.14% | -0.25% | 107% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.38% | 0.96% | -0.25% | 119% | 101% |
| Class F |  |  |  |  |  |

Table 8. Performance of the proposed method on top of EE1-1.6 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.27% | -0.20% | 130% | 99% |
| Class A2 | -0.51% | -0.55% | -0.75% | 137% | 101% |
| Class B | -0.53% | -0.30% | -0.30% | 145% | 99% |
| Class C | -0.60% | 0.14% | 0.02% | 145% | 96% |
| Class E | -0.65% | -0.25% | -0.30% | 130% | 102% |
| Overall | -0.54% | -0.23% | -0.29% | 138% | 99% |
| Class D | -0.53% | -0.04% | 0.27% | 143% | 95% |
| Class F | -0.36% | -0.10% | -0.13% | 125% | 99% |

# Conclusions

This contribution presents an encoder optimization method based on the CNN-based filtering method. The proposed optimization method shows promising coding gains.

# References

- EE1-1.6: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling based on SADL implementation, JVET-Z0112, Y. Li, K. Zhang, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz, Franck Galpin.

- EE1-related: Improved RDO Considering Deep In-Loop Filter, JVET-X0084, J. Li, Y. Li, K. Zhang, L. Zhang.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet-w-ee1/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).