| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0173-v1 |
| --- | --- |

| Title: | Cross-check of test 4.2c of JVET-Z0133 (EE2-4.2: CABAC initialization from previous inter slice and windows adjustment) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kazushi Sato | Tel: Email: | kazushi.sato@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This document provides crosscheck result of EE2: Test4.2 “Temporal CABAC initialization” + “Adaptive window size adjustment”.

Introduction / Problem Statement / …

Improved entropy coding methods are studied under EE4: Test 4. Among the experiments this report this report provides crosscheck information on the following test:

Test4.2c = Test 4.2a + Test 4.2b

(Test4.2a = Temporal CABAC initialization)

(Test4.2b = Adaptive window size adjustment)

Simulation Condition

Proposed method is implemented on top of ECM4.0 by the proponent.

The crosschecker inspected the code and asserted that the proposed method is successfully implemented.

The following macros are enabled for this test:

#define TEMP_CABAC                                        1 // test 4.2a

#define ADAPTIVE_WIN_SIZE                                 1 // test 4.2b

#define CONTEXT_INIT                                      1 // context retraining

#if TEMP_CABAC

#define TEMP_END                                          1 // store contexts at the end of picture

#endif

RA and LDB conditions are tested.

Simulation Result

Summary of the results are shown in Table 3.1 and Table 3.2, respectively. Complete results are shown in JVET-Z0173_Test4.2.xlsm. EncT/DecT are not reliable due to the problem of the cluster.

It is confirmed that the results matched the one provided by the proponent.

Table 3.1: Summary of the Result of RA Test

Table 3.2: Summary of the Result of LDB Test

Comment

For the CABAC initialization table initialization, 3 conditions should be met as:

- Same SliceType

- Same SliceQp

- Same Tid

Under the RA CTC conditions, 3) does not make any difference, as different sliceQPs are assigned for the different temporal layers.

However, if the GOP structure is same as the RA CTC conditions while sliceQPs for all the temporal layers are same, for example, whether there is condition 3) or not makes difference.

It is recommended that this proposal be further studied.

References

[1] “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan 2022

[2] “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Jan 2022

[3] “EE2-4.2: CABAC initialization from previous inter slice and windows adjustment”, JVET-Z0133, Apr 2022