| J oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0175-CTC-v1 |
| --- | --- |

| Title: | JVET common test conditions and evaluation procedures for HDR/WCG video |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. François W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@amazon.com edouard.francois@interdigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp dmytror@qti.qualcomm.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This [draft and work in progress] document defines common test conditions (CTC), conversion practices and software reference configurations to be used in the context of HDR/WCG video coding experiments after the 26th JVET meeting. These common test conditions are recommended for use in technical contributions to the 27th and following JVET meetings, as applicable.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and to ease the comparison of the outcome of experiments.

This document defines four test conditions for HDR/WCG content, reflecting intra-only, random-access, and low-delay settings:

- Intra, 10 bit

- Random access, 10 bit

- Low delay, 10 bit

- Low delay, P slices only, 10 bit

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

When using the VTM software: Version 16.0 of the VTM software is expected to be used for most experiments. More recent versions are encouraged where applicable. The VTM software can be accessed using the Git version control system and is available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git.

When using the HM software: Version 16.25 of the VTM software is expected to be used for most experiments. More recent versions are encouraged where applicable. The HM software can be accessed using the Git version control system and is available at https://vcgit.hhi.fraunhofer.de/jvet/HM.git.

The following sections define test sequences, quantization parameter values, evaluation processes, encoder configuration files, and compile-time options to be used.

Anyone bringing input contributions to JVET meetings should provide a set of results that is as complete as possible and uses the common test conditions that apply to the proposal.

# Test sequences

Table 1 and Table 2 define the sets of test sequences to be used for intra, random-access, and low-delay experiment conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences in the random-access and low-delay test cases described below. For the intra configuration, the first of every eight frames shall be encoded starting with the first frame in the sequence for all sequences, and bit rates shall be calculated using the sequence frame rate (as defined by frame rate in the table) divided by eight. Please note that temporal sub-sampling may be enabled in the HM and VTM software using the parameter TemporalSubsampleRatio.

The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the JVET CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the JVET CTC for the given configuration

- “-” indicates that the test sequence is not requested in the JVET CTC or the given configuration

Original versions of the test sequences in Table 1 and Table 2 are available on ftp://jvet@ftp.ient.rwth-aachen.de/ctc/hdr/.  (Properly qualified participants of JVET may please contact the JVET chairs for login information).  Additional information about the test sequences is provided in Annex D.

Table 1 Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Transfer Function | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H1 | BalloonFestival | 240 | 24 fps | 10 | PQ | M | M | - |
| H1 | Cosmos1_TreeTrunk | 240 | 24 fps | 10 | PQ | M | M | - |
| H1 | EBU_Hurdles | 500 | 50 fps | 10 | PQ | M | M | - |
| H1 | EBU_Starting | 500 | 50 fps | 10 | PQ | M | M | - |
| H1 | Market | 400 | 50 fps | 10 | PQ | M | M | - |
| H1 | ShowGirl | 339 | 25 fps | 10 | PQ | M | M | - |
| H1 | SunRise | 200 | 25 fps | 10 | PQ | M | M | - |

Note: The capture frame rate of the EBU_Hurdles and EBU_Starting sequences was 100fps. However, these sequences are treated as 50fps sequences for the evaluation processes defined in this document.

Table 2 Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Transfer Function | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H2 | DayStreet2 | 300 | 60 fps | 10 | HLG | M | M | - |
| H2 | FlyingBirds3 | 300 | 60 fps | 10 | HLG | M | M | - |
| H2 | PeopleInShoppingCenter2 | 300 | 60 fps | 10 | HLG | M | M | - |
| H2 | SunsetBeach3 | 300 | 60 fps | 10 | HLG | M | M | - |

# Quantization parameter values

Results shall be provided using the four quantization parameter values: 22, 27, 32 and 37. These values define the initial QP values that are specified for the I-frames in a sequence. Results for both class H1 and class H2 sequences shall be provided in this configuration.

# Configuration files

The following section defines encoder configuration files to be used for each test case. Parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system

- FrameRate to reflect the frame rate of a given sequence as per the tables above

- SourceWidth to reflect the width of the source video sequence

- SourceHeight to reflect the height of the source video sequence

- FramesToBeEncoded to reflect the frame count of a given sequence as per Tables above and as further defined in Section 3.

- IntraPeriod to reflect the intra refresh period in the random access test cases. The intra refresh period is dependent on the frame rate of the source and the GOP size in use: a value 32 shall be used for sequences with a frame rate equal to 20fps, 32 for 24fps, 32 for 30fps, 64 for 50fps, 64 for 60fps, and 96 for 100fps.

- QP to reflect the quantization parameter values defined in section 3.

When using the VTM software: Configuration files for the four test conditions are provided with the VTM software in the cfg directory and shall be used. The four configurations provided are as follows:

- “All Intra” (AI): encoder_intra_vtm.cfg

- “Random access” (RA): encoder_randomaccess_vtm.cfg

- “Low-delay B” (LB): encoder_lowdelay_vtm.cfg

- “Low-delay P” (LP, optional): encoder_lowdelay_P_vtm.cfg

When using the HM software: Configuration files for the four test conditions are provided with the VTM software in the cfg directory and shall be used. The four configurations provided are as follows:

- “All Intra” (AI): encoder_intra_main10_ctc_subsample.cfg

- “Random access” (RA): encoder_randomaccess_main10.cfg

- “Low-delay B” (LB): encoder_lowdelay_main10.cfg

- “Low-delay P” (LP, optional): encoder_lowdelay_P_main10.cfg

In addition to the four test conditions, class specific parameter files are also provided with the HM and VTM software in the cfg/per-class directory and shall be used.  These configuration files enable (or disable) coding tools as appropriate for HDR content.

Finally, sequence specific parameters are provided with the HM and VTM software in the cfg/per-sequence-HDR directory and shall used.

# Evaluation

Results reported for the H1 sequence class shall include the following metrics:

- PSNR for Y′, Cb and Cr

- wPSNR for Y’, Cb and Cr

- deltaE100

- PSNRL100

Results reported for the H2 sequence class shall include the following metrics:

- PSNR for Y’, Cb and Cr

When using the VTM software: The PSNR and wPSNR metrics shall be calucalted using the implementations available in the VTM.  The deltaE100 and PSNR-L100 metrics shall be calculated using the HDRLib functionality available in the VTM software.  More information about enabling the HDRLib functionality in the VTM is provided in Annex A.  Additional information for calculating wPSNR, deltaE100 and PSNR-L100 metrics is provided in Annex B.

When using the HM software: The PSNR metric shall be calculated using the implementation available in the HM.  The wPSNR, deltaE100 and PSNR-L100 metrics shall be calculated using the HDRTools software.  More information for using the HDRTools software is provided in Annex C.  The HDRTools software can be accessed using the Git version control system and is available at https://gitlab.com/standards/HDRTools.git.

# Parallel encoding/decoding

Parallel encoding/decoding as described in JVET-B0036 may be applied for RA configurations. If parallel encoding and decoding is used, this method shall be applied for both versions, the reference and the version under test. When reporting results, it shall also be reported whether parallel encoding/decoding was used or not.

Example scripts are provided with the HM and VTM software.

# Results Template

An excel template file for reporting test results is attached to this document.

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the common software. The default settings provided in the source code should be used.

# Annex A: Enabling HDRLib functionality in the VTM software

# Enabling the HDRLib functionality in the VTM software is accomplished by doing the following:

- Create a local copy of the VTM Version 6 or newer software.

- Create a local copy of the HDRTools v0.19.1 or newer utility available at https://gitlab.com/standards/HDRTools.  More recent versions are encouraged where applicable.

- Copy the HDRTools/common directory from the HDRTools utility into the source/Lib/HDRLib directory of the VTM software.

Example: On a Linux system, this could be accomplished with the command:

ln -s <HDRTools_dir>/HDRTools/common <VTM_dir>/souce/Lib/HDRLib

- Append “-DEXTENSION_HDRTOOLS=on” when building the VTM software using CMake.

Example: If the VTM is normally built using the command:

cmake -DCMAKE_BUILD_TYPE=Release

The HDRLib functionality is enabled by using the command:

cmake -DCMAKE_BUILD_TYPE=Release -DEXTENSION_HDRTOOLS=on

The class specific parameters and sequence specific parameters provided with the VTM will then enable and configure the metric calculation parameters appropriately.

# Annex B: Objective distortion metrics for HDR quality evaluations

The common test conditions described in this document requires a number of metrics to be reported for evaluation. In this Annex, further information about the wPSNR, deltaE100, and PSNR-L100 metrics are provided.

wPSNR

The wPSNR, or weighted PSNR, metric is calculated from the weighted mean squared error of the pixel values. An implementation of the metric is provided in the VTM software, and that implementation shall be used for calculation of the metric using the weighting functions provided below. It is noted that the HDRMetrics tool is capable of computing the weighted PSNR metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the VTM software.

As an informative description of the metric follows.

The wPSNR metric is calculated as:

wPSNR=10*logX2*nwMSE,

where X is the maximum pixel value for the specific bit depth, n is the number of samples, and wMSE is given as

wMSE= sampleswilumaxorig,i*(xorig, i-xdec,i)2,

where wilumaxorig,iis a weight that is a function of the luma value corresponding to the pixel xorig,i, xorig,i is the original value at location i, and xdec,i is the reconstructed value at location i.  The wPSNR metric is calculated separately for the luma and two chroma channels.

The calculation of the weight is computed as:

XL = bitDepth >= 10 ? luma(xorig,i) >> (bitDepth - 10) : luma(xorig,i) << (10 - bitDepth);

yi = 0.015*xL − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

where bitDepth is the bit depth of xorig.

The metric is calculated individually for a single luma or chroma channel and then used to compute a Bjøntegaard Delta-Rate and Delta-PSNR rate.

Note: The metric is defined for all bit-depths of xorig.  However, it is not anticipated that bit-depths less than 10-bits would be used in a practical HDR application.

deltaE100-based metric

An implementation of the deltaE100 distortion metric is provided in the VTM software when enabled with the HDRLib functionalty, and that implementation shall be used for calculation nof the metric. It is noted that the HDRMetrics tool is also capable of computing the deltaE100 distortion metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the VTM software.

An informative description of the metric follows.

The orignal and test material must first be coverted to linear-light 4:4:4 RGB EXR. For example, if the material is in the YCbCr BT.2100 4:2:0 PQ 10 bit format, it must be converted to a 4:4:4 RGB BT.2100 OpenEXR file using configuration provided with the HDRTools software.

Subsequently, the following steps should be applied for each (R,G,B) sample,within the content to be compared, i.e. original source (content 1) and test material (content 2) :

- Convert the content to the XYZ colour space

- Convert the content from the XYZ to Lab space using the following equations. Computations are performed using double floating point precision.

  - invYn = 1.0 ÷ Yn, with Yn = 100

  - invXn = invYn ÷ 0.95047

  - invZn = invYn ÷ 1.08883

  - yLab = convToLab( y * invYn )

  - L = 116.0 * yLab − 16.0

  - a = 500.0 * (convToLab(x * invXn) − yLab )

  - b = 200.0 * ( yLab − convToLab (z * invZn) )

with convToLab(x) is defined as

if x >= 0.008856

convToLab(x) = x(1÷3)

else

convToLab(x) = 7.78704 * x + 0.137931

- The deltaE100 distance DE between two samples (L1,a1,b1) and (L2,a2,b2) is then computed as follows:

cRef = sqrt( a1 * a1 + b1 * b1 )

cIn  = sqrt( a2 * a2 + b2 * b2 )

cm = (cRef + cIn) ÷ 2.0

g = 0.5 * ( 1.0 − sqrt( cm7.0 ÷ ( cm7.0 + 257.0 ) ) )

aPRef = ( 1.0 + g ) * a1

aPIn  = ( 1.0 + g ) * a2

cPRef = sqrt( aPRef * aPRef + b1 * b1 )

cPIn  = sqrt( aPIn * aPIn + b2 * b2 )

hPRef = arctan( b1, aPRef )

hPIn  = arctan( b2, aPIn )

deltaLp = L1 − L2

deltaCp = cPRef − cPIn

deltaHp = 2.0 * sqrt( cPRef * cPIn ) * sin( (hPRef − hPIn) ÷ 2.0 )

lpm = ( L1 + L2 ) ÷ 2.0

cpm = ( cPRef + cPIn ) ÷ 2.0

hpm = ( hPRef + hPIn ) ÷ 2.0

rC = 2.0 * sqrt( cpm7.0 ÷ ( cpm7.0 + 257.0 ) )

dTheta = DEG30 * exp(−((hpm − DEG275) ÷ DEG25) * ((hpm − DEG275) ÷ DEG25))

rT = −sin( 2.0 * dTheta ) * rC

t = 1.0 − 0.17 * cos(hpm − DEG30) + 0.24 * cos(2.0 * hpm) +

0.32 * cos(3.0 * hpm + DEG6) − 0.20 * cos(4.0 * hpm − DEG63)

sH = 1.0 + ( 0.015 * cpm * t )

sC = 1.0 + ( 0.045 * cpm )

sL = 1.0 + ( 0.015 * (lpm − 50) * (lpm-50) ÷ sqrt(20 + (lpm − 50) * (lpm − 50)) )

deltaLpSL = deltaLp ÷ sL

deltaCpSC = deltaCp ÷ sC

deltaHpSH = deltaHp ÷ sH

deltaE100 = sqrt( deltaLpSL * deltaLpSL + deltaCpSC * deltaCpSC +

deltaHpSH * deltaHpSH + rT * deltaCpSC * deltaHpSH )

with

DEG275 = 4.7996554429844

DEG30 = 0.523598775598299

DEG6 = 0.1047197551196598

DEG63 = 1.099557428756428

DEG25 = 0.436332

After this process the deltaE100 values for each frame are averaged within the “Distortion” specified window. Finally, a PSNR based value is derived as:

PSNR_DE = 10 * log10( PeakValue ÷ deltaE100 )

where PeakValue is set to 10,000 for sequences in the PQ format and 1,000 for sequences in the HLG format.

PSNR-L100

An implementation of the PSNR-L100 distortion metric is provided in the VTM software when enabled with the HDRLib functionalty, and that implementation shall be used for calculation nof the metric. It is noted that the HDRMetrics tool is also capable of computing the PSNR-L100 distortion metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the VTM software.

An informative description of the metric follows:

PSNR-L100 represents the distortion in the lightness domain of the CIELab colour space. The derivation of Lab values from the linear representation of the signal is similar as given in the description of deltaE100. The mean absolute error (MAE) in the L domain is used to compute the PSNR-L100 as follows:

PSNR-L100 = 10 * log10( PeakValue ÷ MAE )

where PeakValue is set to 10,000 for sequences in the PQ format and 1,000 for sequences in the HLG format.

# Annex C: Using HDRTools to compute metrics with the HM software

# The following commands will compute the HDR metrics when using the HM software.  Version 0.23 of the HDRTools software is expected to be used for the calculations.  More recent versions are encouraged where applicable.:

- Convert the decoded YUV to the EXR format using the following command:

hdrConvert -f <HDRTools_Directory>/bin/JCTVC_CTC_cfgFiles/YCbCr/HDRConvertYCbCr420ToEXR2020.cfg -p SourceFile=<source.yuv> -p OutputFile=<decoded.exr> -p SourceSampleRange=0 -p SourceHeight=<height> -p SourceWidth=<width> -p OutputHeight=<height> -p OutputWidth=<width> -p NumberOfFrame=<num_frames>

- Convert the original YUV to the EXR format using the following command:

hdrConvert -f <HDRTools_Directory>/bin/JCTVC_CTC_cfgFiles/YCbCr/HDRConvertYCbCr420ToEXR2020.cfg -p SourceFile=<source.yuv> -p OutputFile=<original.exr> -p SourceSampleRange=0 -p SourceHeight=<height> -p SourceWidth=<width> -p OutputHeight=<height> -p OutputWidth=<width> -p NumberOfFrame=<num_frames>

- Calculate the deltaE100 and PSNRL100 metrics using the following command:

hdrMetrics -f <HDRTools_Directory>/bin/JCTVC_CTC_cfgFiles/YCbCr/HDRMetric_CfE.cfg -p Input0File=<original.exr> -p Input1File=<decoded.exr> -p Input0Height=<height> -p Input0Width=<width> -p Input1Height=<height> -p Input1Width=<width> -p NumberOfFrames=<num_frames>

Note that when calculating the deltaE100 and PSNRL100 metrics for the ShowGirl sequence, the config file HDRMetric_CfE_ShowGril.cfg should be used instead of HDRMetric_CfE.cfg.  This removes the boundary pixels from the metric calculation.

Note that Version 0.23 of the HDRMetric utility may produce different values of the metric on different platforms when a non-integer framerate is provided.  To ensure consistent performance of the HDRMetric utility across different platforms, use of an integer frame rate input parameters is recommended.  The value of the frame rate parameter should not affect results of HDRMetric computation.

# Annex D: Additional description of test sequences

Table B-1 – H1 test sequence example pictures

| BalloonFestival | Cosmos1_TreeTrunk | EBU_Hurdles | EBU_Starting |
| --- | --- | --- | --- |
| Market | ShowGirl | SunRise |  |

Table B-2 – H2 test sequence example pictures

| DayStreet2 | FlyingBirds3 | PeopleInShoppingCenter2 | SunsetBeach3 |
| --- | --- | --- | --- |

Table B3 – HDR test sequence md5sums

| Sequence name | MD5Sum |
| --- | --- |
| BalloonFestival | a7831be36b65645ad1963bd367370141 |
| Cosmos1_TreeTrunk | da4a2488c249720da0535f01c3693efa |
| EBU_Hurdles | bc3cba849d6f4ee74d39056600722aa5 |
| EBU_Starting | 1cbc416696cb0dfcf4da9886eeb6a4a2 |
| Market | c97abe47455fd12f6d6436cecfad7c7d |
| ShowGirl | 44f1974d68f7799c71eea29fb72b245b |
| Sunrise | 420202b3390998c9148bf5958501186d |
| DayStreet2 | 9514f52b27ee9b7bcfcfdd4bbcdbd11c |
| FlyingBirds3 | 5e792f14d966ee08f1ac5c132cacb019 |
| PeopleInShoppingCenter2 | d029d8e202f0f8611b4cc6e67d1e78b0 |
| SunsetBeach3 | e103c6455ce64c8dc693fa7192f20219 |

Note: The sequences DayStreet2, PeopleInShoppingCenter2 and SunsetBeach3 are distributed as 600 frame files, even though only 300 frames are required by this CTC and specified in Table 2.  The MD5Sum values reported here correspond to the 600 frame files.

The HDRTools utility and VTM software are maintained separately and not released according to the same schedule.  Thus, it is possible that a version of the utilities newer than v0.19.1 may not be compatible with a given version of the VTM software (and, equivalently, a newer version of the VTM software may not be compatible with a given version of the HDRTools utility). It is recommended to use the most recent version of the HDRTools utility when possible.  Otherwise, if there are compatibility issues between the most recent version of the HDRTools and the VTM software, it is then recommended to use v0.19.1.

The wPSNR metric is available in the VTM software beginning with VTM 1.1.  It is not included in the VTM 1.0 release.

For some versions of HDRMetrics, this is accomplished by using the following settings; -p EnableWTPSNR=1 -p Enable JVETPSNR=1 -p WeightTableFile=”hdrTable.txt”

As defined above, reporting the deltaE100, PSNR-L100 and wPSNR metrics is not required for HLG content.  This example is provided solely for completeness.