| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0228-v1 |
| --- | --- |

| Title: | Crosscheck of RPR enabling fix in JVET-Z0062 (AHG12: RPR luma filter modifications and RPR enabling fixes in ECM), JVET-Z0067 (Non-EE2: RPR bug fixes and new results of RPR with luma-only re-scaling) and JVET-Z0079 (Non-EE2: Support RPR on ECM-4.0 and handling method for template matching based coding tools) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking of RPR enabling fix in JVET-Z0062 proposed by Ericsson, JVET-Z0067 proposed by InterDigital and JVET-Z0079 proposed by LG Electronics Inc. There is a merge request 105 (MR105), which is to fix RPR enabling in ECM-4.0. The contribution JVET-Z0067 proposes the RPR enabling fix exactly same as MR105. The contributions JVET-Z0062 and JVET-Z0079 proposes RPR enabling fix with several different aspects comparing to MR105. This document provides analysis of the RPR enabling fix and presents a simulation result using the MR105 as anchor.

The update of the code in each contribution had been reviewed and confirmed consistent with the proposal description.

The verification simulation was run for RA configuration with RPR enabled.

# Verification

It is observed encoder crash in ECM-4.0 when RPR is enabled due to the adoption of tools in ECM-4.0. The contributions JVET-Z0062, JVET-Z0067 and JVET-Z0079 claim the issue in ECM-4.0 for RPR enabling. The following table 1 analysis the claimed issues and the proposed fix from each contribution.

Table 1. RPR enabling issues in ECM-4.0 claimed by JVET-Z0062, Z0067 and Z0079

| Issue claimed in contributions | JVET-Z0062 | JVET-Z0067 (MR105) | JVET-Z0079 |
| --- | --- | --- | --- |
| JVET_Y0065_GPM_INTRA Fetching of motion information and IPM (intra prediction mode) information from reference picture do not consider resolution difference between the reference picture and current picture. | Coordinates scaling according to the resolution ratio. Apply scalePositionInRef() | Same as Z0062 | Same as Z0062 |
| ALF_IMPROVEMENT The SIMD code in function simdFilter9x9Blk() does not support input width being non-dividable by 8. This scenario can happen for boundary chroma CTB for class D when using 1.5x scaling. | In the SIMD function simdFilter9x9Blk(), when the width of the input block is non-dividable by 8, Non-SIMD version of the ALF filtering code is invoked. | None | None |
| JVET_Y0067_ENHANCED_MMVD_MVD_SIGN_PRED a)The template based MVD sign derivation is enabled regardless of resolution difference between the reference picture and the current picture. b) In MMVD candidate reordering (function sortInterMergeMMVDCandidates() and function sortAffineMergeCandidates()), the prediction sample generation for templates does not consider reference picture resolution difference from the current picture resolution. | a) Add reference picture and current picture resolution checks for the template based MVD sign derivation. When the reference picture is determined to not be scaled, the template based MVD sign derivation is enabled. When the reference picture is determined to be scaled, the MVD sign information is signaled and parsed using the conventional VVC approach. b) In MMVD candidate reordering, the corresponding template cost for the candidate is set to 0. (fix for this item is as below, different cost value setting for resampled cands) | The estimation of the candidate costs for Inter Merge MMVD, Affine Merge and MVD/SMVD sign is by-passed (use default max cost) if the current picture is re-scaled. | MVD sign prediction is disabled and sign syntax for MVD is signalled after magnitude not sign-pair index for the reference picture list. Same as Z0062 |
| JVET_Y0134_TMVP_NAMVP_CAND_REORDERING In merge candidate reordering inside a same candidate type group (function adjustMergeCandidatesInOneCandidateGroup()), the prediction sample generation for templates does not consider reference picture resolution difference from the current picture resolution. | If reference picture is resampled, the corresponding TM cost for the candidate is set to 0. | None | Same as Z0062. Except the values of TM cost set to MAX |
| MMVD and Affine MMVD handling JVET-Y0067 TM based reordering for MMVD and affine MMVD and MVD sign prediction | After construction of the candidate list, If the reference picture of the base candidate for the MMVD or Affine MMVD is resampled, the TM cost is set to 0 | Similar to Z0062. Except set TM cost to MAX_UINT>>1 | Similar to Z0062. Except TM cost set to MAX |
| ARMC handling The fix of JVET-Y0128 set the TM cost to 0 when resampling, this leads the resampling candidate prior to other cands | None | None | Modify the TM cost to MAX instead of 0 |

The simulation was conducted under the common test conditions as defined in [4] with RPR enabling configuration. The anchor is ECM-4.0 with MR105. The RPR configuration is set as in table 2.

Table 2. RPR configuration setting

(Resolution switch after every 16 frames)

ScalingRatioHor : 2.0

ScalingRatioVer : 2.0

FractionNumFrames : 1.0

RPR : 1

UpscaledOutput : 2

SwitchPocPeriod : 16

The simulation results for random access configuration are as follows:

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RPR fix in JVET-Z0062 Over ECM4.0 + MR105 |  |  |  |  | RPR fix in JVET-Z0079 Over ECM4.0 + MR105 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.67% | 0.60% | 0.64% | 101% | 96% | 0.32% | 0.39% | 0.43% | 104% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.59% | 0.79% | 0.59% | 100% | 96% | 0.22% | 0.40% | 0.49% | 104% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Reference

[1] R. Yu, K. Andersson (Ericsson), “AHG12: RPR luma filter modifications and RPR enabling fixes in ECM”, JVET-Z0062, Apr, 2022

[2] P. Bordes, F. Galpin, H. Guermoud, E. François (InterDigital), “Non-EE2: RPR bug fixes and new results of RPR with luma-only re-scaling”, JVET-Z0067, Apr, 2022

[3] H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE), “Non-EE2: Support RPR on ECM-4.0 and handling method for template matching based coding tools”, JVET-Z0079, Apr, 2022

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).