| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0250-v1 |
| --- | --- |

| Title: | Alternate proposed fix for ticket #1547 and crosscheck of JVET-Z0249 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposed draft text |  |  |
| Author(s) or Contact(s): | Jonathan Gan Yue Yu | Tel: Email: | v-jonathan.gan@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

VVC bug ticket #1547 reports mismatch between the VVC specification and the VTM reference software for the semantics of the sb_coded_flag syntax element. The mismatch was confirmed on the bug tracker by multiple experts, and it was agreed preferable for the specification to be aligned with the software.

JVET-Z0249 was uploaded as a late contribution to address this issue. However, this contribution asserts that there are some errors in the proposed text of JVET-Z0249. This contribution proposes alternative text which is asserted to fix the issue of ticket #1547 without introducing new inconsistencies.

# Introduction

VVC bug ticket #1547 [1] reports mismatch between the VVC specification and the VTM reference software for the semantics of the sb_coded_flag syntax element. The mismatch was confirmed by multiple independent parties, and furthermore agreed that the mismatch is due to a mistake in text integration during the 14th (“N”) JVET meeting, and not desired decoder behaviour. Further comment was made on the bug tracker that it is estimated most implementations and bitstreams existing prior to this bug report have based their behaviour on VTM. Therefore, it was asserted an unreasonable burden to require all these instances and products in development to align with the specification.

The VVCv1 specification contains the syntax element sb_coded_flag which is defined both for regular residual coding and transform skip residual coding. It is asserted that during integration of TSRC into the draft specification at the 14th (“N”) JVET meeting, the decoding behaviour of sb_coded_flag for RRC was incorrectly modified. The syntax and semantics of sb_coded_flag from JVET-T2001 [2] are reproduced below for reference:

for( i = lastSubBlock; i >= 0; i− − ) {

startQStateSb = QState

xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ]

yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ]

inferSbDcSigCoeffFlag = 0

if( i < lastSubBlock && i > 0 ) {

sb_coded_flag[ xS ][ yS ]

inferSbDcSigCoeffFlag = 1

}

sb_coded_flag[ xS ][ yS ] specifies the following for the subblock at location ( xS, yS ) within the current transform block, where a subblock is an array of transform coefficient levels:

When sb_coded_flag[ xS ][ yS ] is equal to 0, all transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

When sb_coded_flag[ xS ][ yS ] is not present, it is inferred to be equal to 1.

In comparison, the semantics of the corresponding syntax element (coded_sub_block_flag) from JVET-N1001-v6 [3] (prior to TSRC integration) are:

coded_sub_block_flag[ xS ][ yS ] specifies the following for the subblock at location ( xS, yS ) within the current transform block, where a subblock is a (4x4) array of 16 transform coefficient levels:

- If coded_sub_block_flag[ xS ][ yS ] is equal to 0, the 16 transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (coded_sub_block_flag[ xS ][ yS ] is equal to 1), the following applies:

- If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the 16 sig_coeff_flag syntax elements is present for the subblock at location ( xS, yS ).

- Otherwise, at least one of the 16 transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

When coded_sub_block_flag[ xS ][ yS ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, coded_sub_block_flag[ xS ][ yS ] is inferred to be equal to 1:

- ( xS, yS ) is equal to ( 0, 0 ).

- ( xS, yS ) is equal to ( LastSignificantCoeffX  >>  2, LastSignificantCoeffY  >>  2 ).

- Otherwise, coded_sub_block_flag[ xS ][ yS ] is inferred to be equal to 0.

The semantics for coded_sub_block_flag in JVET-N1001-v6 are identical to its semantics in HEVC [4]. It is asserted that the VTM software implements these semantics.

In HEVC, subblocks preceding the subblock containing the last significant coefficient in reverse scan order (ie, subblocks for i > lastSubBlock) are not signalled, and have an inferred value of 0. In JVET-T2001 these subblocks instead have an inferred value of 1. This difference in inferred values leads to divergent behaviour in context initialization, which in turn leads to different decoding behaviour for the signalled subblock flags.

Below is relevant text from JVET-T2001, section 9.3.4.2.6 - Derivation process of ctxInc for the syntax element sb_coded_flag:

- The variable csbfCtx is initialized with 0 and modified as follows:

…

- Otherwise (transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0 or sh_ts_residual_coding_disabled_flag is equal to 1), the following applies:

- When xS is less than ( 1  <<  ( log2TbWidth − log2SbWidth ) ) − 1, csbfCtx is modified as follows:

csbfCtx  +=  sb_coded_flag[ xS + 1 ][ yS ]		(1547)

- When yS is less than ( 1  <<  ( log2TbHeight − log2SbHeight ) ) − 1, csbfCtx is modified as follows:

csbfCtx  +=  sb_coded_flag[ xS ][ yS + 1 ]		(1548)

Eq (1547) and (1548) show that context initialization may have dependency on the value of neighbouring sb_coded_flags. In such cases the neighbouring sb_coded_flag may have an inferred value, and therefore csbfCtx may have a different value depending on whether the inferred value is 0 or 1.

# Issues with JVET-Z0249

Following identification and confirmation of the mismatch, the proponent of TSRC contributed proposed text to fix the issue in JVET-Z0249 [5]. However, this contribution asserts there are further errors in the proposed text, which is reproduced below:

sb_coded_flag[ xS ][ yS ] specifies the following for the subblock at location ( xS, yS ) within the current transform block, where a subblock is an array of transform coefficient levels:

When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0 or sh_ts_residual_coding_disabled_flag is equal to 1, the following applies:

- If coded_sub_block_flag[ xS ][ yS ] is present and is equal to 0, the transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (coded_sub_block_flag[ xS ][ yS ] is present and is equal to 1), the following applies:

- If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the sig_coeff_flag syntax elements is present for the subblock at location ( xS, yS ).

- Otherwise, at least one of the transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

- Otherwise, when coded_sub_block_flag[ xS ][ yS ] is not present, one or more of the following conditions are true, coded_sub_block_flag[ xS ][ yS ] is inferred to be equal to 1:

- ( xS, yS ) is equal to ( 0, 0 ).

- ( xS, yS ) is equal to ( LastSignificantCoeffX  >>  log2SbW, LastSignificantCoeffY  >>  log2SbH ).

- Otherwise, coded_sub_block_flag[ xS ][ yS ] is inferred to be equal to 0.

Otherwise (transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0), the following applies:

- If sb_coded_flag[ xS ][ yS ] is equal to 0, all transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (sb_coded_flag[ xS ][ yS ] is not present), it is inferred to be equal to 1.

The errors are as follows:

- References throughout the proposed text to “coded_sub_block_flag”. The syntax element is sb_coded_flag.

- Due to the change in the condition “If coded_sub_block_flag[ xS ][ yS ] is present and is equal to 0”, there is a hole for the cases when coded_sub_block_flag[ xS ][ yS ] is not present (ie, inferred) and equal to 0. In such case the inferred values for the transform coefficient levels is undefined.

# Proposed text

This contribution proposes the following text instead. It is asserted that this text fixes ticket #1547 and does not introduce the errors identified in Section 2 of this contribution. Additions relative to JVET-T2001 are highlighted in yellow and deletions are shown in red strikethrough.

sb_coded_flag[ xS ][ yS ] specifies the following for the subblock at location ( xS, yS ) within the current transform block, where a subblock is an array of transform coefficient levels:

When sb_coded_flag[ xS ][ yS ] is equal to 0, all transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

When sb_coded_flag[ xS ][ yS ] is not present, it is inferred to be equal to 1.

When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0 or sh_ts_residual_coding_disabled_flag is equal to 1, the following applies:

- If sb_coded_flag[ xS ][ yS ] is equal to 0, the transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (sb_coded_flag[ xS ][ yS ] is equal to 1), the following applies:

- If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the sig_coeff_flag syntax elements is present for the subblock at location ( xS, yS ).

- Otherwise, at least one of the transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

- When sb_coded_flag[ xS ][ yS ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, sb_coded_flag[ xS ][ yS ] is inferred to be equal to 1:

- ( xS, yS ) is equal to ( 0, 0 ).

- ( xS, yS ) is equal to ( LastSignificantCoeffX  >>  log2SbW, LastSignificantCoeffY  >>  log2SbH ).

- Otherwise, sb_coded_flag[ xS ][ yS ] is inferred to be equal to 0.

Otherwise (transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0), the following applies:

- If sb_coded_flag[ xS ][ yS ] is equal to 0, all transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (sb_coded_flag[ xS ][ yS ] is equal to 1), at least one of the transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

- When sb_coded_flag[ xS ][ yS ] is not present, it is inferred to be equal to 1.

# References

[1] https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/1547

[2] B. Bross, J. Chen, S. Liu, Y. K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, Meeting by teleconference, 7-16 October 2020.

[3] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N2001-v6, Geneva CH, 19-27 March 2019.

[4] ITU-T, “Recommendation ITU-T H.265 (08/2021) High efficiency video coding”.

[5] T. Nguyen, “Proposed Fix for Ticket #1547: sb_coded_flag non-present inference”, JVET-Z0249, Meeting by teleconference, 20-29 April 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).