| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z1004-v1 |
| --- | --- |

| Title: | Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP Usage TR |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Errata report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Chris Rosewarne Gary Sullivan Yasser Syed Ye-Kui Wang | Tel: Email: | benjamin.bross@hhi.fraunhofer.de chris.rosewarne@canon.com.au garysull@miscrosoft.com yasser_syed@comcast.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, Video CICP, and the TR on usage of video signal type code points, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing, while some other items have not yet been confirmed. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-Z meeting:

- For AVC, corrections to texts related to alpha blending corresponding to the changes adopted to VSEI from JVET-Z0119.

- Updated the summary and publication status for VVC.

- Updated the summary and publication status for VSEI.

- Updated the summary for HEVC.

Incorporated items at the JVET-Y meeting:

- Removed all errata items for VVC and VSEI, due to that for both VVC and VSEI the second edition texts are being prepared by both ITU-T and ISO/IEC, and all the errata items have been incorporated therein, including fixes for bug tickets #1533 and #1534 for VVC (fixes for these two tickets have not been included in previous versions of the errata report).

- Removed errata items for AVC that have been resolved in both the latest ITU-T text (08/2021) and the new submission (MDS21132_WG05_N00098) of the ISO/IEC 10th edition text.

Incorporated items at the JVET-X meeting:

- Updated the publication status of the standards

- For VVC (the changes are included in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu and Frank Bossen (thanks!)

  - Various changes resulted from reports/suggestions by Peter de Rivaz (huge thanks!)

- For HEVC (the changes are included both below and in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu, Frank Bossen, and Cliff Reader (thanks!)

  - Some changes to the semantics of the entry point offset syntax element, resulted from the discussion of JVET-X0050

  - Changes to the semantics of the alpha channel information SEI message resulted from the discussion of JVET-X0059

- For AVC (the changes are included in an attachment to this document):

  - Some changes to the semantics of the alpha blendnig related syntax elements in clause 7.4.2.1.2, message resulted from the discussion of JVET-X0059

Incorporated items at the JVET-W meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for tickets #1488, #1511, #1491, #1493, #1494

- For HEVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the following fixes:

    - Correcting an “off by one” error in the loop counter for ar_num_object_updates, which indicates the number of updated annotated objects for the annotated regions SEI message.

    - Adding a description of the purpose of some variables that are derived in the semantics of the annotated regions SEI message

    - Adding a missing descriptor to specify the type of coding for the depth_nonlinear_representation_model[ i ] syntax element (the same as used in another specification, so clearly what was intended) for the depth representation information SEI message

    - Adding missing range specifications for a few syntax elements of the depth representation information SEI message

    - Miscellaneous very small cleanups of consistency, grammar and clarity, and correction of typographical errors.

- For AVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the same list of fixes as above for HEVC.

Incorporated items at the JVET-V meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for ticket #1486

- For HEVC

  - Removed the item on the absence of “persistence flag” for the annotated regions SEI message (this was agreed to be removed at the JVET-T meeting in October 2020, but the decision was overlooked)

Incorporated items at the JVET-U meeting:

- For VVC (the changes are included in an attachment to this document):

  - Some minor editorial corrections and improvements (JVET-U0073, JVET-U0085, and one item from Hendry)

  - Fixes for tickets #1416, #1428, #1432, #1454, and #1469

- For AVC: Added a reference to JVET-T1006-v1 for the errata changes, and removed the actual errata text changes from this document. (JVET-U0049)

# General aspects and issues affecting multiple standards

## General and minor matters

Please note that the alignment between the text published by ITU-T and that published by ISO/IEC should also be checked. Minor editorial issues and aspects that are highlighted for potential further checking include the following:

- (for HEVC only, and for the ITU-T text only; for the ISO/IEC text, this has been corrected in w18277_FDIS_23008-2_4thEd_plusAmd1_w18881) In the semantics of matrix_coeffs, there seems to be a problem in the HEVC spec (the AVC text is OK), saying "one or more" instead of "both" regarding the following constraint:

matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	BitDepthC is equal to BitDepthY.

–	chroma_format_idc is equal to 3 (the 4:4:4 chroma format).

- (not for VVC, not for VSEI) Updating the reference to Rec. ITU-R BT.2100 (the current version being BT.2100-2) and the associated equations for the ICTCP matrix coefficients interpretation for HLG (esp. check Video CICP; this is correct in Rec. ITU-T H.265 2019-06).

- (for Video CICP only, in the ISO/IEC version only) Logical indentation nesting in the ISO/IEC version of the Video CICP colour interpretation was inadvertently removed in the publication process.

- (for Video CICP only) The range of values for the extended aspect ratio indicator in Video CICP is not clearly specified and may implicitly be interpreted as inadequate to cover the range of values expressed in the video coding standards.

## sYCC colour indicator interpretation

### Status

This item has been confirmed by the JCT-VC and resolved for the ITU-T text, but has not yet been resolved for the ISO/IEC text. It affects multiple standards: HEVC and Video CICP (and JPEG XR). For background, see JCTVC-AJ0023.

### Description of the issue

This issue relates to the transfer characteristics and matrix coefficients indicators for the sYCC colour representation specified in IEC 61966-2-1. The semantics of transfer characteristics (Table E.4 of HEVC), and matrix coefficients (Table E.5 of HEVC) need correction to address the issue.

The issue is a bit complicated because the same transfer characteristics indicator value is used for both sRGB and sYCC, but IEC 61966-2-1 actually indicates that the transfer characteristics function should be somewhat different for the two cases. In the sRGB case, the range of the input value is constrained to be from 0 to 1, but in the sYCC case, this constraint should not apply.

The agreed correction for this aspect is to condition the interpretation of the transfer_characteristics syntax element for the value 13 on the value of the matrix_coeffs syntax element (since that value would differ between sYCC and sRGB).

Additionally, the the informative remark relating to the matrix coefficients indicator value for IEC 61966-2-1 sYCC should be changed to indicate that sYCC should be indicated with the matrix coefficients indicator equal to 5 (as for Rec. ITU R BT.601) rather than 1 (as for Rec. ITU-R BT.709).

### Description of the proposed fix

In E.3.1 (VUI semantics), in Table E.4 (Transfer characteristics interpretation using the transfer_characteristics syntax element) replace the row for the value 13 with:

| 13 | – If matrix_coeffs is equal to 0 V = α * Lc( 1 ÷ 2.4 ) − ( α − 1 ) for 1 >= Lc >= β V = 12.92 * Lc for β > Lc >= 0 – Otherwise V = α * Lc( 1 ÷ 2.4 ) − ( α − 1 ) for Lc >= β V = 12.92 * Lc for β > Lc > −β V = − α * ( −Lc )( 1 ÷ 2.4 ) + ( α − 1 ) for −β >= Lc | IEC 61966-2-1 sRGB (with matrix_coeffs equal to 0) IEC 61966-2-1 sYCC (with matrix_coeffs equal to 5) |
| --- | --- | --- |

In E.3.1 (VUI semantics), after Equation E-3, replace the next paragraph and associated bullet points with:

In this case, the range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is equal to 11 or 12, or transfer_characteristics is equal to 13 and matrix_coeffs is not equal to 0, E′R, E′G, and E′B are real numbers with values that have a larger range than the range of 0 to 1, inclusive, and their range is not specified in this Specification.

–	Otherwise, E′R, E′G and E′B are real numbers in the range of 0 to 1.

In E.3.1 (VUI semantics), in Table E.5 (Matrix coefficients interpretation using the matrix_coeffs syntax element), move “IEC 61966-2-1 sYCC” from the row for the value 1 to the row for the value 5.

# Reported errata items for VVC

The second edition texts of both ITU-T and ISO/IEC are in the publication process. The work on the third edition has started. JVET-Z2005 includes some VVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (04/2022, Edition 2) Approved 2022-04-29, published 2022-XX-XX

  - As of today (2022-06-15), the status is “In force (prepublished)”.

ISO/IEC 23090-3 (https://www.iso.org/standard/73022.html)

- ISO/IEC 23090-3:202X (Edition 2); published 202X-XX

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (08/2020, Edition 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23090-3 (https://www.iso.org/standard/73022.html)

- ISO/IEC 23090-3:2021 (Edition 1); published 2021-02

# Reported errata items for VSEI

The ITU-T second edition text has been published. The ISO/IEC second edition text is in the publication process. The work on the third edition has started. So far no VSEI errata items based on the second edition texts have been idenified.

## Publication status background

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (05/2022, Edition 2) Approved 2022-05-22, published 2022-06-06

ISO/IEC 23002-7 (https://www.iso.org/standard/79112.html)

- ISO/IEC 23002-7:202X (Edition 2); published 202X-XX

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (08/2020, Edition 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23002-7 (https://www.iso.org/standard/79112.html)

- ISO/IEC 23002-7:2021 (Edition 1); published 2021-01

# Reported errata items for HEVC

Some errata changes for HEVC are included in an attachment to this document. JVET-Z1005 also includes some HEVC errata changes. See also Section 1.

## Publication status background

Rec. ITU-T H.265

- (08/2021, Edition 8) Approved 2021-08-22, published 2021-10-13

- (10/2019, Edition 7) Approved 2019-11-29, published 2020-01-10

- (06/2019, Edition 6) Approved 2019-06-29, published 2019-09-23

- (02/2018, Edition 5) Approved 2018-02-13, published 2018-05-11

ISO/IEC 23008-2

- ISO/IEC 23008-2:2020 (Edition 4); published 2020-08

- ISO/IEC 23008-2:2017 (Edition 3); published 2017-10

- ISO/IEC 23008-2:2017/Amd 1:2018 (Additional colour representation code point); published 2018-03

- ISO/IEC 23008-2:2017/Amd 2:2018 (Main 10 still picture profile); published 2018-03

- ISO/IEC 23008-2:2017/Amd 3:2018 (Additional supplemental enhancement information); published 2018-07

- Work items merged for FDIS ballot as publication office action; Stage 50.00 Final text received or FDIS registered for formal approval as of 2020-03-19

  - ISO/IEC DIS 23008-2:201x (Edition 4); DIS ballot closed 2018-10-06; stage 40.99, Full report circulated: DIS approved for registration as FDIS 2019-02-18; Pending FDIS ballot based on WG 11 N 18277

  - ISO/IEC 23008-2:201x (Edition 4)/DAmd 1:201x (Additional supplemental enhancement information); DAM ballot started 2019-07-10, closed 2019-10-02

## On the general decoding process

### Status

This item applies to both ITU-T and ISO/IEC texts.

This item has been confirmed by the JCT-VC, and the proposed fix has been agreed in spirit. For background, see JCTVC-AI0022 and JCTVC-AJ0021. However, the specific text for the fix is not yet available and hence this has not yet been resolved.

### Description of the issue

The general decoding process is specified in a CVS-by-CVS manner, because the sub-bitstream extraction process relies on the SPS syntax element sps_max_sub_layers_minus1, while the active SPS can change across CVSs.

For each CVS of the input bitstream, the sub-bitstream extraction process is applied first with the output being BitstreamToDecode. Therefore, BitstreamToDecode contains one CVS. After that, the decoding process for a coded picture specified in clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode.

The following is stated in clause 8.1.3:

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

From the yellow-highlighted wording, it is clear that the term "the bitstream" mentioned above was intended to mean something that can contain more than one CVS.

Furthermore, there are many places related to "interpreting the semantics of each syntax element in each NAL unit" that have the wording "the first picture in the bitstream", particularly when used to determine whether a CRA picture starts a CVS. In my understanding, in those places, the intent was that the term "the bitstream" mentioned above was intended to mean something that can contain more than one CVS.

Lastly, at the beginning of the general decoding process for the input bitstream, strictly speaking, determining the CVSs in the input bitstream would not be clear before determination of the variable NoRaslOutputFlag for each IRAP picture, which currently is only part of the decoding process of a CVS.

### Description of the proposed fix

It is proposed to change the description of the general decoding process as follows:

- Firstly the CVSs in the input bitstream are identified.

- Sub-bitstream extraction is applied for each CVS.

- The extracted results of all the CVSs are concatenated to BitstreamToDecode.

- BitstreamToDecode is decoded CVS by CVS, and each CVS picture by picture.

## Miscellaneous items for the published ITU-T H.265 version 7 (potentially also applicable to the ISO/IEC 23008-2 text)

- In clause 5.10, replace "a mixture of lower case and upper case letter" with " a mixture of lower case and upper case letters" (missing 's' in the last word).

- In clause 6.3.3, replace "The following divisions of processing elements of this Specification form spatial or component-wise partitioning:" with "The following divisions of processing elements of this Specification form spatial or component-wise partitionings:" (missing 's' in the last word)? Note that in the ISO version there is the 's'. Which one is (more) correct?

- In clause 7.2, definition of next_bits( n ), replace "Provides a look at the next n bits in the bitstream with n being its argument." with "It provides a look at the next n bits in the bitstream with n being its argument."

- In clause 7.3.3, in the syntax condition for general_max_14bit_constraint_flag, replace the "Enter" with "shift+Enter".

- In clause 7.3.3, in the syntax condition for sub_layer_max_14bit_constraint_flag[ i ], replace two instances of "Enter" with "shift+Enter".

- In clause 7.3.3, resolving ticket #1504, in profile_tier_level syntax in tabular form remove an extra ‘)’ character after a condition check of general_profile_compatibility_flag[ 7 ].

- In clause 7.4.2.4.3, consider resolving ticket #1505 by making the following change suggested in the ticket:

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP access unit, which may be an IDR access unit, a BLA access unit, or a CRA access unit.

- In clause 7.4.4, remove the instance of "and " highlighted below:

When profilePresentFlag is equal to 1, general_profile_idc is not equal to 9 or 11 and is not in the range of 1 to 5, inclusive, and general_profile_compatibility_flag[ 9 ] is not equal to 1, general_profile_compatibility_flag[ 11 ] is not equal to 1, and general_profile_compatibility_flag[ j ] is not equal to 1 for any value of j in the range of 1 to 5, inclusive, the value of general_inbld_flag is inferred to be equal to 0.

- In clause 8.4.4.27, resolving ticket #1500, replace the following equations:

xL = palette_transpose_flag ? x * nSubHeight : x * nSubWidth	(8-69)

yL = palette_transpose_flag ? y * nSubWidth : y * nSubHeight	(8-70)

with the following:

xL = palette_transpose_flag ? y * nSubHeight : x * nSubWidth	(8-69)

yL = palette_transpose_flag ? x * nSubWidth : y * nSubHeight	(8-70)

- In clause 8.6.8.1, replace the following (ticket #1437):

- an array resSampleArrayCb specifying the chroma residual samples of the current block,

- an array resSampleArrayCr specifying the chroma residual samples of the current block.

with the following:

- an array resSampleArrayCb specifying the Cb residual samples of the current block,

- an array resSampleArrayCr specifying the Cr residual samples of the current block.

- In clause 9.3.3.1, resolving ticket #1498, change the following (note that it was said in the JCTVC-AL meeting minutes that this was confirmed present in H.265 (11/19), but it is not in the published ITU-T H.265/HEVC version 7):

cMax = ( PrefixOffset  <<  1 ) > PaletteMaxRunMinus1 ?
( PalletMaxRun − PrefixOffset ) : ( PrefixOffset − 1 )

to

cMax = ( PrefixOffset  <<  1 ) > PaletteMaxRunMinus1 ?
( PalleteMaxRunMinus1 − PrefixOffset ) : ( PrefixOffset − 1 )

- In clause 9.3.4.1, resolving ticket #1491, make the following changes:

The parsing of each bin is specified by the following two ordered steps:

1.	The derivation process for ctxTable, ctxIdx, and bypassFlag as specified in clause 9.3.4.2 is invoked with binIdx as input and ctxTable, ctxIdx and bypassFlag as outputs.

NOTE – As a consequence of invoking the process in clause 9.3.4.2, tThe arithmetic decoding process as specified in clause 9.3.4.3 is also invoked with ctxTable, ctxIdx and bypassFlag as inputs and the value of the bin as output.

- In Table 9-48, resolving ticket #1507, remove the last four rows of the table (i.e., for syntax elements cu_qp_delta_abs, cu_qp_delta_sign_flag, cu_chroma_qp_offset_flag, andcu_chroma_qp_offset_idx. These rows are duplicates of rows appearing earlier in the same table.

- In clause A.4.1, Table A.8, add "MaxSliceSegments" after "Max slice segments per picture".

- In clause A.4.2, Table A.10, some of the numbers, e.g., "xxxx" are not in the form of "x xxx". Make the style of all the numbers consistent.

- In clause D.3.1, replace the following:

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, and 200 to 202, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, and 200 to 202, inclusive.

with the following:

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, and 200 to 202, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, inclusive, and 200 to 202, inclusive.

- In clause D.3.32, NOTE 2, some of the numbers, e.g., "1000" are not in the form of "1 000". Make the style of all the numbers consistent.

- In clause D.3.39, check whether the values "0.0001" and "0.00002" should be in the forms of "0.000 1" and "0.000 02", respectively.

- In the ITU-T version of HEVC and the w18881 MPEG document for FDAM1, in the semantics of ar_object_label_update_flag, “will not” is repeated.

- In D.3.41.6.2 (Projection for one sample location), it says that there are inputs to the process that are “pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units” and “the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values”. It does not seem clear what this means.

- The pictureWidth and pictureHeight variables are also modified internally in D.3.41.6.2, which seems potentially unwise.

- D.3.41.6.4 (Conversion of sample locations for rectangular region-wise packing) has inputs that are “offset values for the sampling position (offsetX, offsetY)”. D.3.41.6.5 says “offsetX is set equal to 0.5 and offsetY is set equal to 0.5.” Are those the only values that are ever used? If so, why define a variable that does not vary in value?

- The variable names offsetX and offset that are used in D.3.41.6.4 are also used in 8.5.3.2, but for a different purpose. It may be unwise to use the same variable names in Annex D.

- The first sentence of D.3.41.6.5 (Mapping of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes) says “This clause specifies the semantics of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes.” Here, “semantics” should probably be “mapping”.

- In clause D.3.41.7.2, Equation D-57, change the last row from "θ = Asin( z ÷ Sqrt( x2 + y2 + z2 ) ) ÷ π" to "θ = Asin( z ÷ Sqrt( x2 + y2 + z2 ) ) * 180 ÷ π".

- The indent distance for the titles of clauses D.3.41.7.3 to D.3.41.7.6 should be aligned to be with those for clauses D.3.41.7.1, D.3.41.7.2, and D.3.41.7.7.

- D.3.41.7.7 is only for the fisheye video information SEI message, which is handled somewhat separately and differently than other projection mappings; is this appropriate and clear enough?

- In clause E.3.1, check whether the values "0.3127" etc. should be in the forms of "0.312 7" etc.

- In clause E.3.1, replace "matrix_coeffs shall not be equal to 0 unless one or more of the following conditions are true:" with "matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:".

- In clause E.3.1, Equations E60-E67, some numbers are in the forms of "0.986566" and "4096" etc. instead of in the forms of "0.986 566" and "4 096" etc.

- In clause E.3.1, Table E.5, some numbers are in the forms of "0.0593" etc. instead of in the forms of "0.059 3"etc.

- In clause E.3.1, NOTE 16, change "1920x540" to "1 920 x 540".

- In clause E.3.2, replace "The need for presence of buffering periods for NAL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application" with "The need for the presence of buffering period parameters for NAL HRD operation in the bitstream in buffering period SEI messages is determined by the application".

- In clause E.3.2, replace "The need for presence of buffering periods for VCL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application" with "The need for the presence of buffering period parameters for VCL HRD operation in the bitstream in buffering period SEI messages is determined by the application".

- In clause E.3.2, replace "The need for presence of CPB and DPB output delays to be present in the bitstream in picture timing SEI messages is determined by the application" with "The need for the presence of CPB and DPB output delays in the bitstream in picture timing SEI messages is determined by the application".

- In clause F.7.4.3.1.1, replace "1023" with "1 023".

- In clauses F.13.2.3 and F.13.4, replace "90000" with "90 000".

- In clause G.14.3.1, replace the following

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 180, inclusive, and 200 to 202, inclusive.

with the following:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 180, inclusive, and 200 to 202, inclusive.

- In clause I.7.4.3.1.7, replace "65535" with "65 535".

- In clause I.7.4.3.2.3, replace "is not used the decoding process" with "is not used in the decoding process", and replace "is not present coding units" with "is not present in coding units".

- In clause I.14.3.1, replace the following:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 181, inclusive, and 200 to 202, inclusive.

with the following:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 181, inclusive, and 200 to 202, inclusive.

## More miscellaneous items from JCTVC-AN0024

For both the ITU-T and the ISO texts:

- Corresponding to the following changes to VVC, an errata report for AVC and HEVC, to change the MinCr limit to be derived from the CPB size limit or add a note for cases where the CPB size imposes a tighter limit than the MinCr does:

MaxCPB = 80 000 for level 6, 120 000 for level 6.1, 180 000 for level 6.2, and change MinCrScaleFactor for the 4:4:4 profile to 0.75, and change MinCrBase to 8 for level 6.2.

- Relax the semantics so that an extension_flag in VPS, SPS, or PPS equal to 1 specifies that specifies extension_data_flag syntax elements may be present. (Currently, it says these flags are present.)

- In clause 8.1.1, add the derivation of the variables DuHrdPreferredFlag and DecodingUnitHrdFlag, similarly as in HEVC.

- Add "The variable DuHrdPreferredFlag is either specified by external means, or when not specified by external means, set equal to 0." to clause 8.1.2.

- Fix the description of the recovery point picture to consider the case with a recovery POC distance of 0.

## More miscellaneous items from JVET-T0054

Agreed at the 20th JVET meeting (i.e., the JVET-T meeting).

- Search and replace "satisify" with "satisfy".

- In clause 9.3.3.6, fix the wrong indention for the paragraph starting with "Otherwise".

- Search and check "CVS", particularly in semantics, and fix to be CLVS as needed.

- Search and replace instances of "clasue" with "clause".

- Remove a tab from each of the two middle rows below:

| content_light_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| max_content_light_level | u(16) |
| max_pic_average_light_level | u(16) |
| } |  |

# Reported errata items for AVC

Some errata changes for AVC are included in an attachment to this document.

## Publication status background

Rec. ITU-T H.264

- (08/2021, Edition 14) Approved 2021-08-22, published 2021-10-13

- (06/2019, Edition 13) Approved 2019-06-13, published 2019-09-06

ISO/IEC 14496-10

- ISO/IEC 14496-10:202X (Edition 10), published 202X-XX

- ISO/IEC 14496-10:2020 (Edition 9), published 2020-12

- ISO/IEC 14496-10:2014 (Edition 8), published 2014-09

- ISO/IEC 14496-10:2014/Amd 1:2015 (Multi-resolution frame compatible stereoscopic video with depth maps, additional supplemental enhancement information and video usability information), published 2015-11

- ISO/IEC 14496-10:2014/FDAMD 2 (Additional Levels and Supplemental Enhancement Information); stage 50.98, deleted in preparation for Edition 9

- ISO/IEC 14496-10:2014/Amd 3:2016 (Additional supplemental enhancement information); published 2016-12, published 2016-12

- ISO/IEC DIS 14496-10:201x (Edition 9); stage 40.99 Full report circulated: DIS approved for registration as FDIS as of 2018-01-31

# Reported errata items for Video CICP

See also Section 1.

## Publication status background

Rec. ITU-T H.273

- (07/21, Edition 2) Approved 2021-07-14, published 2021-09-24

- (12/16, Edition 1) Approved 2016-12-22, published 2017-04-27

ISO/IEC 23091-2 (previously part of ISO/IEC 23001-8)

- ISO/IEC 23091-2:2019, published 2019-07

## Errata items

- There is an error in the range of values for the sample aspect ratio width and height.

- The CICP revision should account for the various errata issues previously noted in AL1004. It was confirmed that the BT.2100 reference needs updating (to -2).

- The equations for ICtCp with HLG have not been updated and need be corrected.

# Reported errata items for Codepoint Usage TR

## Publication status background

Sup. ITU-T H.Sup19

- (04/21, Edition 3) Approved 2021-04-30, published 2021-06-04

- (10/19, Edition 2) Approved 2019-10, published 2019-11-14

- (03/19, Edition 1) Approved 2019-03, published 2019-04-30

ISO/IEC 23091-4

- ISO/IEC 23091-4:202x, stage 60.00 Under publication since 2020-04-05

- ISO/IEC 23091-4:2019, published 2019-08

## Miscellaneous items

- In a liaison letter sent to ITU-T SG16, SMPTE identified an anomaly in Table 7 of the 1st edition, as follows:

| Mastering Display Minimum Luminance | Registration identifier | was 060e2b34.0101010e.04200401.01030000 should be urn:smpte:ul:060e2b34.0101010e.04200401.01040000 see MasteringDisplayMaximumLuminance at https://registry.smpte-ra.org/view/published/elements_by_group_view.html |
| --- | --- | --- |

- SMPTE also recommended using urn notation for writing labels within a text “urn:smpte:ul:”, as they said this significantly increases the ability to machine parse and process documents. The correction to the supplied document was provided in the urn notation above.

- The registers can be checked online at this URL: https://registry.smpte-ra.org/apps/pages/published/. This could be mentioned in the text.

- For 3.14 of the ISO/IEC pre-publication text, an editor suggested that the definition of “narrow range” does not describe narrow range sufficiently, and suggested defining it as “range in a fixed-point (integer) representation where active video does not span the full range of values that could be expressed with that bit depth but does use the remaining range for under/overshoot (processing artifacts) and sync”

- In clause 4 of the ISO/IEC pre-publication text, an editor suggested that UHD now informally includes 1080P with HDR and WCG, and proposed the following “UHD: informally used with several resolutions (3840 × 2160 for television or 4096 × 2160 for film with or without HDR/WCG); (7680 × 4320 or 8192 × 4320 with or without HDR/WCG); (1080P WITH HDR/WCG)”.

- In the last paragraph of clause 6 of the ISO/IEC pre-publication text, an editor suggested to add MaxFall/MaxCLL to make a sentence more complete, as in “The approved colour volume, which may be smaller than the container volume, is often indicated with SMPTE ST 2086 including MaxFall/MaxCLL metadata. Over time, it is expected that WCG and/or high dynamic range (HDR) applications will evolve to use more of the available container colour volume.”

- For 7.1 of the ISO/IEC pre-publication text, an editor suggested to add facts: that MP4 containers (and QuickTime) use color atoms utilizing MPEG CICP values to indicate Color Gamut/Transfer Function/Matrix Coefficients, as in “MP4 (and QuickTime) wrappers utilize MPEG CICP integer values shown in Table 4 to indicate Color Gamut/Transfer Function/Matrix Coefficients. These values are stored in color “atoms”.”

- For an Annex C of the ISO/IEC pre-publication text, an editor suggested to consider adding a reference to RP 214 for carriage of dynamic HDR metadata, saying that RP 214 is being considered for storage of dynamic HDR metadata like ST.2094 and that dynamic metadata can carry more information about content within a stream which assists in remapping HDR video images to a specific displays capabilities. RP 214 describes a means for packing SMPTE KLV encoded metadata and/or essence into SMPTE 291M ancillary data packets. The editor said there is strong interest in carrying audio/visual data, metadata, and/or essence, within the same digital stream and said that the use of one data stream facilitates delivery of the overall multimedia presentation. Metadata is classified as information about the essence.

- Check for P3D65x1000n0005 vs P3D65x1000n005

- Check for P3D65x4000n0005 vs P3D65x4000n005

- Check for “specs"” (ending in a quote mark)

- Review the phrase “and the combination is specified in ITU-R and/or SMPTE specifications”

- 7.2.3 note below table 5 add “(Rec. ITU-R BT.709)” to the sentence: "ATSC specifications list use of the transfer characteristics value of 1 for SDR NCG  (Rec. ITU-R BT.709) video"

- A small punctuation suggestion for 7.2.4 (adding a comma):

“It is important for tools to process video according to the colour volume it is operating in to make sure the conversion is consistent.”

Change to:

“It is important for tools to process video according to the colour volume it is operating in, to ensure the conversion is consistent.”

- A slightly awkward sentence in the same note area in 7.2.4:

"ARIB STD B32 lists use of the transfer characteristics value 1 for HD and 14 for UHD for SDR WCG video.”

Change to:

"ARIB STD B32 lists use of the transfer characteristics value 1 for HD (Rec. ITU-R BT.709) and 14 for UHD SDR WCG (Rec. ITU-R BT.2020) video.”

- To the last sentence in 7.2.4, add as in

“The indicated chroma sample location alignment is only applicable for 4:2:0 chroma sampling. ChromaLocType (the generic label used in this document for the HEVC and AVC bitstream syntax elements: chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field), listed in Tables 1 and 3 of this document, indicates the 4:2:0 chroma sample position alignment.”

Change to:

“The indicated chroma sample location alignment is only applicable for 4:2:0 chroma sampling. ChromaLocType (the generic label used in this document for the HEVC and AVC bitstream syntax elements: chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field), listed in Tables 1 and 3 of this document, indicates the 4:2:0 chroma sample position alignment.  The Rec. ITU-R program signal exchange series (BT.601, BT.709, BT.2020, BT.2100) consider 4:2:2 and 4:4:4 chroma samples to be co-sited with luma.”

- Also insert parenthetic details in Annex A

“The transfer characteristics indicator values of 1, 6, 14, and 15 are functionally the same. Blu-ray BD-ROM 3.1 (“4K”) and the DVB UHD specifications list use of the transfer characteristics value of 14 for SDR/WCG (Rec. ITU-R BT.2020) video. ATSC specifications list use of the transfer characteristics value of 1 for SDR video. ARIB STD B32 lists use of the transfer characteristics value 1 for HD and 14 for UHD for SDR WCG video.”

Change to:

“The transfer characteristics indicator values of 1, 6, 14, and 15 are functionally the same. Blu-ray BD-ROM 3.1 (“4K”) and the DVB UHD specifications list use of the transfer characteristics value of 14 for SDR/WCG (Rec. ITU-R BT.2020) video. ATSC specifications list use of the transfer characteristics value of 1 for SDR NCG (Rec. ITU-R BT.709) and SDR WCG (Rec. ITU-R BT.2020) video. ARIB STD B32 lists use of the transfer characteristics value 1 for HD (Rec. ITU-R BT.709) and 14 for UHD SDR WCG (Rec. ITU-R BT.2020) video.”

- General: Review parentheticals

- General: Review/rephrase instances of “and that”.

## Miscellaneous items from JVET-T0113

- Ballot comments: Removal of version numbers: Agreed, and similarly remove version numbers from BT.656-5, BT.1120-9, BT.2077-2 (Tables 8 & 9).

- Change “Interstitial” to “Vertically interstitial”.

- Typo: “ST.2036-4”

- Copy-paste typo: Bottom line of Table 3 “ChromaLocType = 0” should be “ChromaLocType = 2”

- Left alignment on bibliographic entry for SMPTE RA.