| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0053-v2 |
| --- | --- |

| Title: | AHG12: Using block vector derived from IntraTMP for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Lim Donghyun Kim Sung-Chang Lim Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-1836 woong.lim@etri.re.kr kimddng@etri.re.kr sclim@etri.re.kr jschoi@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution proposes to store and use block vector (BV) derived from intra template matching prediction (IntraTMP) for intra block copy (IBC). IntraTMP finds the best matched template position in the reconstructed area of the current picture and uses the corresponding block as the prediction block of the current block. However, there is no process for storing block vector of the IntraTMP in ECM-5.0. In this contribution, IntraTMP block vectors are stored and added as spatial candidates to block vector candidate list of IBC in order to improve the coding efficiency. Compared to ECM-5.0, it is reported that the proposed method shows average BD-rates of -0.13% and -0.17% for class F and TGM under AI case and average BD-rates of -0.06% and -0.04% for class F and TGM under RA case with negligible complexity impact. And, it is suggested to include the proposed method into the next version of ECM.

Introduction

Intra template matching prediction (IntraTMP) [1] in ECM-5.0 [2] is used for screen contents under common test conditions (CTC). The IntraTMP on/off flag is signalled in CU level and the intra prediction mode for the IntraTMP is stored as PLANAR mode. During IntraTMP process, the block vector (BV) between the current block and the reference block can be simply calculated and stored as the block vector of the current block, but it is not exploited in the current IBC modes despite IntraTMP block vector can be added to block vector candidate list for intra block copy (IBC). In the current IBC in ECM-5.0, construction of IBC block vector candidate list is conducted for block vector prediction of IBC AMVP and merge mode. The block vectors of spatial neighbouring blocks are firstly added to the lists. Then, history-based candidates, a pairwise-average candidate, a set of BVP candidates located in the IBC reference region are added and template matching (TM)-based reordering is applied.

Proposed Method

The proposed method derives IntraTMP block vector and stores them in the IBC block vector buffer. As shown in Figure 1, when the best matched template is found, the block vector between the current block and the reference block can be simply derived and stored. The IntraTMP block vectors can be added to IBC block vector candidate list as candidates.

Figure 1. Derivation of IntraTMP block vector

Figure 2 shows an example of comparing the block vector candidates which are from only IBC coded blocks in the IBC block vector candidate list and the block vector candidates which are from both IBC and IntraTMP coded blocks in the proposed IBC block vector candidate list. In this contribution, the IntraTMP block vectors are added to IBC block vector candidate list as spatial candidates.

(a)

(b)

Figure 2. Examples of IBC block vector candidate list existing (a) only IBC block vectors and (b) both IBC and IntraTMP block vectors

It is noted that the proposed method makes IBC block vector prediction more efficient by using diverse block vectors without additional memory for storing block vectors.

Experimental Results

The experiments were performed based on ECM-5.0 software under JVET ECM common test conditions [3]. And, the experiments were conducted on the platform of Intel Xeon Gold 6256 (3.6GHz) and 256GB RAM with GCC 9.4.0 compiler under Ubuntu 20.04.4 LTS. The experimental results of the proposed method are summarised in Table I and Table II. And, the detailed results can be found in accompanied Excel file.

Table I. Experimental result of the proposed method under AI case

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.13% | -0.13% | -0.13% | 100% | 99% |
| Class TGM | -0.17% | -0.13% | -0.14% | 100% | 97% |

Table II. Experimental result of the proposed method under RA case

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.06% | -0.08% | 0.09% | 100% | 100% |
| Class TGM | -0.04% | -0.01% | -0.02% | 100% | 100% |

Conclusions

This contribution proposed to store and use block vector derived from intra template matching prediction for intra block copy. The experimental result reportedly shows that the proposed method achieves coding performance for screen contents under AI and RA cases with negligible complexity impact. Therefore, it is recommended to include the proposed method into the next version of ECM.

References

- K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher, "EE2: Intra Template Matching," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-V0130, 22nd meeting, by teleconference, Apr. 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-5.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).