| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0069-v2 |
| --- | --- |

| Title: | Non-EE2 : amvpMerge for low delay |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim Seung-Hwan Kim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the ECM-5.0, the amvpMerge is enabled only when a reference picture pair includes both a forward picture and a backward picture because amvpMerge is based on bilateral matching. Considering that amvpMerge could be effective even for the low-delay case without bilateral matching-based reordering and refinement, this contribution suggests enabling the amvpMerge for low delay case. In order to enable amvpMerge for low delay case this contribution proposes three aspects below:

- The amvpMerge is enabled even the reference picture pair is not true bi-directional pictures for low-delay case. Additionally the restriction condition on the resampled, long-term and WP reference picture are relaxed for low-delay picture cases.

- BM cost of an amvpMerge candidate which is not true bi-direction is set to be the maximum value during merge candidate reordering.

- The decoder side motion vector refinement process of the conventional amvpMerge is bypassed for the low delay case.

With combination of above mentioned aspects, following two alternative methods are tested to support the amvpMerge in low delay case.

- Method 1 : aspect 1 + aspect 2

- Method 2 : aspect 1 + aspect 2 + aspect 3

The proposed test also includes !MR184 to prevent an empty merge candidate list for amvpMerge.

The simulation result of Method1 shows {%d, %d, %d / Enc%d, Dec%d} and Method2 shows {%d, %d, %d / Enc%d, Dec%d} for LDB configuration. The negligible performance impart on RA is observed for the both proposed method.

Introduction

The original design of amvpMerge is based on bilateral matching so it has been applied for only random access configuration because amvpMerge is enabled only when the reference picture pair can be constructed as one is a forward picture and the other is a backward picture. Therefore amvpMerge has not been applied in the low-delay case due to the reference pictures being all forward pictures.

However, it is investigated that amvpMerge is effective even for the low-delay case without bilateral matching-based reordering and decoder side motion vector refinement.

Proposed methods

The proposed method purposed to enable amvpMerge for low delay case such as all reference pictures are forward pictures where is not available to construct forward and backward reference picture pair.

The proposed aspects are described as below:

- For the case that all reference pictures are forward pictures, amvpMerge is enabled when at least one reference picture pair in which reference pictures have different POC is available in the slice level. The restriction condition on the resampled, long-term and WP reference picture are relaxed for low-delay picture cases.

- For the bilateral matching-based merge candidate list reordering, if the reference picture of amvpMerge candidate is all forward picture then the cost is set to be the maximum value.

- For the low delay case, the template matching based decoder side motion vector refinement process for amvpMerge is bypassed.

In order to investigate the impact on the proposed aspect, two alternative methods have experimented with different sets of the proposed aspects.

Proposed method 1

The proposed method 1 includes aspect 1 and aspect 2 because motion vector refinement is possible based on the template matching process in the conventional amvpMerge process in the low-delay case. Therefore method 1 could show the compression benefit of amvpMerge for low-delay with template matching based decoder side motion vector refinement unlike the method 2 bypassing the entire decoder side motion vector refinement process for amvpMerge.

Proposed method 2

Proposed method 2 includes all the proposed aspects. Template matching based motion vector refinement is effective but it causes latency delay and throughput issues which is not desirable for low-delay scenarios. Therefore method 2 could indicate the compression benefit of amvpMerge without bilateral matching based on merge candidate reordering and the entire decoder side motion vector refinement process.

Experimental results

The simulation results of the proposed methods in this contribution are achieved on a common test configuration. The experimental result for method 1 is reported in table 1 and also the result for method 2 is reported in table 2.

Table 1. Experimental result of the proposed method 1

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 CTC |  |  |  |  | Over ECM-5.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.02% | -0.05% | 101% | 101% |  |  |  |  |  |
| Class C | 0.00% | -0.05% | -0.02% | 101% | 100% | -0.29% | -0.43% | -0.80% | 107% | 103% |
| Class E |  |  |  |  |  | -0.56% | -0.21% | -0.91% | 106% | 101% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.20% | 0.05% | 101% | 101% | -0.28% | 0.21% | 0.19% | 106% | 100% |
| Class F | 0.00% | -0.03% | 0.00% | 101% | 101% |  |  |  |  |  |

Table 2. Experimental result of the proposed method 2

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 CTC |  |  |  |  | Over ECM-5.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.03% | -0.05% | 100% | 101% |  |  |  |  |  |
| Class C | 0.00% | -0.05% | -0.03% | 101% | 101% | -0.27% | -0.44% | -0.45% | 105% | 102% |
| Class E |  |  |  |  |  | -0.47% | -0.14% | -0.41% | 105% | 102% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.19% | 0.07% | 99% | 99% | -0.28% | 0.01% | -0.41% | 104% | 100% |
| Class F | 0.00% | -0.03% | 0.00% | 99% | 99% |  |  |  |  |  |

Patent rights declaration(s)

LG Electronics Inc. have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).