| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0075 |
| --- | --- |

| Title: | Non-EE2: Template matching based BCW index derivation for merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jie Chen Yan Ye Xinwei Li | Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution proposes to derive BCW index for merge coded CUs based on template matching cost instead of inferring from neighboring blocks. Given a selected merge candidate, the TM cost values are calculated with different BCW indices setting, and then, the BCW index with minimum TM cost value is used to predict the merge CU. In addition, it is proposed to remove negative weight and use three weights {3/8, 4/8, 5/8} for both low delay and non-low delay pictures. It is reported that on top of ECM-5.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.10%, -0.15%, -0.18%, 100%, 100%} for RA and {-0.13%, -0.12%, -0.18%, 101%, 101%} for LDB.

Introduction

In ECM-5.0, BCW is applied to a bi-predicted CU. When BCW is enabled, weighted averaging of the L0 and L1 prediction signals are performed. Five weights {-2/8, 3/8, 4/8, 5/8, 10/8} and three weights {3/8, 4/8, 5/8} are allowed for low delay pictures and non-low delay pictures, respectively. The BCW index is signaled for non-merge coded CUs, whereas for merge coded CUs, the BCW index is inherited from neighboring blocks according to the signaled merge candidate index.

Proposed methods

In this contribution, it is proposed to derive BCW index for merge coded CUs based on template matching cost. When a merge candidate is determined for a merge CU according to the signaled merge candidate index, its BCW index is derived based on TM cost. The TM cost for each BCW index is calculated using the SAD between samples of template of neighboring reconstructed samples and corresponding reference samples, and then, the BCW index with minimum TM cost is used to code the merge CU. Since the inherited BCW index is likely to have higher accuracy than another BCW index, the TM cost of the inherited BCW index is multiplied with 0.90625, that is, the cost is reduced by 3/32. Besides, the TM cost of the equal weight is multiplied with 0.90625 since bi-predicted samples are beneficial for BDOF and BDOF is only applied to CU with equal weight. It is noted that this proposed method is applied to regular merge, template matching, adaptive decoder-side motion vector refinement and affine merge modes. For low delay pictures, this proposed method is additionally applied to MMVD mode.

In addition, it is proposed to remove negative weight for low delay pictures. That is, only three weights {3/8, 4/8, 5/8} are allowed for both low delay and non-low delay pictures.

Simulation results

The proposed methods are implemented on top of ECM-5.0 and are tested under CTC for enhanced compression tool testing [1] with random access and low delay B configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.32% | -0.30% | 99% | 99% |
| Class A2 | -0.10% | -0.19% | -0.14% | 100% | 100% |
| Class B | -0.08% | -0.12% | -0.20% | 101% | 100% |
| Class C | -0.07% | -0.01% | -0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.15% | -0.18% | 100% | 100% |
| Class D | -0.02% | -0.18% | -0.04% | 101% | 102% |
| Class F | -0.11% | -0.14% | -0.10% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | 0.02% | -0.27% | 101% | 101% |
| Class C | -0.10% | 0.04% | -0.16% | 101% | 102% |
| Class E | -0.20% | -0.57% | -0.07% | 100% | 100% |
| Overall | -0.13% | -0.12% | -0.18% | 101% | 101% |
| Class D | -0.05% | 0.78% | 0.55% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).