| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: by teleconference, 13–22 Jul. 2022 | Document: JVET-AA0082-v1 |
| --- | --- |

| Title: | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent Jianghao Jia, Zhenzhong Chen Wuhan University | Email: | {zizhengliu, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents a deep-neural-network-based reference frame generation (DRF) method for VVC inter prediction enhancement. During the encoding and decoding process, the deep reference frame generation network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists for the current encoding picture as an alternative reference to enhance the inter prediction. Different from the previous work in which only the frames with bi-directional reference are processed, this contribution extends the reference frame generation method to uni-directional prediction cases and apply it to LD configuration. It is reported that this method can bring {LDB: 3.41%/13.59%/14.17%, RA: 2.03%/7.02%/6.45%} bitrate savings for Y/U/V components respectively, following the JVET-NNVC CTC.

Introduction

In VVC, the reconstructed pictures stored in decoded picture buffer (DPB) are organized as Reference Picture List (RPL) for current encoding picture. During inter prediction procedure, the current frame will select multiple frames from RPL as reference picture for the motion estimation. The performance of inter prediction is highly dependent on the content of reference frames. Reference frames with fewer compression artifacts or more relevant content can boost prediction performance, which will lead to smaller residual coding consumption in the bitstream. This motivates some methods to facilitate inter prediction by synthesizing better reference pictures.

Recently, deep neural network (DNN) plays an important role in image and video generation task. Many DNN-based frame interpolation or synthesis strategies have been proposed and achieved good generation effect. Based on the existing research of frame interpolation [1], this contribution proposes a deep-neural-network-based reference frame generation method for VVC inter prediction. Specifically, a pair of previously encoded frames are processed by a video frame synthesis network to generate a new reference picture, which can be regarded as the prediction of currently to-be-coded frame. Subsequently, the generated frame is integrated into the reference picture list exploited by inter prediction.

This method has been presented in JVET-X0060 [2] and JVET-Y0096 [3] at previous meetings, in which the bi-directional reconstructed frames are used for reference frame interpolation. Different from the previous work, there are several modifications in this contribution:

a) the reference frame interpolation is extended to uni-directional prediction, which is applicable for Low-delay configuration.

b) the index of the generated reference frame in the RPL is adjusted.

In this contribution, the virtual reference generation method is not only applied in bi-directional prediction frames but employed for uni-directional prediction enhancement. The terminology “interpolation” used in previous work is not suitable for describing the current framework since the generated frame is outside the input two frames in uni-directional prediction cases. We unify the name of the proposed method to Deep Reference Frame generation (DRF) to avoid confusions.

Proposed method

Framework

Fig. 1 The frame work of the proposed deep refrence frame generation (DRF) method

The framework of the proposed DRF method is shown in Fig. 1. In this method, two reconstructed frames selected from DPB together with the QP maps are fed into the well-designed frame synthesis network to synthesize a new frame. Then a cascade filtering network will process the output frame to get the final generated frame. This generated frame will be consequently inserted to both Reference List0 and List1 with the same POC of the current encoding frame as an alternative reference, and the ME process is not modified, no extra information signaled into the bitstream.

Different from the previous work [2] and [3] in which only the frames with bi-directional reconstructed reference frames are processed, we extend this method to uni-directional prediction cases and apply it to LD configuration. The reference frame generation strategies for uni and bi-directional prediction are different due to the availability of reconstructed frames, specifically:

For bi-directional prediction frame, this method selects two nearest reconstructed frames with equal distance from the previous and following reference lists of current encoding frame, which has been illustrated in [3]. The index of the generated frame in RPL is adjusted to 1 in this method, different from the last position of RPL used in [3].

For uni-directional prediction frame, the reconstructed frames with current POC - 1 and current POC - 2 obtained from DPB are used as the input of the frame generation network. The index of the generated reference frame in RPL is set as 2.

Network architecture

Fig. 2 the architecure of the frame synthesis network

Fig. 2 shows the architecture of the employed frame synthesis network. As shown, the synthesis network is a 3D U-Net obtained by extending the popular 2D Unet by replacing all the 2D convolutions in the encoder and decoder with 3D convolutions (3DConv) to accurately model the temporal dynamics between the input frames. Each 3D filter is a 5-dimensional filter of size ci×co×t×h×w, where t=2 is the number of input frames and (h, w) is the spatial size of the kernel, ci and co are the number of input and output channels in the layer. The input is a list consisting of two frames concatenated with the corresponding QP map.

Fig. 3 the architecure of the filter network

The filtering network is composed of several ResBlocks with CBAM module as shown in Fig. 3. The input of the filtering network is the synthezed frame from the frame synthesis network as demonstrated above.

It should be noted that in this method, the model used in uni-directional prediction has the same network structure with the one used in bi-direction prediction but different parameters due to the different training sets. Since the network uses the 3D CNNs to obtain the information used for frame generation with no extra motion indicators like optical flow or deformable kernel needed, the network is determined completely by the input of training settings. Considering an image triplet consisting with three consecutive images named {img1, img2, img3}, the model trained with {img1, img3} as input for predicting img2 is used for bi-directional prediction, and the model trained with {img1, img2} as input for predicting img3 is used for uni-directional prediction.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | A100 40GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 300 |
|  | Batch size: | 256x256 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | 7 Days |
|  | Training data information: | Vimeo (compressed), BVIDVC (compressed) |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 44769K |
|  | Total Number of Parameters (All Models) | 44769K x 2 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 170.78MB x 2 |
|  | Multiply Accumulate (kMAC/pixel) | 1410 |
|  | Calculation Method | On a block basis |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 256 x 256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

This method was implemented on the JVET-NNVC reference software (VTM11+V0056), which is the anchor platform of the comparisons. Following the JVET-NNVC CTC [4], we conducted the compression experiments to evaluate the performance of the proposed deep reference frame generation method. Table 3 and Table 4 show the experimental results of the proposed DRF method compared to the anchor results in LDB and RA configurations, respectively. According to the experimental results, the proposed DRF method can achieve {3.41%/13.59%/14.17%, 2.03%/7.02%/6.45%} coding efficiency gains for YUV components on LDB and RA configuration respectively.

Table 3. Performance of the proposed method (LDB, full length)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - |
| Class B | -2.63% | -12.53% | -14.02% | -3.35% | -15.34% | -20.62% | 225% | 217839% |
| Class C | -3.30% | -14.91% | -14.35% | -4.70% | -15.74% | -13.74% | 211% | 224103% |
| Class E | -4.87% | -8.20% | -9.19% | -5.13% | -9.34% | -9.79% | 458% | 403088% |
| Overall | -3.41% | -13.59% | -14.17% | -4.24% | -15.52% | -17.56% | 263% | 256481% |
| Class D | -3.39% | -14.21% | -12.61% | -4.81% | -11.84% | -7.74% | 222% | 233298% |

Table 4. Performance of the proposed method (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.62% | -4.52% | -4.56% | -1.99% | -5.77% | -5.57% | 213% | 172093% |
| Class A2 | -2.24% | -8.14% | -5.24% | -2.86% | -9.02% | -5.37% | 201% | 158301% |
| Class B | -1.68% | -6.54% | -6.37% | -2.36% | -8.44% | -7.96% | 201% | 163136% |
| Class C | -2.60% | -8.64% | -8.86% | -4.20% | -10.01% | -11.02% | 182% | 170245% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.03% | -7.02% | -6.45% | -2.88% | -8.44% | -7.78% | 198% | 165775% |
| Class D | -2.95% | -10.09% | -11.12% | -4.12% | -11.56% | -12.15% | 194% | 193561% |

It should be clarified that the model used in RA configuration is identical with the one in the previous work [3]. The performance difference comes from the reference index adjustment.

Conclusions

This contribution presents a deep-neural-network-based virtual reference frame generation (DRF) method for VVC. By applying the proposed method, additional BD rate saving can be achieved.

Reference

[] Kalluri T, Pathak D, Chandraker M, et al. FLAVR: Flow-agnostic video representations for fast frame interpolation. arXiv preprint arXiv:2012.08512, 2020.

[2] Z. Liu, X. Xu, S. Liu, Y. Guo, Z. Chen. AHG11: NN-based Reference Frame Interpolation for VVC Hierarchical Coding Structure. JVET-X0060, 24th Meeting, by teleconference, 6–15 Oct. 2021.

[3] Z. Liu, X. Xu, S. Liu, Y. Guo, Z. Chen. AHG11: NN-based Reference Frame Interpolation for VVC Hierarchical Coding Structure. JVET-Y0096, 25th Meeting, by teleconference, 9–18 Jan. 2021.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-Z2016, 26th Meeting, by teleconference, 20–29 Apr. 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.