| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0093-v1 |
| --- | --- |

| Title: | EE2-2.6: Combination test of Test 2.4 (on ARMC with refined motion) and Test 2.5 (on ARMC improvements) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Yang Wang, Kai Zhang, Na Zhang, Zhipin Deng, Li Zhang | Tel: Email: | guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Canon and Bytedance |  |  |

_____________________________

# Abstract

This contribution presents the results of the combination between Test 2.4 and Test 2.5 regarding some improvements around the process called Adaptive Reordering of the Merge Candidates list (ARMC).  Test 2.4 uses refined motion vectors as input of the ARMC process while Test 2.5 introduces a different criteria to order the motion vector candidates.

Compared to ECM-5.0, the respective average BDR gains and runtimes reported in this combination Test 2.6 are as follows:

-0.26%	-0.22%	-0.30%  —  99%, 104% for the RA configuration,

-0.42%	-0.67%	-0.70%	 — 100%, 104% for the Low Delay B configuration.

# Test 2.4 description

This Test 2.4 proposes to use refined motion vectors in ARMC as presented in JVET-Z0145 [4]. More specifically, instead of re-ordering the merge candidate list directly, each merge candidate in the merge candidate list is refined by using TM/multi-pass DMVR first and these refined motion vectors will be used in ARMC to re-order the merge candidate list. ARMC with refined motion is applied to template matching (TM) merge mode, adaptive DMVR mode, and template matching for advanced motion vector prediction (TM-AMVP) mode. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering. When template matching is used to derive the refined motion, the template size is set equal to 1. Only the above or left template is used during the motion refinement of TM when the block is flat (w > 2×h) or narrow (h > 2×w). TM is extended to perform 1/16-pel MVD precision. The first four merge candidates are reordered with the refined motion in TM merge mode. When constructing the AMVP list, an MVP candidate with a TM cost larger than a threshold, which is equal to five times of the cost of the first MVP candidate, is skipped.

# Test 2.5 description

## Diversity reordering

This Test 2.5 modifies the reordering process as initially proposed in JVET-Z0103 [3] . The goal is to create diversity inside the Merge candidates list, the proposed method looks for candidates which are too redundant in the rate-distortion (RD) sense. In the proposed algorithm, a candidate is considered as redundant if the cost difference between a candidate and its predecessor is inferior to a lambda value e.g. |D1-D2| < λ, where D1 and D2 are the costs obtained during the first ARMC ordering and λ is the Lagrangian parameter used in the RD criterion at encoder side.

The proposed algorithm is defined as the following:

- Determine the minimum cost difference between a candidate and its predecessor among all candidates in the list

- If the minimum cost difference is superior or equal to λ, the list is considered diverse enough and the reordering stops.

- If this minimum cost difference is inferior to λ, the candidate is considered as redundant and it is moved at a further position in the list. This further position is the first position where the candidate is diverse enough compared to its predecessor.

- The algorithm stops after a finite number of iterations (if the minimum cost difference is not inferior to λ).

This algorithm is applied to the Regular, TM, BM and Affine merge modes of ECM-5.0. A similar algorithm is applied to the Merge MMVD and sign MVD prediction methods which also use ARMC for the reordering.

The value of λ is set equal to the λ of the rate distortion criterion used to select the best merge candidate at the encoder side for low delay configuration and to the value λ corresponding to a another QP for Random Access configuration. A set of λ values corresponding to each signaled QP offset is provided in the SPS or in the Slice Header for the QP offsets which are not present in the SPS.

## Merge Mode modifications

The following modifications have been applied to regular Merge list:

- The same Motion vector threshold is used for Temporal, non-adjacent and other candidates.

- No ARMC reordering for sub-groups is performed

- Up to 4 pairwise candidates are added after the first ARMC reordering for regular, TM and BM Merge modes.

# Results

The proposed modifications of Test 2.4 and Test 2.5 have been merged into a single software corresponding to Test2.6. These modifications have been merged on top of the ECM-5.0 [1] and tested according to the common test conditions (CTC) for ECM-5.0 [2].

# Conclusion

In this contribution, some modifications regarding the ARMC process have been proposed by combining the Test 2.4 and the Test 2.5 in to a single Test 2.6. The results show that the coding efficiency gains of Test 2.4 and Test 2.5 are almost additive for the RA configuration.

# References

- ECM-5.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-5.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- G. Laroche and P. Onno,” EE2-2.5 related: More test results for ARMC with refined motion”, JVET-Z0103, Apr. 2022.

- Y. Wang, K. Zhang, N. Zhang, Z. Deng, L. Zhang “EE2-2.5 related: More test results for ARMC with refined motion”, JVET-Z1045, Apr. 2022.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).