| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0101-v1 |
| --- | --- |

| Title: | AHG9: On processing order in the neural-network post-filter activation SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Shao, Arjun Arora, Peng Yin, Sean McCarthy, Taoran Lu, Fangjun Pu, Walt Husak | Email: | {sean.mccarthy, pyin} @dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

The following syntax elements for the neural-network post-filter activation SEI message are proposed to be added to the next draft of Additional SEI messages for VSEI (JVET-Z2006). The neural-network post-filter activation SEI message was adopted during the 26th meeting of JVET in April 2022.

- nnpfa_independent_flag to indicate preference that the neural-network post-filter signalled in the SEI be either independent of other neural-network post-filters that may also be used for the current picture, or dependent on the output of one or more such neural-network post-filters

- nnpfa_num_preceding_nnpfa_ids_minus1 to indicate the number of neural-network post-filters on which the current neural-network post-filter may depend

- nnpfa_preceding_nnpfa_id[ i ] to specify the identifying number, nnpfa_id, of the i-th neural-network post-processing filter on which the current neural-network filter may depend

# Introduction

This contribution proposes syntax elements and corresponding semantics to convey information on the preferred dependencies, if any, between neural-network post-filters when more than one neural-network post-filter is active for the current picture. The authors advocate that the proposed syntax elements and corresponding semantics be added to the neural-network post-filter activation (nnpfa) SEI message in the next draft of Additional SEI messages for VSEI (JVET-Z2006 [1]).

# Problem statement

An encoder (i.e., content producer) may prefer to apply neural-network post-filters in a specific sequence when more than one neural-network post-filter is activated for the current picture in order to properly process the content and enable an adequate user experience. For example, a content producer may prefer that an output tensor of a luma-only neural-network post-filter be used to derive an input tensor of a luma-chroma neural-network post-filter, as illustrated in Figure 1A. The reverse order of processing, as illustrated in Figure 1B, could produce a different and undesired result. As another example, a content producer may prefer that an output tensor of a neural-network post-filter to increase the width or height of a decoded picture (nnpfc_purpose equal to 2, 3, or 4) be used to derive the input tensor of a neural-network post-filter to improve video quality (nnpfc_purpose equal to 1), as illustrated in Figure 2A. The reverse process, as illustrated in Figure 2B, could result in an undesired user experience.

Figure 1. Illustration of the effect of order of processing for a luma-only NN post filter and a luma-chroma NN post filter

Figure 2. Illustration of the effect of order of processing for a NN post filter to change the size of a picture and a NN post filter to enhance the quality of a picture

# Proposal

It is proposed to add three syntax elements and corresponding semantics to nnpfa SEI message as follows:

- nnpfa_independent_flag to indicate preference that the neural-network post-filter signalled in the SEI be either independent of other neural-network post-filters that may also be used for the current picture, or dependent on the output of one or more such neural-network post-filters

- nnpfa_num_dependencies_minus1 to indicate the number of neural-network post-filters on which the current neural-network post-filter may depend

- nnpfa_dependency_nnpfa_id[ i ] to specify the identifying number, nnpfa_id, of the i-th neural-network post-processing filter on which the current neural-network filter may depend

It is additionally proposed to add an informative note that different values of nnpfc_id might be specified by neural-network post-processing filter characteristics SEI messages to identify neural-network post-processing filters that have identical characteristics and different preferred order, as illustrated in Figure 3.

Figure 3. Illustration of use of different values of nnpfc_id to indicate a preferred processing order for neural-network post-filters that are otherwise identical

Changes relative to JVET-Z2006 are highlighted in yellow.

## Neural-network post-filter activation SEI message

### Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_id | ue(v) |
| nnpfa_independent_flag | u(1) |
| if( !nnpfa_independent_flag ) { |  |
| nnpfa_num_preceding_nnpfa_ids_minus1 | ue(v) |
| for ( i = 0; i <= nnpfa_num_preceding_nnpfa_ids_minus1; i++ ) |  |
| nnpfa_preceding_nnpfa_id[ i ] | ue(v) |
| } |  |
| } |  |

### Neural-network post-filter activation SEI message semantics

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture and conveys information on dependencies, if any, on other neural-network post-filters that may be present for the current picture.

The neural-network post-processing filter activation SEI message persists only for the current picture.

NOTE 1  – There may be several neural-network post-processing filter activation SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_id specifies that the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture.

nnpfa_independent_flag equal to 0 indicates preference that input to the neural-network post-processing filter with nnfpa_id should depend on the output of one or more other neural-network post-processing filters that pertain to the current picture and have nnpfc_id not equal to nnpfa_id. nnpfa_independent_flag equal to 1 indicates no preference. When only one neural-network post-filter activation SEI message is present for the current picture, the value of nnpfa_independent_flag should be equal to 1.

nnpfa_num_preceding_nnpfa_ids_minus1 plus 1 specifies the number of neural-network post-processing filters that pertain to the current picture that should precede, in processing order, the neural-network post-processing filter specified by nnpfa_id.

nnpfa_preceding_nnpfa_id[ i ] specifies that the neural-network post-processing filter specified by nnpfc_id equal to nnpfa_preceding_nnpfa_id[ i ] should precede, in processing order, the neural-network post-processing filter specified by nnpfa_id.

NOTE 2  – Different values of nnpfc_id might be specified by neural-network post-processing filter characteristics SEI messages to identify neural-network post-processing filters that have identical characteristics and different preferred order.

# References

- S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, “Additional SEI messages for VSEI (Draft 1)” JVET-Z2006, teleconference, Apr 2022

- S. McCarthy, A. Arora, T. Shao, P. Yin, T. Lu, F. Pu, W. Husak, “AHG9: Neural-network post-filtering SEI message” JVET-Z0121, teleconference, Apr 2022

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).