| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0103-v2 |
| --- | --- |

| Title: | EE2-related: CCLM with non-linear term |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Yan Ye Ru-Ling Liao Jie Chen | Tel: Email: | {sid.lxw, yan.ye, ruling.lrl, jiechen.cj }@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, a non-linear term of the down-sampled reconstructed luma value is introduced into the CCLM method. A chroma sample is predicted based on a linear term and a non-linear term of the reconstructed luma sample value, where the model parameters are derived from two lines left and above adjacent chroma samples and the corresponding down-sampled luma samples based on the LDL decomposition method used in EE2 Test 1.1a. It is reported that on top of ECM-5.0, when the CCLM_LT mode is replaced by the proposed method, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.12%, -0.17%, -0.37%, 101%, 101%} for AI and {-0.17%, -0.15%, -0.75%, 101%, 101%} for RA.

Introduction

In ECM-5.0 [1], the Cross Component Linear Model (CCLM) is used to reduce the cross-component redundancy, for which a chroma sample is predicted from the down-sampled collocated luma sample by a linear model as:
predCi,j= α∙recL'i,j+β

where predCi,j represents the predicted value of a chroma sample, recL'i,j represents the reconstructed value of the collocated luma samples which are down-sampled for the case of non-4:4:4 color format and the linear model parameters α and β are derived based on one line reconstructed samples that are adjacent to the current block by the linear minimum mean square error (LMMSE) method.

CCLM assumes that there is a linear correlation between luma component and chroma component in local texture. However, sometimes a linear model cannot accurately represent the relationship between the luma component and the chroma component.

Proposed method

It is proposed to introduce a non-linear term of the reconstructed luma sample value into the CCLM method, so that the chroma sample can be predicted by a non-linear model as:

predCi,j= α0∙(((recL'i,j)2+midValue)≫bitDepth)+α1∙recL'i,j+α2∙midValue

where the model parameters α0, α1 and α2 are derived from the reconstructed values of the adjacent samples based on the LDL decomposition method used in EE2 Test 1.1a [2]. For 10-bit content, the bitDepth is set equal to 10 and the midValue is set equal to 512.

The proposed method is applied to the CCLM_LT mode when slop adjustment factor is equal to 0 without any additional signaling. The top 2 rows and left 2 columns chroma samples and the corresponding down-sampled luma samples are used to derive the model parameters.

Simulation results

The proposed methods are implemented on top of ECM-5.0. The simulation results are tested under ECM test condition [2] with all intra and random access configurations as shown in Table 1.

Table 1. Simulation results of the proposed methods

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.46% |  | -0.35% |  |  | -1.57% |  | 101% |  |  | 101% |  |  |
| Class A2 | -0.05% |  | 0.02% |  |  | -0.17% |  | 101% |  |  | 101% |  |  |
| Class B | -0.06% |  | -0.38% |  |  | -0.27% |  | 101% |  |  | 100% |  |  |
| Class C | -0.04% |  | -0.12% |  |  | -0.16% |  | 102% |  |  | 101% |  |  |
| Class E | -0.04% |  | 0.11% |  |  | 0.18% |  | 101% |  |  | 101% |  |  |
| Overall | -0.12% |  | -0.17% |  |  | -0.37% |  | 101% |  |  | 101% |  |  |
| Class D | 0.00% |  | 0.02% |  |  | -0.16% |  | 101% |  |  | 101% |  |  |
| Class F | -0.12% |  | -0.23% |  |  | -0.39% |  | 100% |  |  | 101% |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.77% |  | -0.30% |  |  | -2.12% |  | 100% |  |  | 100% |  |  |
| Class A2 | -0.04% |  | -0.05% |  |  | -0.16% |  | 101% |  |  | 100% |  |  |
| Class B | -0.02% |  | -0.33% |  |  | -0.70% |  | 102% |  |  | 101% |  |  |
| Class C | -0.01% |  | 0.10% |  |  | -0.24% |  | 102% |  |  | 101% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.17% |  | -0.15% |  |  | -0.75% |  | 101% |  |  | 101% |  |  |
| Class D | -0.04% |  | 0.05% |  |  | -0.13% |  | 102% |  |  | 101% |  |  |
| Class F | -0.10% |  | -0.21% |  |  | -0.04% |  | 101% |  |  | 101% |  |  |

References

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- P. Astola, et. al, “EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, Jul. 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).