| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting: by teleconference, 13–22 July 2022 | Document: JVET-AA0110-v2 |
| --- | --- |

| Title: | AHG9: SEI message with sample phase indication for consistent rendering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes Sachin Deshpande | Tel: Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

# Abstract

An SEI message containing a sample phase indication is proposed. It is asserted to enable more consistent rendering, in particular when rendering at a higher resolution and when switching between multiple resolutions as is common in streaming scenarios where bandwidth may fluctuate.

Compared to JVET-Y0156-v1 presented and discussed at the 25th JVET meeting, this version includes an improved text description including the definition of persistence, as well as numerical examples. The second version of the proposal provides an updated text in response to the suggestions brought forward during the presentation of the proposal:

- Different denominators are now used for horizontal phase and vertical phase.

- Denominators are signalled using “minus1”.

- “cropped decoded output picture” is changed to “cropped decoded picture” to align with the definition in V.SEI.

# Problem Statement

When downsampling video content to generate multiple versions suitable for various bandwidth constraints, a low-pass filter is typically used. A phase shift may be introduced during downsampling, and the relative location between samples at different resolutions may not be known to a client consuming the video. When switching between multiple resolutions a slight geometric shift may be noticeable if the phase shift used in the upsampler does not mirror the phase shift of the downsampler. This issue was previously reported in [1].

Even when content is made available in a single resolution to a client, the client may upsample the content when the display has a higher resolution. Knowing the intended position of samples relative to the video surface also allows for more consistent rendering, which may be useful when multiple videos are rendered to a same screen and require precise alignment.

When rendering a decoded picture using a shader, for example using OpenGL, an offset may be added to texture coordinates to compensate for any phase shift. This is similar to offsets that may be added when accessing chroma samples to account for the position of chroma samples relative to luma samples.

During the presentation of JVET-Y0156-v1, the following questions were raised:

- Would it be useful to define denominator separately for horizontal and vertical?

- How about chroma? Is it enough to have the chroma position from VUI, and what happens if it is unspecified?

- Would it be necessary to signal a different offset depending on the position of the cropping window?

We analyzed these questions further. Question 1 is discussed and addressed in section 3.10. Question 2 is discussed and addressed in section 3.11. Question 3 is discussed and addressed in section 3.12.

# Proposed solution

An SEI message is proposed with the syntax and semantics described below.

## Phase indication SEI message

### Phase indication SEI message syntax

| phase_indication( payloadSize ) { | Descriptor |
| --- | --- |
| hor_phase_num | u(8) |
| hor_phase_den_minus1 | u(8) |
| ver_phase_num | u(8) |
| ver_phase_den_minus1 | u(8) |
| } |  |

### Phase indication SEI message semantics

The phase indication SEI message provides the decoder with information about the position of luma sampling locations in cropped decoded pictures relative to a rendering window. This information may be used by a decoder to ensure the correct spatial alignment of rendered pictures, for example when switching between picture resolutions.

Figure 1: The ratios ab and cd represent the horizontal and vertical locations of the luma samples (marked with x) relative to a rendering window. ab is equal to hor_phase_num ÷ ( hor_phase_den_minus1 + 1 ), and cd to ver_phase_num ÷ ( ver_phase_den_minus1 + 1 ).

Note: When the number of luma output samples in horizontal direction is equal to the width of the rendering window, and hor_phase_num ÷ ( hor_phase_den_minus1 + 1 ) is equal to ½, the picture is intended to be rendered without applying any horizontal phase shift. Correspondingly, when the number of luma output samples in vertical direction is equal to the height of the rendering window, and ver_phase_num ÷ ( ver_phase_den_minus1 + 1 ) is equal to ½, the picture is intended to be rendered without applying any vertical phase shift.

The phase indication SEI message applies to the current cropped decoded picture and persists for all subsequent pictures of the current layer in output order with the same value of ph_pic_parameter_set_id as the current picture until one or more of the following conditions are true:

- A new CLVS of the current layer begins

- The bitstream ends

- A picture in the current layer with an associated phase indication SEI message and the same value of ph_pic_parameter_set_id as the current picture is output and follows the current picture in output order.

hor_phase_num and hor_phase_den_minus1 specify the horizontal position of luma sampling locations relative to a rendering window. The horizontal position hor_phase_num ÷ ( hor_phase_den_minus1 + 1 ) is expressed in units of the horizontal distance between two horizontally adjacent luma sampling locations. The value of hor_phase_num shall be greater than or equal to 0 and less than or equal to hor_phase_den_minus1 + 1.

ver_phase_num and ver_phase_den_minus1 specify the vertical position of luma sampling locations relative to a rendering window. The vertical position ver_phase_num ÷ ( ver_phase_den_minus1 + 1 ) is expressed in units of the vertical distance between two vertically adjacent luma sampling locations. The value of ver_phase_num shall be greater than or equal to 0 and less than or equal to ver_phase_den_minus1 + 1.

Note: The phase indicators may be used during the rendering process. For example, texture coordinates may be offset by an amount proportional to the signaled horizontal and vertical phase indicators.

Note: The signalled phase indicators applies to the luma samples of the decoded pictures. The phase offset for chroma can be derived from the signalled phase indicators taking into account the chroma sample location relative to luma sample location as indicated by vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field. When the value of vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field, as applicable, are equal to 6 or are inferred to be equal to 6, the nominal vertical and horizontal relative locations of luma and chroma samples in pictures as shown in Figure 1 may be assumed.

# Examples

The ratio hor_phase_num ÷ phase_den represents the offset of the luma sampling positions relative to the rectangular window representing the picture area. This offset may typically be assumed to be ½ when capturing a picture. When resizing a picture, the offset associated with a resized picture may be different depending on the phase of the resizing filter being used. A coded picture may thus feature an offset different from ½. When resizing a picture for display, the resized picture should typically feature an offset of ½ to guarantee proper alignment of the displayed picture. A renderer may determine the proper phase shift to use in upsampling filters based on the offset associated with the decoded picture.

The ratio ver_phase_num ÷ phase_den is commonly equal to the ratio hor_phase_num ÷ phase_den, but these may differ, in particular, when different resizing scales are applied horizontally and vertically.

The mapping between the cases in [1] and the proposed syntax is given below:

| JVET-X0092 | hor_phase_num | ver_phase_num | phase_den |
| --- | --- | --- | --- |
| Unknown (0) | 0 | 0 | 0 |
| Co-sited (1) | b | b | 2a |
| Centered (2) | 1 | 1 | 2 |

Where the fraction ab represents the ratio between an original picture size where samples are “centered” relative to the image surface and a current picture size (downsampled).

## Resolutions for ABR streaming

The resolutions listed below with a 16:9 aspect ratio may be commonly used for streaming purposes.

| Width | Height | Ratio relative to 8K |
| --- | --- | --- |
| 7680 | 4320 | 1:1 |
| 3840 | 2160 | 1:2 |
| 2560 | 1440 | 1:3 |
| 1920 | 1080 | 1:4 |
| 1280 | 720 | 1:6 |
| 960 | 540 | 1:8 |
| 854 | 480 | 1:9 |
| 768 | 432 | 1:10 |
| 640 | 360 | 1:12 |
| 480 | 270 | 1:16 |
| 426 | 240 | 1:18 |
| 416 | 234 | 13:240 |

## Resizing filter: centered

A centered resizing filter is commonly used – for example, in widely available software packages such as ffmpeg and ImageMagick, or in graphics rendering engines such as OpenGL. In this case the offset is always equal to ½ (assuming the source picture has an offset equal to ½).

## Resizing filter: co-sited (top-left)

This approach is used in scalable coding such as SHVC and VVC. One advantage of using co-sited filters is the fact that in the upsampling process, some samples may simply be copied from the lower-resolution version. This reduces overfiltering and potentially complexity. Each column represents one of multiple base resolutions from which lower resolutions are derived.

| Resolution | 7680x4320 | 3840x2160 | 2560x1440 | 1920x1080 | 1280x720 |
| --- | --- | --- | --- | --- | --- |
| 7680x4320 | 1/2 |  |  |  |  |
| 3840x2160 | 1/4 | 1/2 |  |  |  |
| 2560x1440 | 1/6 | 1/3 | 1/2 |  |  |
| 1920x1080 | 1/8 | 1/4 | 3/8 | 1/2 |  |
| 1280x720 | 1/12 | 1/6 | 1/4 | 1/3 | 1/2 |
| 960x540 | 1/16 | 1/8 | 3/16 | 1/4 | 3/8 |
| 852x480 | 1/18 | 1/9 | 1/6 | 2/9 | 1/3 |
| 768x432 | 1/20 | 1/10 | 3/20 | 1/5 | 3/10 |
| 640x360 | 1/24 | 1/12 | 1/8 | 1/6 | 1/4 |
| 480x270 | 1/32 | 1/16 | 3/32 | 1/8 | 3/16 |
| 426x240 | 1/36 | 1/18 | 1/12 | 1/9 | 1/6 |
| 416x234 | 1/37 | 13/240 | 13/160 | 13/120 | 13/80 |

## Resizing filter: co-sited (minimize shift)

In this approach, the same filters are used as in the previous case. However, the choice of a co-sited sample is guided by a desire to keep sampling centered with respect to image boundaries, to the extent possible. Each column represents one of multiple base resolutions from which lower resolutions are derived.

| Resolution | 7680x4320 | 3840x2160 | 2560x1440 | 1920x1080 | 1280x720 |
| --- | --- | --- | --- | --- | --- |
| 7680x4320 | 1/2 |  |  |  |  |
| 3840x2160 | 3/4 | 1/2 |  |  |  |
| 2560x1440 | 1/2 | 1/3 | 1/2 |  |  |
| 1920x1080 | 5/8 | 3/4 | 3/8 | 1/2 |  |
| 1280x720 | 7/12 | 1/2 | 3/4 | 1/3 | 1/2 |
| 960x540 | 9/16 | 5/8 | 9/16 | 3/4 | 3/8 |
| 852x480 | 1/2 | 5/9 | 1/2 | 2/3 | 1/3 |
| 768x432 | 11/20 | 1/2 | 9/20 | 3/5 | 3/10 |
| 640x360 | 13/24 | 7/12 | 5/8 | 1/2 | 3/4 |
| 480x270 | 17/32 | 9/16 | 15/32 | 5/8 | 9/16 |
| 426x240 | 19/36 | 1/2 | 7/12 | 5/9 | 1/2 |
| 416x234 | 53/103 | 39/80 | 91/160 | 13/24 | 39/80 |

## Resizing filter: co-sited, then centered

In this case, the second-highest resolution is initially generated using the co-sited filter by a first system. Lower resolutions are then derived with a centered filter by a second system.

| Resolution | 7680x4320 | 3840x2160 | 2560x1440 | 1920x1080 | 1280x720 |
| --- | --- | --- | --- | --- | --- |
| 7680x4320 | 1/2 |  |  |  |  |
| 3840x2160 | 1/4 | 1/2 |  |  |  |
| 2560x1440 | 1/3 | 1/3 | 1/2 |  |  |
| 1920x1080 | 3/8 | 3/8 | 3/8 | 1/2 |  |
| 1280x720 | 5/12 | 5/12 | 5/12 | 1/3 | 1/2 |
| 960x540 | 7/16 | 7/16 | 7/16 | 3/8 | 3/8 |
| 852x480 | 4/9 | 4/9 | 4/9 | 7/18 | 7/18 |
| 768x432 | 9/20 | 9/20 | 9/20 | 2/5 | 2/5 |
| 640x360 | 11/24 | 11/24 | 11/24 | 5/12 | 5/12 |
| 480x270 | 15/32 | 15/32 | 15/32 | 7/16 | 7/16 |
| 426x240 | 17/36 | 17/36 | 17/36 | 4/9 | 4/9 |
| 416x234 | 96/203 | 96/203 | 96/203 | 107/240 | 107/240 |

## Resizing filter with limited number of phases

In this case, the desire is to perform resampling as close as possible to the centered phase but due to limitations in the number of phases supported by the downsampler, a different phase position is used. As an example, resampling from 1080 to 480 results in a scale factor of 9 to 4 (i.e., 2.25) which requires 1/8th sample position for downsampling (positions 0.625, 2.875, 5.125, 7.375, 9.625, etc. relative to the original samples). However, if only ¼th sample phase precision is supported by the downsampler, the offset can be moved by 0.125 (in the original resolution) resulting in sampling positions that are all multiples of ¼ (positions 0.5, 2.75, 5, 7.25, 9.5, etc.). The signalled values for ver_phase_num and phase_den would in this case be 4 and 9, respectively.

## RPR use

While RPR defines filtering conventions to upsample and downsample, there is no guarantee that the coded pictures were actually generated using a filter with the same phase shift as the RPR filter. A player may thus not assume a “top-left co-sited” relationship when a resolution switch occurs between two consecutive pictures in a CLVS. Also, when using RPR, an encoder may choose custom resolutions at a finer granularity. The set of possible resolutions may thus be larger than the one listed in the use cases above.

## Rendering resolution

The proposed indication does not contain any information about “original resolution”, “suggested rendering resolution”, or “nominal rendering resolution”. This is because no such information is needed. The offsets provided in the SEI message will be sufficient for a renderer to determine the correct phase to use, regardless of the size of the rendering window.

One way to think of it is that the phase indication SEI lets you determine how much your decoded video has been phase shifted compared to “centered" scaling. Once you have determined the phase shift relative to “centered scaling” the decoded video can be shifted to that position and then correctly scaled to any resolution. This can be performed in a two-step process, but more commonly (and more efficiently) it would be performed in a single step.

When the resolution of the rendering window is equal to the resolution of the decoded video and ver_phase_num ÷ phase_den and hor_phase_num ÷ phase_den are both equal to ½, the video can be rendered as is, with no phase shift applied.

However, when the resolution of the rendering window is equal to the resolution of the decoded video but at least one of ver_phase_num ÷ phase_den or hor_phase_num ÷ phase_den is not equal to ½, a phase shift must be applied to the video in order to render the video correctly (without shifting the position of the content).

## Origin of rendering target

It should be noted that the origin of the rendering target is defined at the edge of the first pixel of the rendering window and not at the center of the first pixel of the rendering window. The center of the first pixel of the rendering window is positioned at (0.5, 0.5) and the center of the last pixel of the rendering window is positioned at (W-0.5, H-0.5) where W and H are the width and height of the rendering window, respectively. This is done to make definitions and syntax as clear and simple as possible. It can be noted that the origin of the rendering window is similarly defined in common rendering frameworks such as for example Direct3D 10 [2] and OpenGL [3].

## Denominator

The proposed phase indication contains a single denominator used for both vertical and horizontal direction, and it is proposed to use 8 bits precision for the denominator. It can be noted that in all examples in the tables above, the denominator is less than 256. For cases where vertical and horizontal phase offsets are different (e.g., 1/3 and 1/4) a common denominator could typically be found (e.g., 1/12). It is only in extreme cases with large different denominators for vertical and horizontal phase offsets that a common denominator less than 256 cannot be found. However, for cases that cannot be expressed with exact precision, the phase offsets can be rounded with an error in the order of 1/512th of a sample or less.

## Chroma phase

It can be noted that the SEI message only provides information about the phase position for luma samples. This is a deliberate design choice made in order to not interfere or provide potentially inconsistent information relative to chroma location information provided elsewhere (such as  vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field).

When chroma location information is signalled, inferred or assumed in relation to the luma sample positions, the correct phase offsets for chroma can be derived from ver_phase_num ÷ phase_den and hor_phase_num ÷ phase_den.

If chroma location information is not explicitly signalled we propose that the same assumption is suggested regarding chroma location as is already present in the VUI section of the VVC standard: “When the value of vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field, as applicable, are equal to 6 or are inferred to be equal to 6, the nominal vertical and horizontal relative locations of luma and chroma samples in pictures as shown in Figure 1 may be assumed.”.

## Cropping window

The proposed phase indication applies to cropped decoded output pictures. If a cropping window is defined, it is the samples in the cropped output pictures that are relevant for rendering. The phase position of the full (uncropped) decoded picture will be a fixed offset to that, since the cropping window is defined in terms of integer number of samples, but that will not be of any importance to a renderer that only renders the cropped decoded output picture.

# References

- J.Samuelsson, A.Tourapis, D.Podborski, K.Rapaka, D.Singer, “Down-sample phase indication (SEI message),” JVET-X0092, October 2021.

- Coordinate Systems (Direct3D 10)
https://docs.microsoft.com/en-us/windows/win32/direct3d10/d3d10-graphics-programming-guide-resources-coordinates

- gl_FragCoord
https://www.khronos.org/registry/OpenGL-Refpages/gl4/html/gl_FragCoord.xhtml

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).