| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0116 |
| --- | --- |

| Title: | EE2-2.3: MMVD and Affine MMVD Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Na Zhang, Li Zhang (Bytedance) 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com zhangna.cynthia@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports results for EE2-2.3.

A template matching based reordering method for extended MMVD (TMR-EMMVD) was proposed (JVET-Y0067) and adopted into ECM-4.0.

In test EE2-2.3 (JVET-Z0142), a simple extension of JVET-Y0067 is proposed with the following aspects:

- One sided MMVD position is added for the bi-prediction cases.

- Extra base positions are added depending on the neighboring blocks.

On top of the ECM-5.0, simulation results of the proposed method are reported as below:

Test 2.3:

RA: -0.14%, -0.10%, -0.23%, 102%, 100%; LDB: -0.31%, - 0.43%, -0.35%, 103%, 102%

(Additional information) No MMVD base increase on Test 2.3:

RA: -0.10%, -0.11%, -0.15%, 100%, 100%; LDB: -0.18%, - 0.18%, -0.24%, 101%, 102%

# Introduction

In JVET-X0085 [1], a template matching based reordering method for extended MMVD (TMR-EMMVD) was proposed and adopted into ECM-4.0, which improves the coding gain of MMVD and affine MMVD. In JVET-Z0142 [2], an extension of TMR-EMMVD is proposed.

# Proposed methods

In this document we propose an extension of TMR-EMMVD with the following aspects:

Aspect #1:

For a bi-prediction MMVD base in VVC or ECM, the MVD is added to one list, but derived from the first list MVD with a possible scaling and mirroring depending on the POC differences for the other list, known as two-sided MMVD.

On top of the current design, we propose one-sided MMVD (or affine MMVD), where individual MVD is used for each list independently, where a non-zero MVD is applied for list X, but a zero MVD is applied for list (1-X). We also propose to remove the scaling process for the two-sided MMVD. Also, candidates with similar costs are pruned.

Aspect #2:

We propose to increase the number of the bases for MMVD from 2 to 3.

For affine MMVD, we also propose to increase the bases from 1, to up to 3, depending on the neighboring block affine flags.

# Experimental Results

The proposed methods have been implemented on top of the ECM-5.0 [3]. Testing results of EE2-2.3 under RA/LDB configurations are summarized in Table 1. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheets.

In addition, we also test the performance of EE2-2.3, without increasing the number of the bases for MMVD, as shown in Table 2.

Table 1: Performance of the 2.3

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.26% | -0.32% | 102% | 103% |  |  |  |  |  |
| Class A2 | -0.19% | -0.17% | -0.30% | 102% | 101% |  |  |  |  |  |
| Class B | -0.13% | -0.02% | -0.20% | 101% | 99% | -0.33% | -0.60% | -0.37% | 104% | 104% |
| Class C | -0.12% | -0.04% | -0.14% | 103% | 100% | -0.32% | -0.34% | -0.38% | 102% | 101% |
| Class E |  |  |  |  |  | -0.24% | -0.25% | -0.25% | 102% | 100% |
| Overall | -0.14% | -0.10% | -0.23% | 102% | 100% | -0.31% | -0.43% | -0.35% | 103% | 102% |
| Class D | -0.08% | -0.11% | 0.00% | 102% | 102% | -0.31% | 0.75% | 0.04% | 106% | 107% |
| Class F | -0.11% | -0.02% | 0.04% | 101% | 98% | -0.21% | -0.39% | -0.36% | 103% | 98% |

Table 2: Performance of the 2.3 without MMVD Base increase

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.07% | -0.23% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.12% | -0.13% | -0.10% | 100% | 100% |  |  |  |  |  |
| Class B | -0.10% | -0.16% | -0.13% | 101% | 100% | -0.21% | -0.36% | -0.52% | 102% | 102% |
| Class C | -0.09% | -0.16% | -0.15% | 99% | 100% | -0.18% | -0.22% | -0.15% | 100% | 101% |
| Class E |  |  |  |  |  | -0.15% | 0.17% | 0.10% | 101% | 104% |
| Overall | -0.10% | -0.11% | -0.15% | 100% | 100% | -0.18% | -0.18% | -0.24% | 101% | 102% |
| Class D | -0.08% | -0.30% | 0.07% | 100% | 99% | -0.34% | 0.25% | 0.10% | 99% | 104% |
| Class F | -0.10% | -0.03% | 0.05% | 99% | 100% | -0.33% | -0.46% | -0.38% | 100% | 99% |

- Conclusions

In this contribution, an extension of TMR-EMMVD is proposed. With negligible impact on complexity, it brings reasonable gain.  It is recommended to adopt the proposed method in ECM.

# References

[1] M. Salehifar, Y. He, K. Zhang, N. Zhang, L. Zhang, “Non-EE2: Template Matching-based Reordering for Extended MMVD Design,” JVET-X0085, Oct. 2021.

[2] M. Salehifar, Y. He, K. Zhang, N. Zhang, L. Zhang, “On MMVD and Affine MMVD Extension,” JVET-Z0142, April 2022.

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-5.0

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).