| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0119-v1 |
| --- | --- |

| Title: | EE2-1.4a related: Modifications of Spatial GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes two modifications to the SGPM method presented in the EE2-1.4a test. One modification is to extend SGPM to smaller blocks, and the other is to apply an adaptive SGPM blending in the transition region between the two partitions. It is reported that with these modifications, on top of ECM-5.0, the simulation results are as follows:

AI {x% Y, x% U, x% V, x% EncT, x% DecT}

RA {x% Y, x% U, x% V, x% EncT, x% DecT}

Introduction

GPM is an inter-prediction tool in the VVC standard. It combines two uni-directional predictions with a geometrical partitioning mask. In ECM, GPM has been further extended and optimized, which now supports inter-inter and inter-intra partitions. However, GPM with two intra partitions is still missing.

In JVET-Z0124 [1], a method named Spatial GPM (SGPM) was proposed to extend GPM in intra prediction. This new intra-coding tool partitions a coding block into two parts and generates two corresponding intra prediction modes. To efficiently express the partition and associated prediction information in the bit-stream, SGPM employs a template-reordered candidate list, where each candidate in the list comprises a combination of the partition mode and two intra prediction modes, and only signals the candidate index in the bit-stream. The SGPM method was taken into EE2 in JVET Z meeting and further studied in [2].

Proposed method

Extend SGPM to smaller blocks

In EE2-1.4, SGPM can be applied to the blocks whose width and height meet the following restrictions:

- 8<=width<=64

- 8<=height<=64

- width<height*8

- height<width*8

In this contribution, SGPM is extended to smaller blocks. The modified restrictions can be described as follows:

- 4<=width<=64

- 4<=height<=64

- width<height*8

- height<width*8

- width*height>=32

i.e., SGPM can be further applied to 4x8, 8x4, 4x16 and 16x4 blocks.

Adaptive SGPM blending

In EE2-2.7 [3], an adaptive GPM blending scheme is investigated. In this contribution, an adaptive SGPM blending scheme is proposed, where the blending width of SGPM is directly decided by the block size. No signalling is needed for the proposed adaptive blending.

Figure 1 Example of blending weight

Assuming the original blending width in VVC and ECM is τ, as shown in Figure 1, the proposed adaptive SGPM blending can be described as follows:

- If min(width, height)==4, 1/2 τ is selected

- else if min(width, height)==8, τ is selected

- else if min(width, height)==16, 2 τ is selected

- else, 4 τ is selected

In addition, similar to EE2-2.7, the range of the weights is increased from [0, 8] to [0, 32] to accommodate the increased width of the SGPM blending area.

Test results

The proposed method was implemented on top of the ECM-5.0 [4] software and tested based on the common test condition as specified in JVET-Y2017 [5]. To get a better trade-off between coding efficiency and encoding complexity, the number of RDO for SGPM is further reduced from EE2-1.4. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ECM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.19% | -0.10% | -0.13% | 114% | 103% |
| Class C | -0.23% | -0.17% | -0.22% | 112% | 104% |
| Class E | -0.32% | -0.30% | -0.25% | 112% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.12% | 0.14% | -0.28% | 112% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | ECM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.09% | -0.01% | -0.32% | #DIV/0! | #DIV/0! |
| Class C | -0.12% | 0.10% | 0.01% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.06% | 0.12% | 0.24% | #DIV/0! | #DIV/0! |
| Class F | -0.09% | -0.24% | -0.23% | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! |

Conclusion

This contribution proposes two modifications to the SGPM method in the EE2-1.4a test. One modification is to extend SGPM to smaller blocks, and the other is to apply an adaptive SGPM blending in the transition region between the two partitions. It is reported that with these modifications, on top of ECM-5.0, the simulation results are XXX. It is recommended to adopt the proposed method in ECM software.

References

[1] F. Wang, Y. Yu, H. Yu, D. Wang, “Non-EE2: Spatial GPM”, JVET-Z0124, April 2022.

[2] F. Wang, Y. Yu, H. Yu, D. Wang, “EE2-1.4: Spatial GPM”, JVET-AA0118, July 2022

[3] Y. Kidani, H. Katou, K. Unno, K. Kawamura (KDDI), N. Yan, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, X. Wang (Kwai), “EE2-2.7: GPM adaptive blending (JVET-Z0059, JVET-Z0137)”, JVET-AA0058, July 2022

[4] ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

[5] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).