| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0123-v2 |
| --- | --- |

| Title: | [AHG-7] Update on gaming sequences from InterDigital |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Poirier, S. Puri, G. Martin-Cocher E. Faivre d’Arcier | Email: | first.last@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes an update on the new gaming sequences initially proposed in JVET-Y0041. The proposed sequences were created with Unity game engine and include depth and optical flow information. During former visual sessions, some issues were identified for some clips. This contribution completes JVET-Y0041 with updated versions of some content and more detailed information about the content, based on classification recommendations from JVET-AA0046. The compression results along with example bistreams using VTM-17.0 and ECM-5.0 are not available but will be provided in the next meeting.

The contribution proposes to create a new class (class G) of content for gaming sequences, corresponding to cloud gaming and game casting use cases.

# Rational

Game engines provide access to various frame buffers and allow to retrieve information such as Depth and Motion Vector or optical flow. Mobile devices and other accessories such as active cameras are increasingly making use of a variety of sensors, which can be used in video conferencing, telepresence and other industrial scenarios. This information could be useful to enhance the efficiency of compression tools and should particularly be evaluated in low delay / Low Latency Controlled Complexity (LLCC) configurations.

# Sequences creation process

The sequences were created using the Unity game engine to load and play several games. We captured the frame buffers using Unity Recorder and custom post-processing shaders in both the built-in render pipeline and the High-Definition Render Pipeline. We captured the color buffer into an RGBA16 Signed Float texture as well as the depth and optical flow in a second similar texture (with the non-linear depth info encoded into the R channel, and the [X,Y] coordinates of the motion vectors into the [G,B] channels). The textures were recorded at 60FPS in 1080p and 4K resolutions and written into uncompressed EXR 16 bit images.

For each clip, two YUVs sequences were then generated. The first YUV sequence (4:2:0 10 bits) corresponds to the texture. The second YUV sequence (4:4:4 16 bits) corresponds to depth and motion. The depth is stored in the Y channel and converted to 14 bits to avoid high bit-depth coding, vertical and horizontal motion vectors are stored at quarter pixel precision in U/V channels for the corresponding texture frame.

# Sequences description

All sequences are available on the following link:

https://www.interdigital.com/data_sets/gaming-sequences

### 3D_game_kit Sequences description

Two 10 seconds HD sequences representing two different levels in the 3D_game_kit and one 10 seconds 4K sequence were generated from the Unity Asset 3D game kit available at https://assetstore.unity.com/packages/templates/tutorials/3d-game-kit-115747.

3D_game_kit_Level-1_1920x1080_60p_420_10bits

Figure 2: 3D -game kit level-1 screenshot

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  |  |
| 3rd-person perspective |  | X |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  | X |
| Vehicles |  |  |
| Naturalistic rendering |  |  |
| Synthetic rendering |  | X |

3D_game_kit_Level-2_1920x1080_60p_420_10_bits

Figure 3: 3D_game_kit_level-2 screenshot

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  |  |
| 3rd-person perspective |  | X |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  | X |
| Vehicles |  |  |
| Naturalistic rendering |  |  |
| Synthetic rendering |  | X |

3D_game_kit_3840x2160_60p_420_10bits

Figure 4: 3D_game_kit_4K

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  |  |
| 3rd-person perspective | X |  |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters | X |  |
| Vehicles |  |  |
| Naturalistic rendering |  |  |
| Synthetic rendering | X |  |

### Darktree Sequence description

Darktree is a typical first-person shooter (FPS) type of game in a realistic environment generated from the Unity Asset available at:

https://assetstore.unity.com/packages/templates/systems/darktree-fps-v1-4-142383

Darktree_1920x1080_60p_420_10_bits

Figure 5 : DarkTree screenshot

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  | X |
| 3rd-person perspective |  |  |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  | X |
| Vehicles |  |  |
| Naturalistic rendering |  | X |
| Synthetic rendering |  |  |

### Fontainebleau Sequence description

Two 10 seconds HD sequences representing two different modes (cinematic and first person view) in the Fontainebleau demo and one 10 seconds 4K sequence were generated from the Unity Asset available at https://github.com/Unity-Technologies/FontainebleauDemo

The sequences represent the Fontainebleau forest under different lighting conditions.

Fontainebleau_Cinematic_1920x1080_60p_420_10_bits

Figure 6: Fontainebleau-Cinematic

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  | X |
| 3rd-person perspective |  |  |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  |  |
| Vehicles |  |  |
| Naturalistic rendering |  | X |
| Synthetic rendering |  |  |

Fontainebleau_FPV_1920x1080_60p_420_10_bits

Figure 7: Fonctainebleau_FPV

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  | X |
| 3rd-person perspective |  |  |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  |  |
| Vehicles |  |  |
| Naturalistic rendering |  | X |
| Synthetic rendering |  |  |

Fontainebleau_Cinematic_3840x2160_60p_420_10_bits

Figure 8: Fontainebleau_4K

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective | X |  |
| 3rd-person perspective |  |  |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  |  |
| Vehicles |  |  |
| Naturalistic rendering | X |  |
| Synthetic rendering |  |  |

### Racing Sequence description

This car racing 10 second-long 4K sequence is generated from a Unity Project from Pablo Labs. A video of the project is available at: https://www.youtube.com/watch?v=19vUeWcoD6I and the assets at https://www.mediafire.com/file/fvv2dpakkih4mum/Assets.zip/file.

The sequence contains textual overlays.

Racing _3840x2160_60p_420_10bits

Figure 9 : Racing screenshot

| Feature | 3840×2160 | 1920×1080 |
| --- | --- | --- |
| 1st-person perspective |  |  |
| 3rd-person perspective | X |  |
| Other perspectives (name them) |  |  |
| Scores |  |  |
| Chat |  |  |
| Graphical elements |  |  |
| Persons / characters |  |  |
| Vehicles | X |  |
| Naturalistic rendering |  |  |
| Synthetic rendering | X |  |

# Simulation results

The proposed sequences have been encoded using VTM-17.0 and ECM-5.0 in LDB Low Latency and controlled complexity configuration.

Table 1: ECM-5.0 vs VTM-17.0 for the proposed sequences

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-3.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class Gaming HD | -9.52% | -29.60% | -28.17% | 507% | 593% | 4% |
| Class Gaming 4K | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

# Copyright

This video sequences may only be used for the purpose of developing, testing and promulgating technology standards developed by a standardization group (“Purpose”), under InterDigital ("InterDigital") owned or controlled copyrights, by individual(s) or organization(s) that participates, contributes or is part of such standardization groups ("User(s)").

- The 3D_game_kit sequences contain the 3D-game-kit Unity assets created by Unity Technologies available at https://assetstore.unity.com/packages/templates/tutorials/3d-game-kit-115747.

- The racing sequence contains the “Unity Racing Game” Unity assets created by Pablo labs available at https://www.mediafire.com/file/fvv2dpakkih4mum/Assets.zip/file .

- The Fontainebleau sequences contain the FontainebleauDemo Unity assets created by Unity Technologies available at https://github.com/Unity-Technologies/FontainebleauDemo.

- The DarkTree sequence contains the DarkTree DPS v1.4 Unity assets created by DarkTreeDevelopment available at  https://assetstore.unity.com/packages/templates/systems/darktree-fps-v1-4-142383.

All these sequences are licensed by their respective owners and it is the User responsability to comply with the license terms that are provided by such owner. InterDigital claims no ownership nor liability on these assets.

RESTRICTIONS: No license whatsoever is implied except from this expressed limited personal, non-sublicensable, non-transferrable authorization. In particular no right under any patent of InterDigital or its affiliates is granted.

Any other use is strictly prohibited. This video cannot be distributed (freely or not) or otherwise disposed of or made available (via internet or otherwise), broadcasted, disclosed to third parties (except to other User(s) who agreed with the  present terms and conditions, the "Secondary Users"), used for providing any services to third parties or for any commercial use, nor copied (except as technically necessary for the Purpose), modified, adapted, translated, exchanged, publicly performed (even for research, demonstration or educational purposes), integrated, without limitation. This video and all intellectual property rights, titles and interests therein, are and remain the exclusive property of InterDigital or its affiliates or, regarding the Unity assets, of its legitimate owner.

NO WARRANTY: This video is supplied "as is" and without any warranty of any kind, expressed or implied, including but not limited to any warranty for accuracy or performance, fitness for a general or particular purposes, or any warranty arising by statute or by law, or non-infringement of any third parties's rights. In no event will InterDigital be liable for damages of any kind, including without limitation, any special, indirect, incidental, or consequential damages even if InterDigital has been advised of the possibility of such damages.

InterDigital may terminate this license it at any time for any reason upon notice to User(s). In such situation, the relevant User(s) shall inform any Secondary User of such termination and make sure that such Secondary User shall cease to use the video.

This authorization is governed by the laws of France without regard to conflict of laws. It shall be exclusively submitted to the first instance civil court of Paris, France in the event of a dispute not settled amicably.

BY USING THIS VIDEO USER(S) AGREES AND ACCEPTS THE ABOVE MENTIONED TERMS AND CONDITIONS. IF USER(S) DOES NOT AGREE TO THESE TERMS AND CONDITIONS, IT SHALL NOT USE THIS VIDEO. DO NOT REMOVE OR MODIFY INTERDIGITAL OR ITS AFFILIATES' NAMES, SIGNS, LOGOS, NOTICES.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.