| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0124-v1 |
| --- | --- |

| Title: | Non-EE2: Enable AmvpMerge mode on scaled reference pictures when DMVD is disabled |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to enable AmvpMerge mode when DMVD is disabled and RPR is enabled. The proposed method was implemented on top of ECM-5.0 and tested with EE2 RPR configuration settings 1.5x and 2.0x with DMVD additionally disabled in both anchor and test. The test results are summarised as follows:

RA 1.5x: -0.14% (Y), -0.19% (U), -0.22% (V), 101% (EncT), 100% (DecT)

RA 2.0x: -0.10% (Y), -0.15% (U), -0.09% (V), 101% (EncT), 100% (DecT)

# Introduction and proposal

In ECM-5.0, AmvpMerge mode is applied when bilateral matching can be used and for the reference pictures having the same resolution as the current picture. However, when DMVD is disabled, the bilateral matching is not applied but AmvpMerge mode is disabled for RPR. Since the bilateral matching or any other decoder side MV refinement is not used for AmvpMerge mode, this node which is just a regular bi-prediction signalled using merge and AMVP syntax, can be enabled with RPR as any other bi-prediction.

It is proposed to enable AmvpMerge mode with RPR when DMVD is disabled. The proposed method was submitted as a merge request 162 to ECM software.

# Simulation results

The simulation was conducted under the common test conditions as defined in [2] with RPR enabled and DMVD disabled. The anchor software is ECM-5.0 with fixes from ECM master branch related to DMVD off and RPR.

Two RPR configuration setting are tested:

| RPR setting 1.5x | RPR setting 2.0x |
| --- | --- |
| (Resolution switch after every 16 frames) | (Resolution switch after every 16 frames) |
| ScalingRatioHor : 1.5 | ScalingRatioHor : 2.0 |
| ScalingRatioVer : 1.5 | ScalingRatioVer : 2.0 |
| FractionNumFrames : 1.0 | FractionNumFrames : 1.0 |
| RPR : 1 | RPR : 1 |
| UpscaledOutput : 2 | UpscaledOutput : 2 |
| SwitchPocPeriod : 16 | SwitchPocPeriod : 16 |

The simulation results for random access configuration are as follows:

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 with RPR enabled and DMVD disabled |  |  |  |  |  |  |  |  |  |
|  | RPR setting 1.5x |  |  |  |  | RPR setting 2.0x |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.16% | -0.16% | 101% | 100% | -0.11% | -0.26% | -0.21% | 101% | 100% |
| Class A2 | -0.16% | -0.27% | -0.22% | 101% | 100% | -0.10% | -0.02% | -0.13% | 101% | 100% |
| Class B | -0.15% | -0.23% | -0.28% | 101% | 100% | -0.08% | -0.20% | -0.11% | 101% | 100% |
| Class C | -0.09% | -0.11% | -0.19% | 101% | 100% | -0.10% | -0.09% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.19% | -0.22% | 101% | 100% | -0.10% | -0.15% | -0.09% | 101% | 100% |
| Class D | -0.05% | 0.19% | 0.05% | 101% | 100% | -0.02% | 0.05% | -0.10% | 101% | 100% |
| Class F | 0.00% | -0.05% | 0.00% | 101% | 100% | -0.08% | -0.01% | -0.10% | 101% | 100% |

# References

[1] ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[3] H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE), “Non-EE2: AmvpMerge without decoder side mv refinement process”, JVET-Z0085, Apr. 2022.

[4] Merge request 162, “AmvpMerge mode enabling check when RPR is enabled and DMVD is disabled”

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).