| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0131-v3 |
| --- | --- |

| Title: | EE1-related: CNN based in-loop filtering with large activation layer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Samuel Eadie Muhammed Coban Marta Karczewicz | Email: | hongtaow@qti.qualcomm.com seadie@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, an alternative network structure which is derived based on EE1-1.5 and EE1-1.6 is studied. The network studied in EE1-1.6 is used as baseline and the network backbone is modified to use residue blocks with large activation layers. Experimental results reportedly show 11.25 %, 23.82 % and 25.22 % BD rate saving for RA and 7.78%, 20.87 %, 22.09 % BD rate saving for AI, for Y, Cb and Cr components, respectively.

# Introduction

In the JVET-Z meeting, two neural-network (NN) based in-loop filtering solutions were conditionally adopted to the common software basis [1][2]. The two solutions are being verified as EE1-1.5 and EE1-1.6 in this meeting cycle. This contribution studies an alternative NN structure which is developed on the common basis. The solution that was proposed in [1] is used as a baseline and filter blocks with large activation layers that are similar to those in [2] and [3] are used to compose the backbone of the network.

Figure 1 Network structure

The structure of the residue block used in this contribution is shown inFigure 1. The number of ResBlocks (N), number of channels (K, M) are configured to be the same as those in [2] (N=32, K=64, M=160).

The input of the network is same as [1], which includes reconstruction, prediction, boundary strengths, and QP (split is additionally taken as input in intra slice models). On the encoder side, the rate-distortion optimization to select the input QP value for the luma/chroma models is performed at the slice/block level. Chroma components (Cb, Cr) share the same QP value. And residual scaling is performed to combine the output of deblocking filter and NN filter as follows:

RRefine = (RNN-RDB)×w + RDB=w×RNN + (1-w)×RDB

## Inference

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.9M/model (4 models) |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 7.8MB/model, 4 models in total |
|  | Multiply Accumulate (kMAC/pixel) | 633 (picture basis) / 802 (block basis) |
| Optional |  |  |
|  | Total Conv. Layers | 105 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | ~122h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

Simulations are performed based on the common test conditions for neural network-based video coding technology[4]. The results reportedly show 11.25 %, 23.82 % and 25.22 % BD rate saving for RA, 9.84%, 20.28%, 21.36% for LB and 7.78 %, 20.87 %, 22.09 % BD rate saving for AI, for Y, Cb and Cr components, respectively.

Table 1 Simulation results

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | EncT | DecT CPU |  | bit DIFF |
| Class A1 | -10.76% |  | -18.43% |  | -20.85% |  | 404% | 147595% |  | 1% |
| Class A2 | -11.81% |  | -23.07% |  | -27.13% |  | 381% | 139067% |  | 1% |
| Class B | -10.84% |  | -26.39% |  | -25.86% |  | 399% | 144464% |  | 1% |
| Class C | -11.69% |  | -25.21% |  | -26.27% |  | 335% | 141091% |  | 1% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -11.25% |  | -23.82% |  | -25.22% |  | 378% | 143080% |  | 1% |
| Class D | -13.13% |  | -26.64% |  | -28.47% |  | 308% | 130374% |  | 1% |
| Class F | -5.96% |  | -16.20% |  | -15.75% |  | 565% | 61351% |  | 0% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | EncT | DecT CPU |  | bit DIFF |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  | 9% |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  | 9% |
| Class B | -9.19% |  | -19.25% |  | -21.56% |  | 367% | #NUM! |  | 2% |
| Class C | -10.59% |  | -21.57% |  | -21.11% |  | 319% | 130197% |  | 3% |
| Class E | -9.91% |  | -20.67% |  | -22.65% |  | 817% | 112896% |  | 2% |
| Overall | -9.84% |  | -20.28% |  | -21.36% |  | 427% | #NUM! |  | 2% |
| Class D | -11.83% |  | -24.19% |  | -24.88% |  | 295% | 117431% |  | 3% |
| Class F | -5.62% |  | -14.54% |  | -12.01% |  | 559% | 61682% |  | 2% |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | EncT | DecT CPU |  | bit DIFF |
| Class A1 | -6.87% |  | -19.34% |  | -20.16% |  | 329% | 105601% |  | 1% |
| Class A2 | -6.88% |  | -22.02% |  | -22.40% |  | 238% | 85747% |  | 1% |
| Class B | -7.02% |  | -22.24% |  | -22.52% |  | 214% | 81448% |  | 1% |
| Class C | -8.04% |  | -18.17% |  | -21.10% |  | 168% | 55818% |  | 0% |
| Class E | -10.53% |  | -22.55% |  | -24.29% |  | 234% | 90986% |  | 0% |
| Overall | -7.78% |  | -20.87% |  | -22.09% |  | 225% | 80343% |  | 1% |
| Class D | -7.82% |  | -17.47% |  | -20.59% |  | 159% | 52652% |  | 0% |
| Class F | -5.33% |  | -17.09% |  | -15.61% |  | 151% | 65679% |  | 0% |

# References

- EE1-1.7: Combined Test of EE1-1.6 and EE1-1.3, JVET-Z0113, Y. Li, K. Zhang, L. Zhang (Bytedance), H. Wang, M. Coban, A. M. Kotra, M. Karczewicz (Qualcomm), F. Galpin (InterDigital), K. Andersson, J. Ström, D. Liu, R. Sjöberg (Ericsson)

- EE1-1.2: Neural network based in-loop filter with a single model, JVET-Z0091, L. Wang, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital)

- EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer, JVET-V0115, H. Wang, J. Chen, A. M. Kotra, K. Reuzé, M. Karczewicz (Qualcomm)

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).