| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0142-v1 |
| --- | --- |

| Title: | AHG12/Non-EE2: Picture-Level Geometry Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Jia1, Kai Zhang1, Yang Wang2, Tianliang Fu2, Yue Li1, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | wei.jia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, a method of picture-level geometry transform is proposed. Three kinds of geometry transform: horizontal flip, vertical flip and 180°-rotation can be applied at picture level. It is signaled in the slice header to indicate whether a geometry transform is applied, and which transform is applied. On top of ECM-5.0, simulation results of the proposed method are reported as below:

AI: -0.19%,	-0.08%,	-0.10%.

# Introduction

In the current design of video/image coding scheme, the orientation of an input picture is fixed. A method of adaptive picture flipping coding was proposed in [1], trying to code an image with different orientations.

# Proposed method

In this contribution, a method of picture-level geometry transform is proposed. Three kinds of geometry transform: horizontal flip, vertical flip and 180°-rotation can be applied at picture level. It is signaled in the slice header to indicate whether a geometry transform is applied, and which transform is applied.

The encoder tries to encode the picture with each geometry transform as well as non-transform and selects the one with the minimum RD cost. To accelerate the encoder, a bundle of coding tools and partitioning choices are skipped in tentative encoding and only the picture with the selected transform is encoded with all functions.

The decoder will restore the reconstruction picture by transform it inversely according to the parsed type of geometry transform.

# Simulation results

The proposed method is implemented on top of ECM-5.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results of the proposed method compared to ECM-5.0 are shown in Table 1.

Table 1: Coding performance

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.04% | 0.27% | 103% | 99% |  |  |  |  |  |
| Class A2 | -0.46% | -0.32% | -0.17% | 102% | 100% |  |  |  |  |  |
| Class B | -0.29% | -0.04% | -0.41% | 101% | 99% | -0.19% | -0.44% | -0.56% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.03% | 0.14% | 0.02% | 101% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Overall | -0.19% | -0.07% | -0.10% | 101% | 100% |  |  |  |  |  |
| Class D | -0.08% | -0.36% | 0.02% | 104% | 102% | -0.05% | -0.14% | 0.08% | 102% | 99% |
| Class F | -0.05% | -0.08% | 0.01% | 104% | 100% | -0.32% | -0.53% | -0.26% | 103% | 99% |

# Conclusions

In this contribution, a method of picture-level geometry transform is proposed based on ECM-5. It is recommended to study the proposed method in EE2.

# Reference

- T. Murakami, I. Karube, H. Ito and M. Takahashi, "Adaptive picture flipping coding for enhancing H.264/AVC", Proc. 26th Picture Coding Symp. (No. 1027), Nov. 2007.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).