| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0146 |
| --- | --- |

| Title: | AHG12/Non-EE2: Fixes on ECM for 360-degree video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents two fixes to solve the crash issue of ECM for 360-degree video coding. The horizontal wrap around motion compensation, a 360-specific coding tool, is not fully considered by new coding tools in ECM. ECM-5.0 crashes when the horizontal wrap around motion compensation is enabled. Two fixes are proposed to solve the issue. First, clipMv() is replaced by wrapClipMv() in LIC, ARMC, MVD sign prediction, and multi-pass DMVR when the horizontal wrap around motion compensation is enabled. Second, out-of-boundary checking is skipped when the horizontal wrap around motion compensation is enabled. Compared to padded equi-rectangular projection (PERP) coding using VTM-17.0, simulation results of PERP coding using ECM-5.0 with the proposed fixes are reported as below:

End-to-end WS-PSNR: -13.21%, -24.59%, -25.88%;

End-to-end S-PSNR-NN: -13.20%, -24.62%, -25.92%.

Introduction

New coding tools are adopted in ECM to improve the coding performance beyond VVC during past JVET meetings. These coding tools are well evaluated under common test conditions of EE2, but some of them may not be evaluated for 360-degree video coding, resulting in the crash issue of ECM for PERP format.

Proposed method

To solve the crash issue of ECM for 360-degree video coding, two fixes are proposed in this contribution. First, clipMv() is replaced by wrapClipMv() when the horizontal wrap around motion compensation is enabled, which is related to Local illumination compensation (LIC), Adaptive re-ordering of merge candidates (ARMC), Motion vector difference (MVD) sign prediction, and multi-pass Decoder motion vector refinement (DMVR). Second, out-of-boundary (OOB) checking is skipped when the horizontal wrap around motion compensation is enabled, since the motion vector can be adjusted inside the picture using the wrap-around offset.

In addition, to enable coding 360-degree video using ECM-5.0 and 360Lib-13.2, MR !2062 [1] of code clean for VTM is used.

Simulation results

The proposed fixes are evaluated using ECM-5.0 [2] and 360Lib-13.2 [3]. Simulations are performed following the 360-degree video coding common test conditions [4]. Compared to VTM-17.0, simulation results of PERP coding using ECM-5.0 with the proposed fixes are shown in Table 1.

Table 1: PERP: ECM-5.0 fix over VTM-17.0

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.52% | -23.28% | -23.84% | -11.51% | -23.32% | -23.87% |
| Class S2 | -15.74% | -26.56% | -28.94% | -15.74% | -26.57% | -29.00% |
| Overall | -13.21% | -24.59% | -25.88% | -13.20% | -24.62% | -25.92% |

Simulation results of generalized cubemap (GCMP) coding using ECM-5.0 with the proposed fixes are shown in Table 2.

Table 2: GCMP: ECM-5.0 fix over VTM-17.0

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.24% | -22.95% | -23.02% | -11.23% | -22.98% | -23.04% |
| Class S2 | -14.50% | -26.24% | -28.42% | -14.50% | -26.27% | -28.49% |
| Overall | -12.54% | -24.27% | -25.18% | -12.54% | -24.30% | -25.22% |

Conclusions

In this contribution, two fixes are proposed to enable ECM for 360-degree video coding. It is recommended to adopt the proposed fixes in ECM.

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/merge_requests/2062.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- 360Lib-13.2, https://vcgit.hhi.fraunhofer.de/jvet/360lib/-/tree/360Lib-13.2.

- Y. He, J. Boyce, K. Choi, and J.-L. Lin, “JVET common test conditions and evaluation procedures for 360° video,” Document of Joint Video Experts Team, JVET-U2012, by teleconference, January 2021.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).