| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0168 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0095 (EE2-5.2: Using sample before deblocking filter for adaptive loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Wenbin Yin Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | yinwenbin.hit@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AA0095 [1], which proposes to use sample before deblocking filter in adaptive loop filter. A final ALF sample is derived by weighting the regular ALF and the filter applied to the samples before the deblocking filter. In EE2-5.2, a 3x3 or 5x5 diamond shape is applied to samples before deblocking filter. The simulation results match with those provided in JVET-AA0095.

Experimental results

The modifications on using sample before deblocking filter for adaptive loop filter were implemented on top of ECM-5.0. The simulations were conducted under the common test conditions of EE2 [2]. The crosscheck results of EE2-5.2-3x3 are shown in Table 1 and the crosscheck results of EE2-5.2-5x5 are shown in Table 2.

Table 1. Results of EE2-5.2 3x3

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.01% | 0.01% | 101% | 102% | -0.10% | -0.02% | 0.03% | 100% | 103% |  |  |  |  |  |
| Class A2 | -0.20% | 0.02% | 0.02% | 100% | 101% | -0.20% | 0.14% | 0.05% | 100% | 102% |  |  |  |  |  |
| Class B | -0.21% | 0.01% | 0.02% | 100% | 101% | -0.16% | 0.06% | -0.04% | 99% | 100% | -0.07% | 0.11% | 0.09% | 101% | 102% |
| Class C | -0.11% | 0.00% | 0.00% | 100% | 101% | -0.08% | 0.03% | 0.11% | 101% | 100% | -0.04% | 0.14% | 0.19% | 99% | 99% |
| Class E | -0.09% | 0.02% | 0.02% | 101% | 102% |  |  |  |  |  | -0.17% | -0.05% | 0.57% | 101% | 103% |
| Overall | -0.16% | 0.01% | 0.01% | 100% | 101% | -0.14% | 0.05% | 0.03% | 100% | 101% | -0.08% | 0.08% | 0.24% | 100% | 101% |
| Class D | -0.06% | 0.03% | 0.03% | 101% | 102% | -0.07% | -0.06% | 0.22% | 102% | 102% | -0.15% | 0.47% | 0.72% | 102% | 99% |
| Class F | -0.18% | 0.04% | 0.04% | 101% | 101% | -0.15% | 0.00% | 0.12% | 102% | 102% | -0.43% | -0.09% | 0.22% | 102% | 101% |

Table 2. Results of EE2-5.2 5x5

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.02% | 0.02% | 106% | 105% | -0.11% | 0.08% | -0.02% | 102% | 103% |  |  |  |  |  |
| Class A2 | -0.25% | 0.03% | 0.03% | 103% | 104% | -0.25% | 0.21% | 0.05% | 102% | 103% |  |  |  |  |  |
| Class B | -0.24% | 0.03% | 0.03% | 103% | 102% | -0.21% | 0.10% | 0.09% | 103% | 102% | -0.10% | 0.08% | 0.01% | 103% | 103% |
| Class C | -0.14% | 0.01% | 0.01% | 102% | 101% | -0.10% | 0.07% | 0.03% | 102% | 101% | -0.08% | -0.17% | 0.18% | 103% | 103% |
| Class E | -0.10% | 0.04% | 0.04% | 103% | 104% |  |  |  |  |  | 0.05% | -0.30% | 0.15% | 107% | 102% |
| Overall | -0.19% | 0.02% | 0.03% | 103% | 103% | -0.17% | 0.11% | 0.04% | 102% | 102% | -0.05% | -0.10% | 0.10% | 104% | 102% |
| Class D | -0.08% | 0.06% | 0.06% | 103% | 102% | -0.03% | -0.14% | 0.14% | 104% | 101% | -0.19% | 0.35% | 0.60% | 105% | 102% |
| Class F | -0.23% | 0.07% | 0.07% | 101% | 103% | -0.16% | 0.06% | 0.04% | 105% | 102% | -0.36% | -0.17% | 0.52% | 105% | 103% |

References

N. Hu, V. Seregin, M. Karczewicz, W. Yin, K. Zhang amd L. Zhang, “EE2-5: Adaptive filter shape switch and using sample before deblocking filter for adaptive loop filter,” Document of Joint Video Experts Team, JVET-AA0095, by teleconference, July 2022.

M. Karczewicz, and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.