| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0192-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AA0192-v1 (EE2-2.8, 2.9, 2.10: Longer luma and chroma filters for RPR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Cagan Selim Coban, Hussein Gulestani | Email: | ccoban@qti.qualcomm.com hgol@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for JVET-AA0042-v1 [1], which is a modification for luma and chroma filters for RPR. The cross-check was done for EE2-2.10 [2], which covers Test 2.8 and Test 2.9. It is reported that BD-rate numbers that are available in this crosscheck match those by the proponent.

# Experimental Results

The crosscheck has been performed in Low Delay B configuration as specified in [5]. The anchor is ECM-5.0. The corresponding simulations were conducted on Intel Xeon cluster with Linux OS, with a GCC 6.2.0 compiler. Both BD-rate PSNR1 and BD-rate PSNR2 results are partially computed.

## Results for 2.10a (Test 2.8 + Test 2.9a)

### Results for 2.10a under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.72% | -1.07% | -0.54% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.40% | -1.17% | -0.71% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

### Results for 2.10a under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.44% | -1.28% | -1.44% | 100% | 100% |
| Class E | -0.92% | -0.19% | -1.20% | 99% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.73% | -0.03% | -2.01% | 97% | 98% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.25% | -1.08% | -1.30% | 77% | 85% |
| Class E | -0.27% | -1.53% | -1.63% | 84% | 88% |
| Overall |  |  |  |  |  |
| Class D | -0.39% | -1.16% | -1.76% | 84% | 84% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

## Results for 2.10b (Test 2.8 + Test 2.9b)

### Results for 2.10b under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E | -1.56% | -2.17% | -1.32% | 99% | 101% |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E | -0.88% | -2.78% | -1.74% | 99% | 101% |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

### Results for 2.10b under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.44% | -1.28% | -1.44% | 100% | 100% |
| Class E | -0.92% | -0.19% | -1.20% | 99% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.73% | -0.03% | -2.01% | 97% | 98% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.18% | -1.43% | -1.27% | 100% | 100% |
| Class E | -0.19% | -1.44% | -1.80% | 99% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.34% | -0.68% | -2.47% | 97% | 98% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# References

- K. Andersson, R. Yu. EE2-2.8, 2.9, 2.10: Longer luma and chrome filters for RPR. JVET-AA0042. July 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang , “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Z2024, April 2022.

- J. Luo, V. Seregin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).