| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AAxxxx_v1 |
| --- | --- |

| Title: | DRAFT BoG on Neural Network Video Coding (NNVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | - Segall, E. Alshina | Tel: Email: | asegall@amazon.com elena.alshina@huawei.com |
| Source: | BoG |  |  |

_____________________________

This is a draft report of activities from the BoG on Neural Network Video Coding (NNVC).  The BoG held the following meetings during the 27th JVET meeting:

- July 19 – 05:00-7:00 UTC

- July 19 – 15:20-X:00 UTC

- …

Recommendations from the BoG are summarized as:

    - XXX

    - XXX

# Mandates

The BoG was established with the following mandates:

- Summarize the takeaways from the training crosschecks

- Develop and/or refine recommendations for executing and reporting

- Determine if common software base and/or methods can be defined

- Develop and/or refine recommendations for executing and reporting

# Training cross-checks in EE1

JVET-AA0174 [EE1] Crosscheck of training stage for EE1-1.5 and EE1-1.6 tests [J. Sauer, B. Wang, E. Alshina (Huawei)] [late]

Notes from plenary:

This document reports difficulties encountered during the attempted crosschecks of EE1-1.5 and EE1-1.6 and suggests to align the training methods for both experiments. Training data preparation is the most time consuming part, it takes thousands of hours. Without unification at least naming convention, number of coded frames, QPs, training cross-check efforts are double.

Suggestions for faster crosschecks

It is expected that a successful crosscheck will produce a model which is close in performance to the one of the proponent. If we can assume that this implies the performance of the generated model for a given epoch during the training process should also be similar for different training runs we can speed-up the crosscheck procedure as follows:

- The proponent runs his training. Proponent should keep checkpoints for each epoch and the log files for training and validation error

- The crosschecker requests checkpoints for certain epochs, decided (possibly randomly) by the crosschecker
Question/Note: First few epochs might have to be excluded. After how many epochs can similar model performance be expected?

- Proponent provides the requested checkpoints

- The crosschecker continues/resumes training from the provided checkpoints for some small number of epochs. Note that if resources are available, training resuming from those checkpoints can be done in parallel. It is expected that training and validation loss should be close to the one the proponent observed.

Performance of final model should still be checked in VTM/ECM

JVET-AA0178 Crosscheck of JVET-AA0088 (EE1-1.5: Neural network based in-loop filter with a single model) [H. Wang (Qualcomm)] [late]

| Trained and Tested by cross-checker | Original proposal |
| --- | --- |
| JVET-AA0178 JVET-AA0178-EE1.5.1.xlsm (“pseudo” training) | JVET-Z0091 JVET-Z0091-EE1-1.2.1.xlsm |
| Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.36%; -16.04%; -20.32%. Class A2; -9.19%; -18.44%; -15.13%. Class B; -8.31%; -20.46%; -19.87%. Class C; -8.11%; -22.17%; -21.90%. Class E. Overall; -8.64%; -19.63%; -19.55%. Class D; -9.13%; -21.22%; -21.23%. Class F; -3.65%; -13.05%; -12.41%. Class H; -7.95%; -15.40%; -18.51%. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1. Class A2. Class B; -7.41%; -16.26%; -17.49%. Class C; -7.79%; -18.74%; -18.89%. Class E; -7.85%; -13.63%; -13.82%. Overall; -7.65%; -17.36%; -18.11%. Class D; -8.93%; -17.25%; -19.50%. Class F; -4.04%; -12.15%; -11.47%. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.17%; -11.96%; -15.75%. Class A2; -5.66%; -14.20%; -12.54%. Class B; -5.79%; -14.47%; -15.50%. Class C; -6.35%; -15.23%; -17.09%. Class E; -8.56%; -15.03%; -15.39%. Overall; -6.42%; -14.27%; -15.38%. Class D; -6.41%; -13.60%; -16.15%. Class F; -4.03%; -11.63%; -11.90%. Class H; -6.50%; -11.39%; -12.78%. | Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.34%; -15.99%; -18.74%. Class A2; -9.30%; -17.82%; -14.84%. Class B; -8.29%; -18.99%; -19.70%. Class C; -8.20%; -20.74%; -21.33%. Class E. Overall; -8.68%; -18.62%; -18.97%. Class D; -9.02%; -19.10%; -21.32%. Class F; -3.77%; -12.08%; -11.64%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -8.70%; -12.09%; -18.04%. Class A2; -7.75%; -15.42%; -14.21%. Class B; -7.27%; -14.59%; -16.42%. Class C; -7.83%; -17.03%; -17.91%. Class E; -7.71%; -13.81%; -15.08%. Overall; -7.57%; -15.68%; -17.08%. Class D; -8.85%; -17.16%; -18.37%. Class F; -4.16%; -11.18%; -10.06%. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.18%; -13.48%; -16.09%. Class A2; -5.80%; -14.77%; -12.04%. Class B; -5.86%; -14.79%; -16.04%. Class C; -6.44%; -15.94%; -17.95%. Class E; -8.69%; -15.12%; -17.00%. Overall; -6.50%; -14.88%; -15.96%. Class D; -6.47%; -15.00%; -17.77%. Class F; -4.06%; -11.21%; -11.44%. Class H; -6.18%; -13.48%; -16.09%. |

Notes from BoG:

The cross-checker reported that a “pseudo-training” process was employed to perform the cross-check.  This means that the second stage training used bit-streams provided by the proponent.  Specifically, it was reported the model provided by Tencent (JVET-X0054) was used.

The cross-checker reported the following challenges in performing the cross-check:

- The amount of time needed for the two stages of training

- The training took 18 days

- The data preparation took an additional 21 days

A participant commented that for pseudo-training the training data for the second stage can be generated in parallel with the first stage training.

A participant commented that the data preparation depends on the architecture of the compute environment.

A participant commented that the size of the training data was on the order of terabytes.

A participant commented that the training time depends on the disk, GPU speed and GPU utilization.

A participant commented that some of the data preparation time could be reduced by optimizing the data transfer paths within a compute platform/cluster.

A participant reported that the data preparation had taken 5 days for each stage (totaling 10 days) in their experience.  A second participant reported that the data preparation took approximately 7 days in their experience.

JVET-AA0181 Cross-check of JVET-AA0111 (EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation) [K. Lin, C. Jia, S. Wang (PKU)] late]

Notes from plenary:

Training crosscheck gives deviations 0.4% for RA, 0.1% for AI (luma), slightly higher for chroma. Perfect match for inference in integer. Training cross-check was done from scratch. No results for LB.

| Trained and Tested by cross-checker | Original proposal |
| --- | --- |
| JVET-AA0181 JVET-AA0181-1.6.1.xlsm(‘real” training) | JVET-Z0113 JVET-Z0113_8ResidualBlocks.xlsm |
| Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.48%; -17.90%; -20.07%. Class A2; -10.57%; -21.57%; -22.09%. Class B; -9.37%; -24.81%; -23.95%. Class C; -9.94%; -23.11%; -24.11%. Class E. Overall; -9.78%; -22.33%; -22.84%. Class D; -11.49%; -24.87%; -26.04%. Class F; -4.52%; -13.41%; -13.45%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1. Class A2. Class B. Class C. Class E. Overall. Class D. Class F. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.56%; -18.49%; -18.95%. Class A2; -6.62%; -20.73%; -18.91%. Class B; -6.57%; -21.25%; -21.79%. Class C; -7.52%; -19.91%; -22.76%. Class E; -9.90%; -21.15%; -21.37%. Overall; -7.34%; -20.39%; -20.98%. Class D; -7.45%; -20.13%; -22.11%. Class F; -3.99%; -13.30%; -12.92%. Class H. | Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.82%; -18.12%; -20.88%. Class A2; -10.96%; -21.33%; -18.33%. Class B; -9.77%; -24.24%; -23.49%. Class C; -10.39%; -23.22%; -24.16%. Class E. Overall; -10.18%; -22.16%; -22.11%. Class D; -11.84%; -25.05%; -25.52%. Class F; -5.04%; -12.95%; -12.56%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1. Class A2. Class B; -8.48%; -16.03%; -16.43%. Class C; -9.72%; -15.09%; -15.67%. Class E; -9.17%; -16.95%; -17.46%. Overall; -9.07%; -15.95%; -16.43%. Class D; -11.11%; -18.86%; -18.46%. Class F; -5.52%; -8.86%; -6.29%. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.43%; -17.35%; -18.50%. Class A2; -6.47%; -19.73%; -17.36%. Class B; -6.48%; -21.17%; -21.17%. Class C; -7.43%; -18.84%; -21.35%. Class E; -9.81%; -20.58%; -20.50%. Overall; -7.24%; -19.68%; -20.02%. Class D; -7.38%; -18.97%; -21.05%. Class F; -4.03%; -11.98%; -11.28%. Class H. |

Notes from BoG:

The cross-checker reported that all of the models were trained scratch in JVET-AA0181

The cross-checker reported the following challenges:

- The amount of time to perform the cross-check

- A need to update the training scripts to run in the cross-checker’s environment

A participant commented that a cross-check may generate a better result than the proponent.

A participant commented that models from a common software could be used as a starting point for future development and that using this starting could speed up the training process.

A participant commented that using the common software as a starting point should not be mandated.

A participant commented that if there is a reference software, the following could apply:

- Proponent trains the reference model scratch and compares to the reference software

- Proponent trains proposal from scratch and compares to the reference software

A participant commented that there could be three options for training a proposal using common software:

- Proponent trains a model and necessary training data completely from scratch

- Proponent downloads existing models and uses the existing models to generate training data.  The proposed model is then trained from scratch using this training data.

- Proponent downloads existing models and uses the parameters of the model for initialization of the proposed model (tentatively “The weight initialization approach”).  The proponent also uses the existing models to generate training data.

|  | Data generation | Model training |
| --- | --- | --- |
| Option 1 (above) | From Scratch | From Scratch |
| Option 2 (above) | Using existing model | From Scratch |
| Option 3 (above) | Using existing model | Weight/parameter initialization from existing model |

A participant commented that it could be useful for a proponent and cross-checker to compare their results at different intermediate points in the training process.  One example would be to provide the BD-rate using an intermediate model.

A participant commented that it might be desirable to have a threshold to determine if a cross-check is successful.

JVET-AA0217 Crosscheck of JVET-AA0088(EE1-1.5: neural network based in-loop filter with a single model) [Z. Xie (OPPO)] [late]

Notes from presentation in plenary:

Re-training crosscheck gave similar gains (0.04% difference for luma, however but up to 1% for chroma) More investigation necessary why in the re-training crosschecks (not only for this proposal) chroma deviations are larger. One reason might be the chroma BD computation relative to total rate, but that may not explain such large deviation. It was commented by the proponents that the current training procedure gives more weight to luma in the loss function (10:1:1), which could explain such behaviour.

It is further reported that the results reported here (as well as in JVET-AA0178) are based on pseudo training, initializing the second stage (which uses data from decoded outputs of the NN codec) from the results of the proponents. The full training from scratch (including the first stage which starts from VVC decoded data, and then codes with the NN codec) is not finished yet. Revisit when results are available, and then also analyse how close pseudo training (which is much faster) comes to the real training. Also times should be reported.

Initial results were shown in session 12 which indicated good match both between pseudo training and the real training crosscheck for RA and AI, but some larger deviation for LB (where the results were still incomplete). More analysis to be done after completion.

| Trained and Tested by cross-checker | Original proposal |
| --- | --- |
| JVET-AA0217 JVET-AA0217-EE1-1.5.1.xlsm (“pseudo” training) | JVET-Z0091 JVET-Z0091-EE1-1.2.1.xlsm |
| Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.40%; -15.37%; -19.68%. Class A2; -9.25%; -18.07%; -15.09%. Class B; -8.27%; -20.16%; -20.25%. Class C; -8.30%; -22.51%; -21.81%. Class E. Overall; -8.70%; -19.41%; -19.52%. Class D; -9.15%; -20.75%; -21.55%. Class F; -3.85%; -12.43%; -11.79%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1. Class A2. Class B; -7.39%; -14.40%; -15.87%. Class C; -8.01%; -19.67%; -17.14%. Class E; -7.89%; -14.15%; -12.89%. Overall; -7.72%; -16.74%; -16.44%. Class D; -9.02%; -18.86%; -18.43%. Class F; -4.33%; -11.29%; -10.39%. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.22%; -13.66%; -15.84%. Class A2; -5.76%; -14.61%; -12.37%. Class B; -5.81%; -15.04%; -16.20%. Class C; -6.40%; -16.32%; -17.79%. Class E; -8.64%; -15.33%; -15.66%. Overall; -6.47%; -15.07%; -15.77%. Class D; -6.42%; -14.27%; -17.14%. Class F; -4.18%; -11.06%; -11.62%. Class H. | Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.34%; -15.99%; -18.74%. Class A2; -9.30%; -17.82%; -14.84%. Class B; -8.29%; -18.99%; -19.70%. Class C; -8.20%; -20.74%; -21.33%. Class E. Overall; -8.68%; -18.62%; -18.97%. Class D; -9.02%; -19.10%; -21.32%. Class F; -3.77%; -12.08%; -11.64%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -8.70%; -12.09%; -18.04%. Class A2; -7.75%; -15.42%; -14.21%. Class B; -7.27%; -14.59%; -16.42%. Class C; -7.83%; -17.03%; -17.91%. Class E; -7.71%; -13.81%; -15.08%. Overall; -7.57%; -15.68%; -17.08%. Class D; -8.85%; -17.16%; -18.37%. Class F; -4.16%; -11.18%; -10.06%. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.18%; -13.48%; -16.09%. Class A2; -5.80%; -14.77%; -12.04%. Class B; -5.86%; -14.79%; -16.04%. Class C; -6.44%; -15.94%; -17.95%. Class E; -8.69%; -15.12%; -17.00%. Overall; -6.50%; -14.88%; -15.96%. Class D; -6.47%; -15.00%; -17.77%. Class F; -4.06%; -11.21%; -11.44%. Class H; -6.18%; -13.48%; -16.09%. |
| Trained and Tested by cross-checker (“real” training) | Original proposal |
| JVET-AA0217 JVET-AA0217-EE1-1.5.1_additional.xlsm | JVET-Z0091 JVET-Z0091-EE1-1.2.1.xlsm |
| Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1. Class A2. Class B; -8.14%; -18.60%; -20.53%. Class C; -8.47%; -19.97%; -21.25%. Class E. Overall. Class D; -9.15%; -20.75%; -21.55%. Class F; -3.85%; -12.43%; -11.79%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1. Class A2. Class B. Class C. Class E. Overall. Class D; -8.48%; -13.36%; -17.12%. Class F. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; #VALUE!; #VALUE!; #VALUE!. Class A2; #VALUE!; #VALUE!; #VALUE!. Class B; -5.99%; -15.66%; -17.03%. Class C; -6.22%; -14.15%; -17.06%. Class E; -8.53%; -14.74%; -16.37%. Overall; #VALUE!; #VALUE!; #VALUE!. Class D; -6.26%; -12.34%; -16.52%. Class F. Class H. | Random access Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -9.34%; -15.99%; -18.74%. Class A2; -9.30%; -17.82%; -14.84%. Class B; -8.29%; -18.99%; -19.70%. Class C; -8.20%; -20.74%; -21.33%. Class E. Overall; -8.68%; -18.62%; -18.97%. Class D; -9.02%; -19.10%; -21.32%. Class F; -3.77%; -12.08%; -11.64%. Class H. Low delay B Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -8.70%; -12.09%; -18.04%. Class A2; -7.75%; -15.42%; -14.21%. Class B; -7.27%; -14.59%; -16.42%. Class C; -7.83%; -17.03%; -17.91%. Class E; -7.71%; -13.81%; -15.08%. Overall; -7.57%; -15.68%; -17.08%. Class D; -8.85%; -17.16%; -18.37%. Class F; -4.16%; -11.18%; -10.06%. Class H. All Intra Main10. BD-rate Over VTM-11.0. Y-PSNR; U-PSNR; V-PSNR. Class A1; -6.18%; -13.48%; -16.09%. Class A2; -5.80%; -14.77%; -12.04%. Class B; -5.86%; -14.79%; -16.04%. Class C; -6.44%; -15.94%; -17.95%. Class E; -8.69%; -15.12%; -17.00%. Overall; -6.50%; -14.88%; -15.96%. Class D; -6.47%; -15.00%; -17.77%. Class F; -4.06%; -11.21%; -11.44%. Class H; -6.18%; -13.48%; -16.09%. |

Notes from BoG:

The cross-checker reported the following challenges:

- The cross-check took a lot of time training

- The cross-check took a lot of time for data generation.  However, the data transfer was efficient.

- Total time was about one month

The cross-checker reported that the real training was trained for 27 days, which included the data generation for the second stage.  It was reported that this resulted in less training time than the pseudo-training, which was trained for 20 days.  This may have resulted in a larger difference to the anchor.

## Discussion

### Do we consider the proposals sufficiently cross-checked?

### A participant that they felt the proposals were sufficiently cross-checked, as the results are close to the anchor.  The participant also commented that the cross-check results suggested that the models were not overfitted, which is desirable.

A participant expressed concern that some results were missing in JVET-AA0181 and JVET-AA0217.  The participant also expressed concern that the available results are using a pseudo training approach.

A participant expressed that low-delay results are less important in the cross-checking procedure.  The participant also expressed that the results were close to the anchor and therefore sufficiently cross-checked.

A participant expressed that having a close match on random access may be more difficult than all intra.

Another participant expressed that they viewed the results as sufficiently cross-checked.

A participant expressed that the models were trained for all intra and random access – and are not trained on low-delay.

A participant suggested that the group can accept a cross-check that shows lower gains than the proponent, if the cross-check still shows gains.

The consensus of the group is that the proposals are sufficient cross-checked.

Recommendation: Consider the proposals sufficiently cross-checked.

## Common Software Questions

### What elements should be in a common software base?

The group discussed what elements should be in a common software base.  As a result of the discussion, it was suggested that the common software base should contain:

- Encoder software

  - Support for inference

- Decoder software

  - Data dumper for VTM11

  - Data dumper for VTM11+reference models

  - Support for inference

- Training scripts

  - Support for Options 1, 2 and 3 (in the table in the discussion of JVET-AA0181)

  - Data loader that works in all compute environments

A participant asked if we should have a common naming convention for the training data

A second participant asked if we should have a common format for storing parameters that are common across proposals, such as the QP, prediction samples, reconstructed samples, etc.

A participant commented that the common format and naming convention could be determined as part of the implementation and specification of the data dumper.

A participant commented that the group should be open to proposals that do not use quantization.

Continue in second BoG meeting.

### How should we go forward in creating the common software base?

TBD

## Recommendation Refinement

### How should we update the recommendations for executing and reporting cross-check procedures to address challenges that were reported in the BoG?

As a summary, the following challenges had been reported:

JVET-AA0174

- The data preparation is the most time consuming part and took thousands of hours

- The cross-checking effort would be improved by defining the naming convention for common training data, such as the number of coded frames and QPs.

JVET-AA0178

- The amount of time needed for the two stages of training

- The training took 18 days

- The data preparation took an additional 21 days

JVET-AA0181

- The amount of time to perform the cross-check

- A need to update the training scripts to run in the cross-checker’s environment

JVET-AA0217

- The cross-check took a lot of time training

- The cross-check took a lot of time for data generation.  However, the data transfer was efficient.

- Total time was about one month

### TBD

### Should “pseudo” training and/or “real” training be used going forward?

TBD

## Takeaways

### What are the takeaways from the training crosschecks?

Note: This has not been reviewed in the BoG.

Draft: Key takeaways from the training crosschecks are:

- The cross-checks took a lot of time for training

- The cross-checks took a lot of time for data generation.  In some cases, this was further increased due to data transfer.

- The training scripts provided by proponents did not work in all compute environments

- The cross-checking effort would be improved by defining the naming convention for common training data, such as the number of coded frames and QPs.

TBD

## Future Planning

### What action should be taken with the network models that were conditionally adopted at the last meeting and cross-checked at this meeting with respect to the common software base?

TBD

### What should be the anchor of the EE if the common software is not available at this meeting?

TBD

Mandate reminders (to be removed)

- Summarize the takeaways from the training crosschecks

- Determine if common software base and/or methods can be defined

- Develop and/or refine recommendations for executing and reporting