| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com fabricel@xiaomi.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 27th and 28th JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-6.0 software version and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

- Coordinate with AG 5, AHG4, and AHG12 on preparing a subjective test to be conducted during the upcoming JVET meeting.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following fixes were integrated on top of ECM-5.0 and it is released as ECM-5.1:

Fix for valid number of affine candidates (MR 170)

Optimization for GDR software to lower runtime (MR 169)

Fix for reference picture reordering when DMVD if disabled (MR 168)

Fix for IBC macros with ARMC usage (MR 171)

Fix for GDR with IBC HMVP (MR 172)

Fix for pairwise candidates in GPM merge list (MR 173)

Fix for lintf under (MR 176)

Fix decoder crash when AML=0 and IBC=1 (MR 174)

Fix for storing and loading CABAC windows for temporal CABAC (MR 175)

Port the memory from VTM for EncDbOpt (MR 161)

Port y4m support from VTM (MR 181)

Fix for macro and config parameter difference for GPM split reordering (MR 180)

Fix initialization of adaptive QP map for BIM (MR 179)

Fix wrong L1 MVP in SMVD for MVD sign prediction (MR 178)

Encoder fix for IBC-TM when DMVD is disabled (MR 185)

Fix macro interaction for IBC BVD and IBC TM (MR 186)

Fix GPM intra encoder/decoder mismatch when sign prediction is disabled (MR 187)

Fix for MVP index bit count for SMVD when DMVD is disabled (MR 189)

Fix array out of bound for MVP cost when DMVD is disabled (MR 190)

The following adopted aspects were integrated into ECM-6.0:

JVET-AA0042: Longer RPR filters for luma and chroma (Test 2.10b) (MR 195)

JVET-AA0097: Fix for block-level OOB check (MR 191)

JVET-AA0146: Fixes on ECM for 360-degree video coding (MR 193)

JVET-AA0058_Test2.7a: GPM adaptive blending (MR 194)

JVET-AA0093: EE2-2.6: Combination tests of Test 2.3, Test 2.4 and Test 2.5 (MR 197)

JVET-AA0107: RMVF affine merge candidates (MR 200)

JVET-AA0129: inter-hash RDO with OBMC-off (MR 206)

JVET-AA0124: AmvpMerge mode enabling check when RPR is enabled and DMVD is disabled (MR 162)

JVET-AA0057: CCCM (MR 192)

JVET-AA0128 test b: Increased number of CABAC contexts of affine merge index (MR 207)

JVET-AA0095: Longer luma filter shape and using samples before deblocking filter for ALF (MR 199)

JVET-AA0098: MTTdepth by Tid for class A random access with QP 22 (MR 201)

JVET-AA0126: GLM (MR 209)

JVET-AA0096: Motion compensated picture boundary padding (MR 202)

JVET-AA0133: Inter MTS optimization (MR 211)

JVET-AA0132: ECM software configuration parameters for template matching tools (MR 198)

JVET-AA0106: Adjust IBC reference area to 2x128 rows above the current CTU (MR 215)

JVET-AA0062: BVD for IBC with IBC flipping (MR 216)

CTC: Enable palette for SCC (MR 222)

Included fixes:

Initialize GOP list when GDR is enabled (MR 205)

Fix ref pic reordering overhead (MR 204)

Fix deblocking RDO chroma distortion calculation for HDR-PQ coding (MR 182)

Fix for chroma 400 format (MR 213)

Prevent empty merge candidate list generation for AmvpMerge (MR 184)

Fix for bilateral filter encoder (MR 210)

Set default values for BCW and HPEL interpolation filter (MR 225)

Fix crash and mismatch IBC with disabled AML (MR 224)

Fix creating coding structure for GDR (MR 227)

Fix conditional jumps on uninitialized variables (MR 233)

Fix uninitialized variables for SAO (MR 236)

Reduce compile time for VC (MR 237)

Fix MD5 error triggers with disabled AML (MR 239)

The following adopted aspects were integrated into VTM-11ecm6 anchor:

JVET-AA0098: MTTdepth by Tid for class A random access with QP 22 (MR 208)

JVET-AA0133: Enable inter MTS (MR 214)

CTC: Enable palette for SCC (MR 223)

Fix: deblocking RDO chroma distortion calculation for HDR-PQ coding (MR 212)

ECM-5.1 was tagged on July 25, 2022.

ECM-6.0 was tagged on September 4, 2022.

VTM-11ecm6 was tagged on September 4, 2022.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-Y2017 are summarized.

The below tables show ECM-5.1 performance over ECM-5.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 104% | 104% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class TGM | 0.00% | 0.00% | 0.00% | 98% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.10% | -0.07% | 102% | 103% |
| Class A2 | -0.03% | 0.02% | 0.08% | 101% | 101% |
| Class B | 0.00% | -0.10% | -0.08% | 99% | 99% |
| Class C | 0.00% | 0.04% | 0.02% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.00% | -0.11% | -0.12% | 98% | 98% |
| Class F | -0.01% | -0.06% | 0.02% | 98% | 99% |
| Class TGM | 0.06% | 0.06% | 0.05% | 98% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.13% | -0.16% | 100% | 102% |
| Class C | 0.00% | -0.12% | -0.20% | 95% | 98% |
| Class E | -0.16% | 0.25% | 0.24% | 98% | 100% |
| Overall | -0.03% | -0.03% | -0.07% | 98% | 100% |
| Class D | 0.01% | 0.85% | -0.28% | 97% | 99% |
| Class F | -0.29% | 0.05% | -0.37% | 98% | 101% |
| Class TGM | -0.03% | -0.03% | 0.01% | 98% | 102% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.22% | -0.08% | 100% | 102% |
| Class C | -0.02% | 0.23% | -0.18% | 101% | 101% |
| Class E | -0.03% | -0.34% | -1.23% | 99% | 99% |
| Overall | -0.02% | -0.10% | -0.40% | 100% | 101% |
| Class D | 0.03% | -0.01% | -0.66% | 101% | 102% |
| Class F | 0.15% | -0.26% | -0.18% | 99% | 101% |
| Class TGM | 0.00% | 0.10% | 0.07% | 98% | 96% |

Next tables show ECM-6.0 performance over ECM-5.1 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.41% | -2.56% | -4.75% | 115% | 115% |
| Class A2 | -5.87% | -4.58% | -7.42% | 117% | 116% |
| Class B | -0.81% | -3.57% | -1.80% | 104% | 105% |
| Class C | -0.27% | -0.80% | -0.63% | 106% | 106% |
| Class E | -0.45% | -3.35% | -1.17% | 104% | 105% |
| Overall | -1.57% | -2.92% | -2.86% | 108% | 109% |
| Class D | -0.16% | -0.07% | -0.36% | 105% | 107% |
| Class F | -4.00% | -6.49% | -6.17% | 108% | 105% |
| Class TGM | -12.50% | -16.87% | -16.28% | 102% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.30% | -3.06% | -4.85% | 115% | 120% |
| Class A2 | -3.45% | -2.93% | -4.57% | 122% | 130% |
| Class B | -1.68% | -4.10% | -2.80% | 120% | 113% |
| Class C | -1.60% | -0.96% | -1.31% | 115% | 114% |
| Class E |  |  |  |  |  |
| Overall | -2.13% | -2.82% | -3.17% | 118% | 118% |
| Class D | -1.70% | -0.93% | -0.51% | 116% | 116% |
| Class F | -5.00% | -5.97% | -6.32% | 113% | 109% |
| Class TGM | -10.25% | -10.47% | -10.46% | 108% | 113% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.79% | -2.35% | -2.39% | 116% | 130% |
| Class C | -1.92% | -1.11% | -1.51% | 117% | 126% |
| Class E | -0.98% | -4.53% | -2.90% | 118% | 121% |
| Overall | -1.63% | -2.48% | -2.23% | 117% | 126% |
| Class D | -2.20% | -1.55% | -1.79% | 121% | 126% |
| Class F | -4.19% | -4.60% | -4.17% | 116% | 114% |
| Class TGM | -7.89% | -7.06% | -7.21% | 110% | 106% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-5.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.47% | -1.50% | -1.00% | 116% | 129% |
| Class C | -1.32% | -0.69% | -0.36% | 120% | 124% |
| Class E | -1.01% | -3.07% | -1.33% | 114% | 117% |
| Overall | -1.31% | -1.62% | -0.87% | 117% | 124% |
| Class D | -1.43% | -0.44% | -0.87% | 120% | 123% |
| Class F | -3.40% | -3.64% | -3.31% | 116% | 114% |
| Class TGM | -4.76% | -4.65% | -4.71% | 108% | 110% |

The below tables show ECM-6.0 performance comparing to VTM-11.0ecm6 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.15% | -16.36% | -23.24% | 490% | 338% |
| Class A2 | -11.93% | -19.18% | -22.12% | 468% | 311% |
| Class B | -6.68% | -18.88% | -17.18% | 413% | 285% |
| Class C | -7.36% | -11.49% | -11.91% | 414% | 277% |
| Class E | -8.32% | -16.40% | -15.40% | 388% | 300% |
| Overall | -8.22% | -16.45% | -17.55% | 430% | 298% |
| Class D | -6.07% | -9.36% | -9.16% | 413% | 298% |
| Class F | -17.83% | -24.91% | -24.43% | 336% | 299% |
| Class TGM | -30.28% | -35.62% | -34.91% | 303% | 304% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -18.76% | -21.33% | -28.13% | 526% | 652% |
| Class A2 | -20.59% | -26.86% | -29.77% | 544% | 884% |
| Class B | -16.88% | -26.97% | -24.74% | 449% | 678% |
| Class C | -18.75% | -21.90% | -21.83% | 427% | 692% |
| Class E |  |  |  |  |  |
| Overall | -18.50% | -24.47% | -25.65% | 475% | 713% |
| Class D | -19.78% | -23.23% | -23.42% | 426% | 747% |
| Class F | -21.21% | -27.21% | -27.56% | 384% | 442% |
| Class TGM | -29.30% | -33.62% | -34.08% | 417% | 356% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.49% | -29.59% | -28.03% | 390% | 595% |
| Class C | -16.89% | -24.94% | -25.01% | 396% | 580% |
| Class E | -14.04% | -22.57% | -22.67% | 390% | 390% |
| Overall | -15.59% | -26.29% | -25.68% | 392% | 531% |
| Class D | -19.04% | -26.06% | -26.25% | 392% | 669% |
| Class F | -19.92% | -27.91% | -26.96% | 384% | 391% |
| Class TGM | -28.62% | -34.50% | -34.38% | 401% | 324% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.02% | -29.61% | -27.54% | 371% | 580% |
| Class C | -16.58% | -24.65% | -23.84% | 343% | 571% |
| Class E | -14.04% | -21.98% | -22.82% | 363% | 392% |
| Overall | -15.30% | -26.05% | -25.13% | 359% | 523% |
| Class D | -19.39% | -26.08% | -26.11% | 333% | 621% |
| Class F | -19.18% | -27.86% | -26.86% | 383% | 391% |
| Class TGM | -25.55% | -32.36% | -32.30% | 423% | 327% |

The Excel files with the complete ECM results are attached to this report.

# ECM repository access

To access ECM software and EE2 repositories, a GitLab account shall be converted to internal. To convert an account to internal, the request should be send to VTM software coordinators as detailed in VTM Wiki https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/wikis/VVC-Software-Development-Workflow. Internal account provides read access, ability to submit merge requests, and issues.

Alternatively, ECM can be accessed using VCEG or MPEG shared accounts.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.