| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0011-v1 |
| --- | --- |

| Title: | JVET AHG report: Neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Franck Galpin Yue Li Hongtao Wang Liqiang Wang Zhao Wang Mathias Wien Ping Wu | Email: | elena.alshina@huawei.com shanl@tencent.com asegall@amazon.com Franck.Galpin@InterDigital.com yue.li@bytedance.com hongtaow@qti.qualcomm.com liqiangwang@tencent.com baixiu.wz@alibaba-inc.com wien@lfb.rwth-aachen.de pwuzte@gmail.com |
| Source: | AHG11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural network-based video coding between the 27th meeting (13 – 22 July 2022) held by teleconference and the 28th meeting (20 – 28 October 2022) held in Mainz, DE.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures.

- Refine the test conditions for NN-based video coding. Generate and distribute anchor encoding, and develop supporting software as needed.

- Study the impact of training (including the impact of loss functions) on the performance of candidate technologies, and identify suitable material for testing and training.

- Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology.

- Create a common software base according to the decisions in the 27th meeting. Prepare and release anchor data for all configurations of the software; prepare and release verification data for the included training scripts.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed.

- Finalize and discuss the EE on neural network-based video coding.

- Coordinate with other relevant groups, including SC29/AG5 on the evaluation and assessment of visual quality and AHG12 on the interaction with ECM coding tools. In the case of conducting subjective viewing, provide support to AHG4 for the timely availability of test materials and test subjects.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. Three emails were exchanged on the reflector related to the AHG mandates.

## Common Test Conditions

### Document

The AHG released revised common test conditions as decided at the 27th meeting.  The final version was uploaded as document JVET-AA2016 on August 6, 2022.

### Anchor Software

Software for the revised common conditions was made available on the Git repository used for the AHG activity.  A full version of the anchor software was tagged as VTM-11_nnvc-2.0 on August 4, 2022.  The software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc-2.0.  Additionally, a patch file capturing the changes to the CTC was provided on August 5, 2022 and available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

### Anchor Encoding

Anchors for the NN-based video coding activity were released on September 5, 2022.  The anchors were also made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master.

### Additional Comments

The major change in the common conditions was to enable the EncDbOpt for the RA, LDB and LDP configurations.  This was done to follow the group decision to enable EncDbOpt, and the NNVC CTC follows the same approach as what is done in the VTM and ECM CTCs.

Following the release of the CTC, the AhG further discussed if EncDbOpt should have been enabled for AI.  It is unclear from the notes at the last meeting if this was intended, and the AhG decided to not re-issue the CTC.  Having said that, the AhG would recommend that the group discuss if EncDbOpt should be enabled for the AI configuration during the 28th meeting.  This could include the NNVC and/or other CTCs.  Additionally, different configurations of the candidate software (discussed in Section 2.4) take different approaches to enabling the EncDbOpt parameter.  So guidance from the group would be helpful for those configurations as well.

## EE Coordination

The AHG finalized, conducted and discussed the EE on NN based video coding.  The final version of the EE description was uploaded to the document repository on August 6, 2022.

A summary report for the EE is available at this meeting as:

| JVET-AB0023 | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding | E. Alshina, F. Galpin, Y. Li, M. Santamaria, H. Wang, L. Wang, Z. Xie |
| --- | --- | --- |

## Study and Maintain SADL

The SADL library was updated during the AHG period.  Updates were provided on October 13, 2022.  The software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl.

Change logs include:

Commit cd3f79eb6b8d8c2d5b6fd0715c14db7fd5d9c7b7

Changes:

- add conv2d 5x5 (support for one case for auto encoder)

- increase maximum total size of tensor

- better output for info on model

- clean python code

## Create Common Software

The AhG created a common software base according to the decisions in the 27th meeting.  The software is tentatively called the Neural Compression Software (NCS).

A first release candidate for the NCS was announced on August 24, 2022 and available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NCS-rc1.0. It included the following features:

- Common API for data dumping (VTM side) and loading (python side)

- Common API for inference (VTM side)

- Common part for SPS etc.

- Two neural network-based filter sets:

  - set 0 (corresponding to AA0088)

  - set 1 (corresponding to AA0111)

- Training scripts for each set, including the stages:

  - Data dumping

  - Dataset generation

  - Training (from scratch or from already generated models)

  - Conversion to SADL format

An official candidate was announced on September 5, 2022, and available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NCS-1.0.

### Anchor Encoding

The coding performance of the common software as input to the meeting in document JVET-AB0176.   As reported, the performance of NCS-1.0 NN-based filter set #0 (int16 precision) performance compared to the NNVC-2.0 anchor is

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.15% | -13.89% | -18.71% | 153% | 73018% |
| Class A2 | -9.37% | -19.11% | -14.46% | 149% | 70078% |
| Class B | -8.33% | -18.68% | -20.42% | 156% | 70892% |
| Class C | -8.37% | -20.30% | -20.35% | 143% | 60876% |
| Class E |  |  |  |  |  |
| Overall | -8.71% | -18.24% | -18.87% | 151% | 68316% |
| Class D | -9.58% | -19.42% | -20.42% | 139% | 58850% |
| Class F | -3.72% | -11.90% | -10.92% | 197% | 32349% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.32% | -17.59% | -20.15% | 144% | 69084% |
| Class C | -7.86% | -20.18% | -20.40% | 131% | 55098% |
| Class E | -8.43% | -17.14% | -16.50% | 213% | 54715% |
| Overall | -7.78% | -18.74% | -20.26% | 154% | 60438% |
| Class D | -9.21% | -18.73% | -19.41% | 132% | 51121% |
| Class F | -4.17% | -10.03% | -8.79% | 185% | 32820% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.12% | -14.26% | -17.26% | 211% | 50641% |
| Class A2 | -5.73% | -15.81% | -12.98% | 159% | 41067% |
| Class B | -5.93% | -15.54% | -17.46% | 149% | 38489% |
| Class C | -6.40% | -16.42% | -18.16% | 133% | 27723% |
| Class E | -8.83% | -15.25% | -16.12% | 167% | 44744% |
| Overall | -6.52% | -15.52% | -16.61% | 159% | 38826% |
| Class D | -6.41% | -15.30% | -18.56% | 126% | 25806% |
| Class F | -4.02% | -11.81% | -11.71% | 128% | 32540% |

The performance of the NCS-1.0 NN-based filter set #0 (float precision) over the NNVC-2.0 anchor is:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.21% | -14.66% | -18.35% | 170% | 97724% |
| Class A2 | -9.37% | -19.29% | -14.32% | 163% | 93013% |
| Class B | -8.31% | -19.02% | -20.33% | 172% | 93877% |
| Class C | -8.34% | -20.27% | -20.09% | 159% | 80518% |
| Class E |  |  |  |  |  |
| Overall | -8.71% | -18.53% | -18.67% | 166% | 90671% |
| Class D | -9.48% | -19.52% | -20.41% | 162% | 77552% |
| Class F | -3.73% | -11.83% | -11.00% | 229% | 42242% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.31% | -18.18% | -20.28% | 161% | 91967% |
| Class C | -7.85% | -19.93% | -20.16% | 142% | 73330% |
| Class E | -8.48% | -16.46% | -16.42% | 250% | 72758% |
| Overall | -7.78% | -18.96% | -20.23% | 172% | 80428% |
| Class D | -9.22% | -18.82% | -19.35% | 148% | 66622% |
| Class F | -4.23% | -9.99% | -8.98% | 211% | 42845% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.13% | -14.24% | -17.26% | 251% | 67748% |
| Class A2 | -5.75% | -15.93% | -12.99% | 176% | 54763% |
| Class B | -5.93% | -15.56% | -17.45% | 166% | 51137% |
| Class C | -6.38% | -16.47% | -18.12% | 148% | 35566% |
| Class E | -8.81% | -15.27% | -16.32% | 180% | 58517% |
| Overall | -6.51% | -15.55% | -16.64% | 177% | 51141% |
| Class D | -6.40% | -15.48% | -18.58% | 141% | 32964% |
| Class F | -4.04% | -11.89% | -11.74% | 137% | 44279% |

The performance of the NCS-1.0 NN-based filter set #1 (int16 precision) over the NNVC-2.0 anchor is:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.92% | -15.94% | -18.51% | 210% | 52791% |
| Class A2 | -10.15% | -20.20% | -17.29% | 200% | 50531% |
| Class B | -9.01% | -23.08% | -21.77% | 209% | 51646% |
| Class C | -9.82% | -21.78% | -22.31% | 195% | 50009% |
| Class E |  |  |  |  |  |
| Overall | -9.44% | -20.73% | -20.37% | 203% | 51205% |
| Class D | -11.43% | -23.49% | -23.88% | 196% | 47037% |
| Class F | -4.69% | -11.70% | -10.66% | 298% | 25719% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.80% | -16.00% | -14.02% | 196% | 51463% |
| Class C | -9.14% | -15.17% | -14.89% | 171% | 45228% |
| Class E | -8.63% | -17.25% | -17.03% | 344% | 43518% |
| Overall | -8.45% | -15.63% | -14.40% | 216% | 47271% |
| Class D | -10.78% | -18.72% | -17.99% | 174% | 39565% |
| Class F | -4.97% | -8.98% | -6.08% | 272% | 23963% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.45% | -17.96% | -19.88% | 212% | 41577% |
| Class A2 | -6.50% | -20.46% | -17.82% | 164% | 33736% |
| Class B | -6.52% | -21.87% | -21.59% | 157% | 31884% |
| Class C | -7.40% | -18.86% | -21.76% | 153% | 23335% |
| Class E | -9.87% | -20.85% | -20.66% | 171% | 36317% |
| Overall | -7.26% | -20.14% | -20.56% | 168% | 32075% |
| Class D | -7.32% | -19.50% | -21.39% | 153% | 20201% |
| Class F | -4.06% | -11.76% | -10.93% | 134% | 21676% |

The performance of the NCS-1.0 NN-based filter set #1 (float precision) over the NNVC-2.0 anchor is:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.86% | -16.12% | -18.83% | 257% | 80229% |
| Class A2 | -10.16% | -20.28% | -17.34% | 246% | 77940% |
| Class B | -8.99% | -22.95% | -21.83% | 255% | 78170% |
| Class C | -9.83% | -21.70% | -22.38% | 226% | 71605% |
| Class E |  |  |  |  |  |
| Overall | -9.42% | -20.72% | -20.48% | 245% | 76716% |
| Class D | -11.45% | -23.40% | -23.80% | 231% | 70983% |
| Class F | -4.64% | -11.65% | -10.59% | 363% | 37408% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.75% | -15.97% | -13.64% | 244% | 76605% |
| Class C | -9.14% | -15.24% | -15.19% | 201% | 68109% |
| Class E | -8.52% | -17.06% | -17.80% | 458% | 65923% |
| Overall | -8.41% | -15.65% | -14.33% | 268% | 70947% |
| Class D | -10.76% | -18.20% | -17.94% | 200% | 58824% |
| Class F | -4.97% | -9.04% | -5.93% | 351% | 35888% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.42% | -17.97% | -19.74% | 247% | 64929% |
| Class A2 | -6.48% | -20.48% | -17.81% | 182% | 52976% |
| Class B | -6.49% | -21.90% | -21.61% | 174% | 49418% |
| Class C | -7.42% | -18.84% | -21.71% | 156% | 33740% |
| Class E | -9.87% | -20.81% | -20.61% | 188% | 56011% |
| Overall | -7.25% | -20.15% | -20.52% | 184% | 49081% |
| Class D | -7.35% | -19.43% | -21.33% | 149% | 32772% |
| Class F | -4.08% | -11.73% | -10.89% | 141% | 33401% |

## Technical Evaluation

The AHG made meaningful progress on the mandate to evaluate and quantify potential NN based video coding technologies.  A summary of AHG11 related non-EE1 contributions provided as input to the 28th meeting is provided below:

|  | Title | Common Test Conditions | Results |  |  | Training Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA | LDB | AI | CTC | Additional |
| Loop Filter |  |  |  |  |  |  |  |
| JVET-AB0108 | AHG11: ALF-SPLIT for NCS-1.0 | No | No | No | Yes | Unknown | - |
| JVET-AB0136 | AHG11: Complexity Reduction on Neural-Network Loop Filter | No | Yes | No | No | BVI-DVC |  |
| JVET-AB0158 | [AHG11] On chroma order adjustment in NNLF | No | Yes | Yes | No | Unknown |  |
| JVET-AB0159 | [AHG11] On adjustment of residual for NNLF | No | Yes | Yes | No | Unknown |  |
| JVET-AB0160 | [AHG11] Combination of chroma order adjustment and residual adjustment for NNLF | No | Yes | Yes | No | Unknown |  |
| Post Filtering |  |  |  |  |  |  |  |
| JVET-AB0109 | AHG11: Content-adaptive NN post-filter with new QP normalisation | No | No | No | No | - | JVET-CTC |
| JVET-AB0046 | AHG9: On StrengthControlVal of the NNPFC SEI message | No | No | No | No | - | - |
| JVET-AB0047 | AHG9: nnpfc_mode_idc related changes to the NNPFC SEI message | No | No | No | No | - | - |
| JVET-AB0049 | AHG9: Miscellaneous aspects of the two neural-network post-filtering SEI messages | No | No | No | No | - | - |
| JVET-AB0050 | AHG9: Activation of a neural-network post-processing filter for multiple pictures | No | No | No | No | - | - |
| JVET-AB0058 | AHG9: Frame Rate Upsampling Information in Neural-network Post-filter Characteristics SEI Message | No | No | No | No | - | - |
| JVET-AB0059 | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message | No | No | No | No | - | - |
| JVET-AB0060 | AHG9: On activation of the neural-network post-filter characteristics SEI message | No | No | No | No | - | - |
| JVET-AB0074 | AHG9: Auxiliary input for neural-network post-processing filter | No | No | No | No | - | - |
| JVET-AB0075 | AHG9: Separate processing of chroma components for neural-network post-processing filter | No | No | No | No | - | - |
| JVET-AB0134 | AHG9: On NN post-filter activation SEI message | No | No | No | No | - | - |
| JVET-AB0135 | AHG9: On complexity metrics for NN post-filter characteristics SEI message | No | No | No | No | - | - |
| JVET-AB0152 | AHG9: Regional on/off control and selection of NNPFs | No | No | No | No | - | - |
| JVET-AB0193 | AHG9: A summary of proposals on NNPF SEI messages | No | No | No | No | - | - |
| Super-Resolution |  |  |  |  |  |  |  |
| JVET-AB0107 | Non-EE1: CNN-based super resolution with luma-only rescaling | No | No | No | No | BVI-DVC | DIV2K |
| JVET-AB0101 | AHG11: Lightweight CNN filter for RPR-based SR with Wavelet Decomposition | No | No | No | No | BVI-DVC | DIV2K |
| JVET-AB0102 | AHG11/EE1-related: Updates on RPR encoder and filters | Yes | Yes | No | Yes | Unknown | - |
| Inter-Prediction |  |  |  |  |  |  |  |
| JVET-AB0114 | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement | Yes | Yes | Yes | No | Unknown | - |
| JVET-AB0121 | AHG11: Assistant Reference Picture Method for NNVC | No | No | No | No | Unknown |  |
| Intra Prediction |  |  |  |  |  |  |  |
| JVET-AB0149 | Non-EE1: neural network-based intra prediction with learned mapping to VVC intra prediction modes | No | Yes | No | Yes | BVI-DVC, TVD, UVG | DIV2K, ILSVCR2012 |

# Input contributions

There are 60 input contriubtions related to the AHG mandates. Twenty-seven of the contributions are part of the EE activity, while the remaining 33 contributions are related to AHG11 but not part of the EE. The list of input contributions is provided below.

## EE and Related Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AB0023 | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding | E. Alshina, F. Galpin, Y. Li, M. Santamaria, H.Wang, L.Wang, Z.Xie |
| EE Technology |  |  |
| JVET-AB0048 | EE1-1.1: Content-adaptive post-filter with SADL inference and signalling of NN post-filter characteristics and activation SEI messages | M. Santamaria, R. Yang, F. Cricri, J. Lainema, H. Zhang, R. G. Youvalari, M. M. Hannuksela (Nokia) |
| JVET-AB0052 | EE1-1.5: One luma model with IPB and/or skip for filtering intra and inter luma slices | D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson (Ericsson) |
| JVET-AB0053 | EE1-1.2: NN intra model without attention and partitioning strength | J. Ström, D. Liu, K. Andersson, P. Wennersten, M. Damghanian, R. Yu (Ericsson) |
| JVET-AB0054 | EE1-1.3: CNN Based In-Loop Filter with WCDANN | H. Zhang, C. Jung (Xidian Univ.), D. Zou, M. Li (OPPO) |
| JVET-AB0068 | EE1-1.6: RDO Considering Deep In-Loop Filtering | J. Li, Y.Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AB0073 | EE1-1.4: Deep In-Loop Filter with Additional Input Information | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AB0076 | EE1-2.1: RPR-Based Super-Resolution Guided by Partition Information | Q. Han, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AB0077 | EE1-2.2: CNN Filter for Super-Resolution with RPR functionality in VVC | S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AB0083 | EE1-1.8: More refinements on NN based in-loop filter with a single model | L. Wang, X. Xu, S. Liu (Tencent), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AB0084 | EE1-2.3：A CNN-based Super Resolution Method with GOP Level Adaptive Resolution | R. Chang, L. Wang, X. Xu, S. Liu (Tencent), J. Nam, S. Yoo, J. Lim, S. Kim (LGE) |
| JVET-AB0164 | EE1-1.7: Capacity Ablation of CNN-based in-loop filtering | S. Eadie, H. Wang, M. Coban, M. Karczewicz (Qualcomm) |
| EE Related |  |  |
| JVET-AB0090 | EE1-1.3 related: Lightweight and Efficient CNN In-loop Filter | H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AB0093 | EE1-2.2 related: Lightweight CNN Filter for Super-Resolution with RPR functionality in VVC | S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AB0098 | EE1-2.3 related: GOP Level Adaptive Resampling with CNN-based Super Resolution | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AB0141 | EE1-related: QP-based loss function design for NN-based in-loop filter | C. Zhou, Z. Lv, J. Zhang (vivo), W. Chen, J. Guo, B. Ai (BJTU) |
| JVET-AB0146 | EE1-1.8-related: encoder-only optimization for NN based in-loop filter with a single model | L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AB0147 | EE1-1.8-related: using additional models for higher temporal layers | L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AB0179 | EE1-related: Deep In-Loop Filter with Wide Activation and Large Receptive Field | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| Cross Checks |  |  |
| JVET-AB0063 | Cross-check of JVET-AB0053 (EE1-1.2: NN intra model without attention and partitioning strength) | M. Santamaria, F. Cricri (Nokia) |
| JVET-AB0064 | Cross-check of JVET-AB0054 (EE1-1.3: CNN Based In-Loop Filter with WCDANN) | M. Santamaria, F. Cricri (Nokia) |
| JVET-AB0088 | Crosscheck of JVET-AB0083 (EE1-1.8: More refinements on NN based in-loop filter with a single model) | D. Liu (Ericsson) |
| JVET-AB0089 | Crosscheck of JVET-AB0052 (EE1-1.5: One luma model with IPB and/or skip for filtering intra and inter luma slices) | L. Wang (Tencent) |
| JVET-AB0097 | Crosscheck of JVET-AB0084 (EE1-2.3：A CNN-based Super Resolution Method with GOP Level Adaptive Resolution) | D. Liu (Ericsson) |
| JVET-AB0105 | Crosscheck of JVET-AB0076 (EE1-2.1: RPR-Based Super-Resolution Guided by Partition Information) | R. Chang (Tencent) |
| JVET-AB0106 | Crosscheck of JVET-AB0077 (EE1-2.2: CNN Filter for Super-Resolution with RPR functionality in VVC) | R. Chang (Tencent) |
| JVET-AB0137 | Crosscheck of JVET-AB0068 (EE1-1.6: RDO Considering Deep In-Loop Filtering) | J. Ström (Ericsson) |

## Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AB0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall (co chairs), F. Galpin, Y. Li, H. Wang, L. Wang, Z. Wang, M. Wien, P. Wu (vice chairs) |
| Loop Filtering |  |  |
| JVET-AB0108 | AHG11: ALF-SPLIT for NCS-1.0 | W.Zou, Y.Zhou, C.M.Gu (Xidian University), C.Huang, Y.X.Bai, Y.J. Zhang(ZTE Corporation) |
| JVET-AB0136 | AHG11: Complexity Reduction on Neural-Network Loop Filter | J. N. Shingala, S. Kadaramandalgi, A. Shyam (Ittiam), T. Shao, A. Arora, P. Yin, Sean McCarthy (Dolby) |
| JVET-AB0158 | [AHG11] On chroma order adjustment in NNLF | Z. Dai, Y. Yu, H. Yu, D. Wang(OPPO) |
| JVET-AB0159 | [AHG11] On adjustment of residual for NNLF | Z. Dai, Y. Yu, H. Yu, D. Wang(OPPO) |
| JVET-AB0160 | [AHG11] Combination of chroma order adjustment and residual adjustment for NNLF | Z. Dai, Y. Yu, H. Yu, D. Wang(OPPO) |
| Post Filtering |  |  |
| JVET-AB0109 | AHG11: Content-adaptive NN post-filter with new QP normalisation | M. Santamaria, F. Cricri, M. M. Hannuksela (Nokia) |
| JVET-AB0046 | AHG9: On StrengthControlVal of the NNPFC SEI message | M. M. Hannuksela, M. Santamaria, F. Cricri (Nokia) |
| JVET-AB0047 | AHG9: nnpfc_mode_idc related changes to the NNPFC SEI message | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AB0049 | AHG9: Miscellaneous aspects of the two neural-network post-filtering SEI messages | Y.-K. Wang, Y. Li, C. Lin, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AB0050 | AHG9: Activation of a neural-network post-processing filter for multiple pictures | Y.-K. Wang, K. Zhang, L. Zhang, C. Lin, J. Li, Y. Li (Bytedance) |
| JVET-AB0058 | AHG9: Frame Rate Upsampling Information in Neural-network Post-filter Characteristics SEI Message | S. Deshpande, A. Sidiya (Sharp) |
| JVET-AB0059 | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message | S. Deshpande (Sharp) |
| JVET-AB0060 | AHG9: On activation of the neural-network post-filter characteristics SEI message | T. Chujoh, Y. Yasugi, T. Ikai (Sharp) |
| JVET-AB0074 | AHG9: Auxiliary input for neural-network post-processing filter | Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K. Wang (Bytedance) |
| JVET-AB0075 | AHG9: Separate processing of chroma components for neural-network post-processing filter | Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K. Wang (Bytedance) |
| JVET-AB0134 | AHG9: On NN post-filter activation SEI message | M. Pettersson, R. Sjöberg, J. Ström (Ericsson) |
| JVET-AB0135 | AHG9: On complexity metrics for NN post-filter characteristics SEI message | M. Pettersson, R. Sjöberg, J. Ström (Ericsson) |
| JVET-AB0152 | AHG9: Regional on/off control and selection of NNPFs | J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K Wang, Y. Li (Bytedance) |
| JVET-AB0193 | AHG9: A summary of proposals on NNPF SEI messages | Y.-K. Wang (Bytedance) |
| Super Resolution |  |  |
| JVET-AB0107 | Non-EE1: CNN-based super resolution with luma-only rescaling | C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AB0101 | AHG11: Lightweight CNN filter for RPR-based SR with Wavelet Decomposition | H. Lan, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AB0102 | AHG11/EE1-related: Updates on RPR encoder and filters | J. Nam, S. Yoo, J. Lim, S. Kim (LGE) |
| Intra Prediction |  |  |
| JVET-AB0149 | Non-EE1: neural network-based intra prediction with learned mapping to VVC intra prediction modes | T.Dumas, F.Galpin, P.Bordes (InterDigital) |
| Inter Prediction |  |  |
| JVET-AB0114 | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement | J. Jia, Y. Zhang, H. Zhu, Z. Chen (Wuhan Univ.), Z. Liu, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AB0121 | AHG11: Assistant Reference Picture Method for NNVC | C. M. Gu, W. Zou, Y. Zhou, J. W. Fan (Xidian Univ.), Y. X. Bai, C. Huang, Y. J. Zhang (ZTE) |
| E2E Methods |  |  |
| JVET-AB0125 | AHG11 - CompressAI models integration using SADL | F. Galpin, F.Levebvre, F.Racape (InterDigital) |
| Software |  |  |
| JVET-AB0126 | AhG11 - SADL update | F.Galpin, T.Dumas, P.Bordes, E.Francois (InterDigital) |
| JVET-AB0176 | NCS-1.0 status report | Y. Li (Bytedance), F. Galpin (InterDigital), H. Wang (Qualcomm), L. Wang (Tencent) |
| JVET-AB0183 | Preliminary draft of algorithm description for Neural Compression Software (NCS-1) | Y. Li, K. Zhang, L. Zhang (Bytedance), H. Wang, S. Eadie, M. Coban, M. Karczewicz (Qualcomm), L. Wang, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital), J. Ström (Ericsson) |
| Cross Checks |  |  |
| JVET-AB0204 | Crosscheck of JVET-AB0158 ([AHG11] On chroma order adjustment in NNLF) | L. Wang (Tencent) |
| JVET-AB0205 | Crosscheck of JVET-AB0159 ([AHG11] On adjustment of residual for NNLF) | L. Wang (Tencent) |
| JVET-AB0206 | Crosscheck of JVET-AB0160 ([AHG11] Combination of chroma order adjustment and residual adjustment for NNLF) | L. Wang (Tencent) |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Discuss and align on the configuration of EncDbOpt for the AI configuration.

- Discuss if the new common software is acceptable to the group and update CTC as appropriate.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.