| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li K. Naser H. Yang | Author(s) or Contact(s): | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com karam.naser@interdigital.com haitao.yang@huawei.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 27th meeting by teleconference, 13-22 July 2022, and the 28th meeting in Mainz (DE), 20-28 October 2022.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM6 algorithm description JVET-AA2025.

- Study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-Y2017.

- Analyse the results of exploration experiments described in JVET-AA2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

- Coordinate with AG 5, AHG4, and AHG6 on preparing a subjective test to be conducted during the upcoming JVET meeting.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AA2024). The combined improvements of the ECM-6.0 over VTM-11.0ecm anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.15% | -16.36% | -23.24% | 490% | 338% |
| Class A2 | -11.93% | -19.18% | -22.12% | 468% | 311% |
| Class B | -6.68% | -18.88% | -17.18% | 413% | 285% |
| Class C | -7.36% | -11.49% | -11.91% | 414% | 277% |
| Class E | -8.32% | -16.40% | -15.40% | 388% | 300% |
| Overall | -8.22% | -16.45% | -17.55% | 430% | 298% |
| Class D | -6.07% | -9.36% | -9.16% | 413% | 298% |
| Class F | -17.83% | -24.91% | -24.43% | 336% | 299% |
| Class TGM | -30.28% | -35.62% | -34.91% | 303% | 304% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -18.76% | -21.33% | -28.13% | 526% | 652% |
| Class A2 | -20.59% | -26.86% | -29.77% | 544% | 884% |
| Class B | -16.88% | -26.97% | -24.74% | 449% | 678% |
| Class C | -18.75% | -21.90% | -21.83% | 427% | 692% |
| Class E |  |  |  |  |  |
| Overall | -18.50% | -24.47% | -25.65% | 475% | 713% |
| Class D | -19.78% | -23.23% | -23.42% | 426% | 747% |
| Class F | -21.21% | -27.21% | -27.56% | 384% | 442% |
| Class TGM | -29.30% | -33.62% | -34.08% | 417% | 356% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.49% | -29.59% | -28.03% | 390% | 595% |
| Class C | -16.89% | -24.94% | -25.01% | 396% | 580% |
| Class E | -14.04% | -22.57% | -22.67% | 390% | 390% |
| Overall | -15.59% | -26.29% | -25.68% | 392% | 531% |
| Class D | -19.04% | -26.06% | -26.25% | 392% | 669% |
| Class F | -19.92% | -27.91% | -26.96% | 384% | 391% |
| Class TGM | -28.62% | -34.50% | -34.38% | 401% | 324% |

The rate reduction for natural sequences over VTM for RA configuration for {Y, U, V} increased from {-16.80%, -22.11%, -23.07%} to {-18.50%, -24.47%, -25.65%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from {-24.41%, -28.17%, -28.50%} to {-29.30%, -33.62%, -34.08%}.

Jointly with AG5 and JVET AHG4 and AHG7, AHG12 organized teleconferences to prepare a subjective assessment of the ECM performance at the 28th JVET meeting.

# Contributions

In addition to 26 EE2 contributions, 45 (comparing to 38 last meeting) related contributions were received which can be subdivided as follows:

## In Loop Filters (3)

JVET-AB0181, "Non-EE2: Using prediction samples or residual samples for adaptive loop filter", C. Ma, X. Xiu, C.-W. Kuo, W. Chen, H.-J. Jhu, N. Yan, X. Wang (Kwai)

JVET-AB0185, "Non-EE2: ALF with Diversified Extended Taps", W. Yin, K. Zhang, L. Zhang”, (Bytedance)

JVET-AB0186, "EE2-related: Modification of extended offline-filter taps for ALF", I. Jumakulyyev, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

## Intra (16)

JVET-AB0065, "Non-EE2: Adaptive reference region DIMD", Z. Fan, Y. Yasugi, T. Ikai (Sharp)

JVET-AB0099, "Non-EE2: CCCM with Multi-shape filters", C. Fang, S. Peng, D. Jiang, J. Lin, X. Zhang (Dahua)

JVET-AB0103, "EE2-1.16 related: Modifications of picture-level geometry transform", J. Choi, S. Yoo, J. Lim, S. Kim (LGE)

JVET-AB0104, "EE2-related: On directional planar prediction", S. Yoo, J. Choi, J. Nam, M. Hong, J. Lim, S. Kim (LGE)

JVET-AB0110, "EE2-1.15-related: Improvements on planar horizontal and planar vertical mode", K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)

JVET-AB0111, "Non-EE2: Directional intra prediction fusion with PMPM list", G. Moon, D. Park, Y.-U. Yoon, J.-G. Kim (KAU)

JVET-AB0116, "AHG12 - Location-dependent Decoder-side Intra Mode Derivation", S. Blasi, J. Lainema (Nokia)

JVET-AB0117, "AHG12 - Template-based Intra Mode Derivation with Directional blending", S. Blasi, J. Lainema, I. Zupancic, D. Bugdayci Sansli (Nokia)

JVET-AB0119, "Non-EE2: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction", R. G. Youvalari, P. Astola, J. Lainema (Nokia)

JVET-AB0128, "EE2-related: CCCM template selection", P. Bordes, K. Naser, E. Francois, F. Galpin (InterDigital)

JVET-AB0138, "EE2-related: MRL candidate list reordering", Yujin Lee, Bumyoon Kim, Byeungwoo Jeon (SKKU)

JVET-AB0139, "EE2-related: On Chroma Fusion improvement", C. Zhou, Z. Lv, J. Zhang (vivo)

JVET-AB0142, "Non-EE2: optimizing the use of available decoded reference samples", T. Dumas, K. Reuzé, Y. Chen, K. Naser (Interdigital)

JVET-AB0161, "EE2-1.16 related: Encoder optimization for picture-level geometry transform", W. Jia, K. Zhang, Y. Wang, T. Fu, Y. Li, L. Zhang (Bytedance)

JVET-AB0162, "EE2-ralated: On horizontal and vertical planar modes", X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-AB0174, "AHG12: Division-free operation and dynamic range reduction for convolutional cross-component model (CCCM)", A. Aminlou, J. Lainema, R. G. Youvalari, P. Astola (Nokia)

## Inter (9)

JVET-AB0145, "EE2-2.6-related: On Decoder-side Affine Model Refinement (DAMR)", J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)

JVET-AB0151, "EE2-2.1 related: ARMC merge candidate list reordering for AMVP-merge mode for low-delay pictures", K. Cui, C. S. Coban, Z. Zhang, V. Seregin, H. Huang, M. Karczewicz (Qualcomm), H. Jang (LGE)

JVET-AB0166, "Non-EE2: Unified pruning of affine merge candidates derivation", Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AB0168, "Non-EE2: Pixel based affine motion compensation", Z. Zhang, H. Huang, Y. Zhang, P. Garus, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AB0177, "EE2-related: Sub-block processing for affine DMVR", H. Huang, Y. Zhang, Z. Zhang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AB0178, "EE2-related: Control-point motion vector refinement for affine DMVR", H. Huang, C.-C. Chen, Y. Zhang, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AB0182, "Non-EE2: Bi-predictive local illumination compensation", X. Xiu, N. Yan, H.-J. Jhu, W. Chen, C.-W. Kuo, C. Ma, X. Wang (Kwai)

JVET-AB0189, "AHG12: On bit length control of regression based affine merge candidate derivation", Y. Zhang, H. Huang, V. Seregin, C.-C. Chen, M. Karczewicz (Qualcomm)

JVET-AB0192, "Non-EE2: Extended partitioning mode for the inter/intra prediction", Y. Kidani, H. Kato, K. Kawamura (KDDI)

## RPR (2)

JVET-AB0082, "AHG12: Fixes for RPR", K.Andersson, R. Yu, (Ericsson)

JVET-AB0133, "AHG12: Inter-RPL and 1-byte NAL unit headers", R. Sjöberg, M. Pettersson, J. Ström (Ericsson)

## Transform Coding (3)

JVET-AB0100, "Non-EE2: Separable KLT for intra coding", M. Koo, J. Zhao, J. Lim, S. Kim (LGE)

JVET-AB0115, "EE2-1.14 related: Modifications of MTS and LFNST for IntraTMP coded block", D. KIM, K. KIM, J. Son, J. S KWAK(WILUS)

JVET-AB0175, "Non-EE2: Non-Separable Primary Transform for Intra Coding", P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

## Screen Content Coding (11)

JVET-AB0062, "EE2-related: Modifications of EE2-3.2 and EE2-3.3", W. Lim, D. Kim, J. Kim, S.-C. Lim, J. S. Choi (ETRI), K. Naser, T. Dumas, T. Poirier, F. Galpin, A. Robert (InterDigital)

JVET-AB0094, "Non-EE2: Direct block vector (DBV) mode for chroma prediction", J.-Y. Huo, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), J. Ren, M. Li (OPPO)

JVET-AB0095, "Non-EE2: Block Vector Difference Sign Prediction (BVDSP) for IBC blocks", J.-Y. Huo, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), J. Ren, M. Li (OPPO)

JVET-AB0144, "EE2 related: Extension of test EE2-3.3", F. Le Leannec, P. Andrivon, M. Radosavljević, M. Blestel (Xiaomi)

JVET-AB0170, "Non-EE2: Block Vector Difference Prediction for IBC blocks", A. Filippov, V. Rufitskiy (Ofinno)

JVET-AB0173, "AHG12: BVP candidates clustering and BVD sign derivation for Reconstruction-Reordered IBC mode", Damian Ruiz Coll, Vikas Warudkar, Jung-Kyung Lee, (Ofinno)

JVET-AB0180, "Non-EE2: CCCM using non-downsampled luma samples", H.-J. Jhu, C.-W. Kuo, X. Xiu, W. Chen, N. Yan, C. Ma, X. Wang (Kwai)

JVET-AB0187, "Non-EE2: No luma subsampling for CCCM", V. Seregin, Y.-J. Chang, B. Ray, M. Karczewicz (Qualcomm)

JVET-AB0188, "Non-EE2: Extensions of intra block copy", Y. Wang, K. Zhang, L. Zhang, N. Zhang (Bytedance)

JVET-AB0190, "Non-EE2: Combination of JVET-AB0094 and JVET-AB0095 for screen content", J.-Y. Huo, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), J. Ren, M. Li (OPPO)

JVET-AB0191, "Non-EE2: Combined intra block copy and intra mode", C. Ma, X. Xiu, W. Chen, J.-H. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

## Other (1)

JVET-AB0066, "ECM-6 intra performance evaluation on non-CTC dataset", Y. Yasugi, T. Ikai (Sharp)

# Recommendations

The AHG recommends to:

- To review all the related contributions.

- Conduct a subjective assessment of the ECM performance.