| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0049-v2 |
| --- | --- |

| Title: | AHG9: Miscellaneous aspects of the two neural-network post-filtering SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Yue Li1, Chaoyi Lin2, Junru Li2, Kai Zhang1, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Email: | yekui.wang@bytedance.com yue.li@bytedance.com linchaoyi.cy@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes for the neural-network post-filter characteristics (NNPFC) SEI message and the neural-network post-filter activation (NNPFA) SEI message:

- When encountering a NNPFC SEI message with a reserved nnpfc_id value, the decoder shall ignore the NNPFC SEI message (instead of just ignoring the nnpfc_id value).

- Specify different types of NNPF visual quality improvement, including general visual quality improvement, objective-oriented/fidelity-oriented visual quality improvement, subjective-oriented visual quality improvement, and film grain-oriented visual quality improvement.

- Specify the value 0 for nnpfc_purpose to be a specific purpose (instead of "unknown or unspecified").

- Specify the value range of nnpfc_purpose to be 0 to 255, inclusive (instead of 0 to 232 − 2, inclusive).

- Specify the following constraint to replace the existing constraint on the value of nnpfc_out_sub_c_flag, as an asserted bug fix: When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

- Delta code filter output picture width and height while fixing the asserted bug of missing value ranges for ue(v)-coded syntax elements.

- Specify external-means provided patch size when nnpfc_constant_patch_size_flag is equal to 0, as an asserted bug fix.

- Add the following constraint: An NNPFA SEI message with a particular value of nnpfa_id shall not be present in a current PU unless one or both of the following conditions are true: a) Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id present in a PU preceding the current PU in decoding order; b) There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id in the current PU.

- Add the following constraint: When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order.

- Specify the value range and decoder handling of reserved values for nnpfa_id similarly as for nnpfc_id.

# Decoder handling of reserved nnpfc_id values

The semantics of nnpfc_id of the NNPFC SEI message is as follows:

nnpfc_id contains an identifying number that may be used to identify a post-processing filter. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive.

Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

"It" in the last sentence in the semantics means "the value of nnpfc_id". However, it is not sufficient to always just ignore the value of nnpfc_id when the value is one of the reserved values. For example, if there are multiple neural-network filters specified, each being associated with a particular value of nnpfc_id, and if one or more of these nnpfc_id values are in the reserved value ranges, then switching between the different filters would be problematic as the IDs of some or all of the filters have been ignored (i.e., some of the filters are just there without IDs).

It is therefore proposed specified that, when encountering a NNPFC SEI message with a reserved nnpfc_id value, the decoder shall ignore the NNPFC SEI message (instead of just ignoring the nnpfc_id value).

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts, same for proposed changes in Sections 2 to 9):

    - Neural-network post-filter characteristics SEI message semantics

This SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

...

nnpfc_id contains an identifying number that may be used to identify a post-processing filter. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive.

Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of neural-network post-filter characteristics SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it the SEI message.

...

# Different types of visual quality improvement for neural-network post-filtering

## Introduction

One of the purposes specified for a neural-network post-processing filter by nnpfc_purpose of the NNPFC SEI message is visual quality improvement. However, visual quality improvement may have different interpretations, such as fidelity-constrained visual quality improvement, generative adversarial network (GAN)-based visual quality improvement, film grain-based visual quality improvement, etc. Different video applications may prefer different types of visual quality improvements. For example, fidelity is important in surveillance scenario, but may not be as critical in some user-generated videos.

Therefore, it is desirable to be able to identify the type of visual quality improvement.

## Proposal

We propose the following syntax and semantics changes to define and signal the different types of visual quality improvements:

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 1 ) |  |
| nnpfc_visual_quality_improvement_type | ue(v) |
| ... |  |
| } |  |

8.28.2 Neural-network post-filter activation SEI message semantics

...

nnpfc_purpose indicates the purpose of post-processing filter as specified in Table 20. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table 20 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |

NOTE 1 – When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2 or 4.

nnpfc_visual_quality_improvement_type indicates the type of visual quality improvement as specified in Table 21. The value of nnpfc_visual_quality_improvement_type shall be in the range of 0 to 255, inclusive. Values of nnpfc_visual_quality_improvement_type that do not appear in Table 21 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification encountering a value of nnpfc_visual_quality_improvement_type greater than 3 shall ignore it.

Table 21 – Definition of nnpfc_visual_quality_improvement_type

| Value | Interpretation |
| --- | --- |
| 0 | General visual quality improvement, targeting at increasing either the fidelity or the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The improvement may be measured by any objective or subjective metric. |
| 1 | Objective-oriented/fidelity-oriented visual quality improvement, targeting at increasing the fidelity of the reconstructed picture after applying the neural-network post-processing filter. The fidelity may be measured by PSNR, MS-SSIM etc. |
| 2 | Subjective-oriented visual quality improvement, targeting at increasing the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The subjective visual quality may be measured by LPIPS or MOS. |
| 3 | Film grain-oriented visual quality improvement, with synthesizing filter grain on the reconstructed picture after applying the neural-network post-processing filter. |

NOTE 2 – When a reserved value of nnpfc_visual_quality_improvement_type is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_visual_quality_improvement_type being equal to that value.

# Value 0 for nnpfc_purpose

## Introduction

The semantics of nnpfc_purpose of the NNPFC SEI message is as follows:

nnpfc_purpose indicates the purpose of post-processing filter as specified in Table 20. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table 20 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |

NOTE 1 – When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2 or 4.

The value 0 for nnpfc_purpose is specified to be "unknown or unspecified". However, what is the usefulness of specifying this "unknown or unspecified" purpose? Note that some aspects related to the case of nnpfc_purpose equal to 0 are still specified, e.g., the values of outSubWidthC and outSubHeightC by the semantics of nnpfc_out_sub_c_flag regardless of whether nnpfc_purpose is equal to 0. In addition, if the purpose is unknown or unspecified, why was the entire SEI message included by the encoder in the bitstream at the first place? And how is the decoder / client supposed to use such an SEI message? Finally, for extensibility, there are already plenty of reserved values specified for this syntax element.

## Proposal

It is therefore proposed that the value 0 for nnpfc_purpose syntax element is specified to be a specific purpose (instead of "unknown or unspecified").

The proposed text changes are as follows:

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 2 1 ¦ ¦ nnpfc_purpose = = 4 3) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( nnpfc_purpose = = 3 2 ¦ ¦ nnpfc_purpose = = 4 3) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| ... |  |
| } |  |

8.28.2 Neural-network post-filter characteristics SEI message semantics

This SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

...

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table 20 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 1 0 | Visual quality improvement |
| 2 1 | Chroma format upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 2 | Increasing either of both of the width and height of the cropped decoded output picture without changing the chroma format |
| 4 3 | Increasing either or both of the width and height of the cropped decoded output picture and upsampling the chroma format |

NOTE 1 – When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2 or 4 1 or 3.

...

nnpfc_out_order_idc indicates the output order of samples resulting from the post-processing filter. Table 24 contains an informative description of nnpfc_out_order_idc values. The semantics of nnpfc_out_order_idc in the range of 0 to 3, inclusive, are specified in Table 25, which specifies a process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for different values of nnpfc_out_order_idc and a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors. When nnpfc_purpose is equal to 2 or 4 1 or 3, nnpfc_out_order_idc shall not be equal to 3 to 2. The value of nnpfc_out_order_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_out_order_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_out_order_idc.

...

# Value range of nnpfc_purpose

The current value range of nnpfc_purpose is 0 to 232 − 2, inclusive. We think that the upper limit is overly and unnecessarily high from practical usage point of view. Therefore, we propose to specify the value range of nnpfc_purpose to be 0 to 255, inclusive, instead.

# Constraint on the value of nnpfc_out_sub_c_flag

The semantics of nnpfc_out_sub_c_flag of the NNPFC SEI message is a follows:

nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC. If SubWidthC is equal to 2 and SubHeightC is equal to 1, nnpfc_out_sub_c_flag shall not be equal to 0.

The last sentence specifies a constraint on the value of nnpfc_out_sub_c_flag. However, it is possible that SubWidthC is equal to 2 and SubHeightC is equal to 1, while at the same time nnpfc_out_sub_c_flag is not present (when nnpfc_purpose is equal to a value that is neither 2 nor 4). In this case, per the constraint, nnpfc_out_sub_c_flag shall be equal to 1, while when nnpfc_out_sub_c_flag is equal to 1, per the first sentence in the semantics, outSubWidthC is equal to 1 and outSubHeightC is equal to 1. On the other hand, per the second sentence in the semantics, when nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC, and therefore in this case the values of outSubWidthC and outSubHeightC would be inferred to be equal to 2 and 1, respectively, i.e., outSubWidthC and outSubHeightC would have different values than per the first and last sentences. In short, the constraint specified by the last sentence is contradicting with the second sentence under certain condition.

It is therefore proposed to fix the above issue by specifying the following constraint instead:

When SubWidthC is equal to 2 and SubHeightC is equal to 1 (i.e., ChromaFormatIdc is equal to 2), either nnpfc_purpose shall be equal to a value that is neither 2 nor 4 (i.e., nnpfc_out_sub_c_flag is not present), or nnpfc_out_sub_c_flag shall not be equal to 0 (i.e., nnpfc_out_sub_c_flag shall be equal to 1).

For editorial simplicity, the above proposed constraint is further proposed to be editorially simplified to be the following:

When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

# Coding of filter output picture width and height

## Introduction

The filter output picture width and height are signalled by the ue(v)-coded syntax elements nnpfc_pic_width_in_luma_samples and nnpfc_pic_width_in_height_samples of the NNPFC SEI message.

The following issues are observed:

- The value ranges for these two ue(v)-coded syntax elements are missing.

- Both of the output picture width and height are signalled even when only one of them changes compared to the input value(s).

- Per the semantics of nnpfc_purpose, when nnpfc_purpose is equal to 3 or 4, at least one of the output picture width and height increases, and the width and height never become smaller. Therefore, it would be much more efficient to signal the delta using ue(v)-coded syntax elements.

## Proposal

To resolve all the above issues, it is proposed to delta code filter output picture width and height while fixing the bug of missing value ranges for ue(v)-coded syntax elements. The proposed changes are as follows:

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples nnpfc_delta_pic_width | ue(v) |
| nnpfc_pic_height_in_luma_samples nnpfc_delta_pic_width | ue(v) |
| } |  |
| ... |  |
| } |  |

8.28.2 Neural-network post-filter characteristics SEI message semantics

This SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

...

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting by applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively.

nnpfc_delta_pic_width and nnpfc_delta_pic_height specify the increments to the width and height, respectively, of the luma sample array of the picture resulting by applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_delta_pic_width and nnpfc_delta_pic_height are not present, they are both inferred to be equal to 0. The value of nnpfc_delta_pic_width shall be in the range of 0 to CroppedWidth * 15 − 1, inclusive. The value of nnpfc_delta_pic_height shall be in the range of 0 to CroppedHeight * 15 − 1, inclusive.

The variables nnpfcPicWdith and nnpfcPicHeight, specifying the width and height, respectively, of the luma sample array of the picture resulting by applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture, are derived as follows:

nnpfcPicWdith = CroppedWidth + nnpfc_delta_pic_width
nnpfcPicHeight = CroppedWidth + nnpfc_delta_pic_height

...

nnpfc_overlap specifies the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

The variables inpPatchWidth, inpPatchHeight, outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, outPatchCHeight, and overlapSize are derived as follows:

inpPatchWidth = nnpfc_patch_width_minus1 + 1
inpPatchHeight = nnpfc_patch_height_minus1 + 1
outPatchWidth = ( nnpfc_pic_width_in_luma_samples nnpfcPicWdith * inpPatchWidth ) / CroppedWidth
outPatchHeight = ( nnpfc_pic_height_in_luma_samples nnpfcPicHeight * inpPatchHeight ) / CroppedHeight
horCScaling = SubWidthC / outSubWidthC
verCScaling = SubHeightC / outSubHeightC
outPatchCWidth = outPatchWidth * horCScaling
outPatchCHeight = outPatchHeight * verCScaling
overlapSize = nnpfc_overlap

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfc_pic_width_in_luma_samples nnpfcPicWdith * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfc_pic_height_in_luma_samples nnpfcPicHeight.

...

Table 25 – Process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnpfc_out_ order_idc | Process StoreOutputTensors( ) for deriving sample values in the filtered picture from the output tensors |
| --- | --- |
| 0 | for( yP = 0; yP < outPatchHeight; yP++) for( xP = 0; xP < outPatchWidth; xP++ ) { yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples ) if ( yY < nnpfcPicHeight && xY < nnpfcPicWdith ) if( nnpfc_component_last_flag = = 0 ) FilteredYPic[ xY ][yY ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) else FilteredYPic[ xY ][ yY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) } |
| 1 | for( yP = 0; yP < outPatchCHeight; yP++) for( xP = 0; xP < outPatchCWidth; xP++ ) { xSrc = cLeft * horCScaling + xP ySrc = cTop * verCScaling + yP if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC && xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC ) if ( ySrc < nnpfcPicHeight / outSubHeightC && xSrc < nnpfcPicWdith / outSubWidthC ) if( nnpfc_component_last_flag = = 0 ) { FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ 1 ][ yP ][ xP ] ) } else { FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 1 ] ) } } |
| 2 | for( yP = 0; yP < outPatchHeight; yP++) for( xP = 0; xP < outPatchWidth; xP++ ) { yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP yC = yY / outSubHeightC xC = xY / outSubWidthC yPc = ( yP / outSubHeightC ) * outSubHeightC xPc = ( xP / outSubWidthC ) * outSubWidthC if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples ) if ( yY < nnpfcPicHeight && xY < nnpfcPicWdith) if( nnpfc_component_last_flag = = 0 ) { FilteredYPic[ xY ][ yY ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredCbPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ 1 ][ yPc ][ xPc ] ) FilteredCrPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ 2 ][ yPc ][ xPc ] ) } else { FilteredYPic[ xY ][ yY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredCbPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ 1 ] ) FilteredCrPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ 2 ] ) } } |
| 3 | for( yP = 0; yP < outPatchHeight; yP++ ) for( xP = 0; xP < outPatchWidth; xP++ ) { ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP if ( ySrc < nnpfc_pic_height_in_luma_samples / 2 && xSrc < nnpfc_pic_width_in_luma_samples / 2 ) if ( ySrc < nnpfcPicHeight / 2 && xSrc < nnpfcPicWdith / 2 ) if( nnpfc_component_last_flag = = 0 ) { FilteredYPic[ xSrc * 2 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredYPic[ xSrc * 2 + 1 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ 1 ][ yP ][ xP ] ) FilteredYPic[ xSrc * 2 ][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ 2 ][ yP ][ xP ] ) FilteredYPic[ xSrc * 2 + 1][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ 3 ][ yP ][ xP ] ) FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ 4 ][ yP ][ xP ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ 5 ][ yP ][ xP ] ) } else { FilteredYPic[ xSrc * 2 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredYPic[ xSrc * 2 + 1 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 1 ] ) FilteredYPic[ xSrc * 2 ][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 2 ] ) FilteredYPic[ xSrc * 2 + 1][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 3 ] ) FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 4 ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 5 ] ) } } |
| 4..255 | reserved |

...

# External-means provided patch size

## Introduction

The semantics of nnpfc_constant_patch_size_flag of the NNPFC SEI message is a follows:

nnpfc_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. When nnpfc_constant_patch_size_flag is equal to 0 the patch size width shall be less than or equal to CroppedWidth. When nnpfc_constant_patch_size_flag is equal to 0 the patch size height shall be less than or equal to CroppedHeight. nnpfc_constant_patch_size_flag equal to 1 specifies that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input.

As can be seen, the value of nnpfc_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. However, on the one hand, it is allowed for nnpfc_constant_patch_size_flag to be equal to 0, while on the other hand, regardless of the value of nnpfc_constant_patch_size_flag, the filtering process specified as part of the semantics of the NNPFC SEI message always use exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input, by the following equations (as part of the semantics of nnpfc_overlap):

inpPatchWidth = nnpfc_patch_width_minus1 + 1
inpPatchHeight = nnpfc_patch_height_minus1 + 1

In other words, the actual support of taking as input any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 when nnpfc_constant_patch_size_flag is equal to 0 is missing.

## Proposal

To provide the missing functionality, it is proposed to specify external-means provided patch size when nnpfc_constant_patch_size_flag is equal to 0.

The proposed changes are as follows:

8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. When nnpfc_constant_patch_size_flag is equal to 0 the patch size width shall be less than or equal to CroppedWidth. When nnpfc_constant_patch_size_flag is equal to 0 the patch size height shall be less than or equal to CroppedHeight. nnpfc_constant_patch_size_flag equal to 1 specifies that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input.

nnpfc_patch_width_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, specifies the horizontal sample counts of the patch size required for the input to the post-processing filter. When nnpfc_constant_patch_size_flag is equal to 0, any positive integer multiple of ( nnpfc_patch_width_minus1 + 1 ) may be used as the horizontal sample counts of the patch size used for the input to the post-processing filter. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32766, CroppedWidth − 1 ), inclusive.

nnpfc_patch_height_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, specifies the vertical sample counts of the patch size required for the input to the post-processing filter. When nnpfc_constant_patch_size_flag is equal to 0, any positive integer multiple of ( nnpfc_patch_height_minus1 + 1 ) may be used as the vertical sample counts of the patch size used for the input to the post-processing filter. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are provided by external means not specified in this Specification.

–	The value of inpPatchWidth shall be a positive integer multiple of nnpfc_patch_width_minus1 + 1 and shall be less than or equal to CroppedWidth. The value of inpPatchHeight shall be a positive integer multiple of nnpfc_patch_height_minus1 + 1 and shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

nnpfc_overlap specifies the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

The variables inpPatchWidth, inpPatchHeight, outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, outPatchCHeight, and overlapSize are derived as follows:

inpPatchWidth = nnpfc_patch_width_minus1 + 1 
inpPatchHeight = nnpfc_patch_height_minus1 + 1 
outPatchWidth = ( nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / CroppedWidth
outPatchHeight = ( nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / CroppedHeight
horCScaling = SubWidthC / outSubWidthC
verCScaling = SubHeightC / outSubHeightC							(79)
outPatchCWidth = outPatchWidth * horCScaling
outPatchCHeight = outPatchHeight * verCScaling
overlapSize = nnpfc_overlap

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfc_pic_height_in_luma_samples * inpPatchHeight.

...

# Constraint: No NNPFC, no NNPFA

An NNPFA SEI message with a particular neural-network post-filter ID may be present in a picture unit even if there is no NNPFC SEI message with the same neural-network post-filter ID present in the bitstream. However, in this case, the presence of the NNPFA SEI message would be meaningless.

Therefore, the following constraint is proposed:

An NNPFA SEI message with a particular value of nnpfa_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id in the current PU.

# Constraint: NNPFC shall precede NNPFA

Within a PU, an NNPFA SEI message with a particular neural-network post-filter ID may be present preceding a NNPFC SEI message with the same neural-network post-filter ID in decoding order. However, it would be much cleaner for an NNPFA SEI message to succeed the associated NNPFC SEI message in decoding order.

Therefore, the following constraint is proposed:

When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order.

# Value range for the ue(v)-coded nnpfa_id and handling of the reserved values

The nnpfa_id of the NNPFA SEI message is a ue(v)-coded syntax element and the semantics is as follows:

nnpfa_id specifies that the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture.

There are two issues here:

- As a ue(v)-coded syntax element, the value range of nnfpa_id is missing and needs to be specified.

- The handling by a decoder when encountering a value of nnpfa_id equal to a reserved value of nnpfc_id is missing and needs to be specified.

It is proposed to specify the value range and decoder handling of reserved nnfpa_id values similarly as for nnpfc_id of the NNPFC SEI message, as follows:

The value of nnpfa_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfa_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a NNPFA SEI message with nnpfa_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).